3GPP TSG-RAN WG4 Meeting #107		R4-2307486

# Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	UE Tx requirement for inter-band CA/EN-DC PC1.5 with 3Tx

Agenda item:	7.29.3.1

Document for:	Discussion

Introduction

In this paper, we provide our views on UE maximum output power and configured transmitted power for inter-band CA/EN-DC PC1.5 with 3Tx.

Discussion

Based on the WID[2],  inter-band CA/EN-DC power class 2 and power class 1.5 are targeted.

- CA power class or EN-DC power class is PC2 - PC3 FDD band 1Tx + PC2 TDD band 2Tx (UL MIMO and TxD) - PC3 FDD band 1Tx + PC3 TDD band 2Tx (UL MIMO) - PC3 TDD band 1Tx + PC2 TDD band 2Tx (UL MIMO) - CA power class or EN-DC power class is PC1.5 - PC3 FDD band 1Tx + PC1.5 TDD band 2Tx (UL MIMO and TxD)

According to the existing requirements, power class 1.5 is not specified for inter-band CA/EN-DC. However, the power class 1.5 for inter-band CA is listed in the WID.

Table 4.1-2: Band combinations for 3Tx with CA/EN-DC power class 1.5 [2]

| Band combination | UL configuration | Power class |
| --- | --- | --- |
| CA_n41A-n71A | CA_n41A-n71A | PC3@n71 1Tx; PC1.5@n41 2Tx; CA power class PC1.5 |

The related issues were listed as issue 2-2-4 and issue 2-5-1 in WF [1]. The issues are further discussed.

Issue 2-2-4: Pcmax changes

- Proposals - Option 1: Define configured transmitted power by considering up to PC1.5 for inter-band CA/EN-DC PC1.5 with 3Tx. For inter-band CA/EN-DC PC1.5 with 3Tx, the configured transmitted power needs to be defined by considering up to PC1.5. For example, - ΔPPowerClass,EN-DC = 6 dB for a power class 1.5 capable UE - ΔPPowerClass,EN-DC = 3 dB for a power class 2 capable UE - 0dB otherwise. - WF - Further study the ΔPpowerclass,CA changes for the SAR solutions

For the existing inter-band CA, the configured transmitted power is specified by considering up to PC2.

- ΔPPowerClass,CA = 3 dB for a power class 2 capable UE

- 0dB otherwise

For inter-band CA PC1.5 with 3Tx, the configured transmitted power needs to be defined by considering up to PC1.5.  For example,

- ΔPPowerClass,CA = 6 dB for a power class 1.5 capable UE

- ΔPPowerClass,CA = 3 dB for a power class 2 capable UE

- 0dB otherwise

All the Pcmax requirements have been specified without separating UE type for a single carrier, CA and EN-DC. The same principle should be considered.

Considering that inter-band EN-DC PC1.5 is in-scope in this WID, the configured transmitted power needs also to be defined by considering up to PC1.5 for the inter-band EN-DC PC1.5 if introduced.

Proposal 1: Define the configured transmitted power by considering up to PC1.5 for inter-band CA PC1.5 with 3Tx.

- ΔPPowerClass,CA = 6 dB for a power class 1.5 capable UE

- ΔPPowerClass,CA = 3 dB for a power class 2 capable UE

- 0dB otherwise

Issue 2-5-1: PC1.5 inter-band CA/EN-DC SAR compliance

- Proposals - Option 1: Use 0.5*maxUplinkDutyCycle-interBandCA-PC2 if indicated as the threshold for PC1.5 UL duty cycle, below which PC1.5 requirements apply. If absent, UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary. - Option 2: For PC1.5 band combination, SAR compliance with the possible new signalling for the duty cycle mechanism need to be further studied. - Option 3: Study whether the existing UE capabilities (PC2-related maxUplinkDutyCycle) can be applied or new UE capabilities are needed. - WF - Further study

For discussion on which option is reasonable, we assume the following parameters.

- Z1  = maxUplinkDutyCycle-interBandCA-PC2 (threshold)

- Z2 = new_maxUplinkDutyCycle-interBandCA-PC1.5 (threshold)

- X = average percentage of uplink symbols transmitted in a certain evaluation period

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2(=Z1) is not absent and the average percentage (=X) of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class (=PC3) to the supported power class and set the configured transmitted power as specified in clause 6.2A.4; – else; – shall apply all requirements for the supported power class (PC2) and set the configured transmitted power as specified in clause 6.2A.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as (X=)50% ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

For the existing PC2 inter-band CA (e.g, PC3 + PC2 CA),

- X  > Z1, 	PC3 requirements applies

- X ≤ Z1, 	PC2 requirements applies

For PC1.5 inter-band CA (e.g., PC3 + PC1.5 CA), 2 alternatives can be considered.

Alt1) Use 0.5*maxUplinkDutyCycle-interBandCA-PC2 for PC1.5 threshold, e.g, 0.5*Z1,

- X  > Z1, 		PC3 requirements applies

- 0.5*Z1 < X  ≤ Z1, 	PC2 requirements applies

- X ≤ 0.5*Z1,  		PC1.5 requirements applies

Alt2) Use new_maxUplinkDutyCycle-interBandCA-PC1.5 for PC1.5 threshold, e.g, Z2,

- X  > Z1, 		PC3 requirements applies

- Z2 < X  ≤ Z1, 		PC2 requirements applies

- X ≤ Z2,  		PC1.5 requirements applies

Here, X needs to be updated as follows.

- X = 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, )

- maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band or   maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

  - if power class of one or both of the bands within the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

  - else if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

  - else if the band is configured with power class 3;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Considering forward compatibility, Alt2 is preferable.

Proposal 2: Define either Alt1 or Alt2 for inter-band CA PC1.5 with 3TX.

Alt1) Use 0.5*maxUplinkDutyCycle-interBandCA-PC2 for PC1.5 threshold, e.g, 0.5*Z1,

- X  > Z1, 		PC3 requirements applies

- 0.5*Z1 < X  ≤ Z1, 	PC2 requirements applies

- X ≤ 0.5*Z1,  	PC1.5 requirements applies

Alt2) Use new_maxUplinkDutyCycle-interBandCA-PC1.5 for PC1.5 threshold, e.g, Z2,

- X  > Z1, 		PC3 requirements applies

- Z2 < X  ≤ Z1, 	PC2 requirements applies

- X ≤ Z2,  		PC1.5 requirements applies

Proposal 3: Update the corresponding maxDutyNR,x or maxDutyNR,y

- maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band or   maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

  - if power class of one or both of the bands within the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

  - else if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

  - else if the band is configured with power class 3;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Conclusion

In this contribution, we provided our view on the configured transmitted power and UE maximum output power for inter-band CA PC1.5 with 3Tx. The followings are proposed.

Pcmax changes

Proposal 1: Define the configured transmitted power by considering up to PC1.5 for inter-band CA PC1.5 with 3Tx.

- ΔPPowerClass,CA = 6 dB for a power class 1.5 capable UE

- ΔPPowerClass,CA = 3 dB for a power class 2 capable UE

- 0dB otherwise

PC1.5 inter-band CA/EN-DC SAR compliance

Proposal 2: Define either Alt1 or Alt2 for inter-band CA PC1.5 with 3TX.

Alt1) Use 0.5*maxUplinkDutyCycle-interBandCA-PC2 for PC1.5 threshold, e.g, 0.5*Z1,

- X  > Z1, 		PC3 requirements applies

- 0.5*Z1 < X  ≤ Z1, 	PC2 requirements applies

- X ≤ 0.5*Z1,  	PC1.5 requirements applies

Alt2) Use new_maxUplinkDutyCycle-interBandCA-PC1.5 for PC1.5 threshold, e.g, Z2,

- X  > Z1, 		PC3 requirements applies

- Z2 < X  ≤ Z1, 	PC2 requirements applies

- X ≤ Z2,  		PC1.5 requirements applies

Proposal 3: Update the corresponding maxDutyNR,x or maxDutyNR,y

- maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band or   maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

  - if power class of one or both of the bands within the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

  - else if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

  - else if the band is configured with power class 3;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

- References

[1] R4-2306554, “WF on Tx requirements for 3Tx”, Oppo

[2] RP-230161, “Revised WID for Low NR band 4Rx and 3Tx”, Oppo