3GPP TSG-WG RAN4 Meeting #107	R4-2307499

# Incheon, Republic of Korea, May 22 – 26, 2023

Source: 				Nokia, Nokia Shanghai Bell

Title: 				Discussion on intrabandENDC-Support

Agenda Item: 	10.2.3

Document for:	Approval

# Introduction

RAN2 LS [1] is received in response to RAN4 LS on intra-band EN-DC support [2]. RAN2 has recommended early implementation of a new UE capability signalling intraBandENDC-Support-UL to distinguish UL capability from DL so that possible compatibility issues can be avoided.

In this paper, we discuss RAN2 recommended solution and RAN4 specification impacts.

# Discussion

In Table 1, we have applied the RAN2 recommendation to show which intra-band EN-DC configurations are supported for band combinations of band 48 and n48. The cases for two and three carriers in DL and two carriers in UL are included. They cover all intra-band EN-DC so far introduced in RAN4, i.e., two carriers with bandwidth class A, three carriers with bandwidth class A, and one bandwidth class A NR carrier with bandwidth class C E-UTRA carriers.

The first column is the supported band combinations (with bandwidth class) for downlink. The second column is the existing UE capability, intraBandENDC-Support. The third column is the new UE capability, intraBandENDC-Support-UL, proposed by RAN2. The last column is the supported EN-DC configurations in the RAN4 notation.

By introducing new capability signalling, there is no ambiguity whether contiguous or non-contiguous intra-band EN-DC configurations are supported in uplink.

One remaining issue is the consistency of RAN2 signalling with the RAN4 categorization of contiguous, non-contiguous and mixed intra-band EN-DC configurations. RAN4 has introduced Table 5.3B.1.3-2 in TS 38.101-3 to list the mixed intra-band contiguous and non-contiguous EN-DC, which includes one of EN-DC configuration in Case #27 in Table 1, DL DC_48A-(n)48AA, UL DC_(n)48AA. However, this is categorized as contiguous in RAN2 definition, since this is only about the adjacent NR and E-UTRA carriers. Even if E-UTRA carriers are non-contiguous, that is not related to the intraband EN-DC capability. Therefore, it should be moved this configuration to the contiguous EN-DC table in 5.3B.1.2-1 to keep the consistency of RAN2 signalling and RAN4 defined EN-DC configuration.

Proposal 1: Intra-band EN-DC configuration DC_48A-(n)48AA with UL DC_(n)48AA in 5.3B.1.3-2 (mixed case) should be moved to in 5.3B.1.2-1 (contiguous case).

Table 1: Summary of supported EN-DC configurations for band combinations of 2 or 3 carriers (1 NR + 2 or 3 E-UTRA) in DL and 2 carriers (1 NR + 1 E-UTRA) in UL

| # | Reported band combination and DL bandwidth class (Class A for 2 UL) | intra-bandENDC-Support | intra-bandENDC-Support-UL | Supported configurations |
| --- | --- | --- | --- | --- |
| 1 | {48A, n48A} | absent (contiguous) | absent | DL DC_(n)48AA, UL DC_(n)48AA |
| 2 | {48A, n48A} | absent (contiguous) | non-contiguous | not allowed (since not possible to have such config for two carriers) |
| 3 | {48A, n48A} | absent (contiguous) | both | not allowed (since not possible to have UL non-contiguous DL contiguous for two carriers) |
| 4 | {48A, n48A} | non-contiguous | absent | DL DC_48A_n48A, UL DC_48A_n48A |
| 5 | {48A, n48A} | non-contiguous | non-contiguous | not allowed (since UL capability is the same as DL) |
| 6 | {48A, n48A} | non-contiguous | both | not allowed (since not possible to have UL contiguous DL non-contiguous for two carriers) |
| 7 | {48A, n48A} | both | absent | DL DC_(n)48AA, UL DC_(n)48AA DL DC_48A_n48A, UL DC_48A_n48A |
| 8 | {48A, n48A} | both | non-contiguous | not allowed (since not possible to have UL non-contiguous DL contiguous for two carriers) |
| 9 | {48A, n48A} | both | both | not allowed (since UL is the same as DL) |
| 10 | {48C, n48A} | absent (contiguous) | absent | DL DC_(n)48CA, UL DC_(n)48AA |
| 11 | {48C, n48A} | absent (contiguous) | non-contiguous | DL DC_(n)48CA, UL DC_48A_n48A |
| 12 | {48C, n48A} | absent (contiguous) | both | DL DC_(n)48CA, UL DC_48A_n48A DL DC_(n)48CA, UL DC_(n)48AA |
| 13 | {48C, n48A} | non-contiguous | absent | DL DC_48C_n48A, UL DC_48A_n48A |
| 14 | {48C, n48A} | non-contiguous | non-contiguous | not allowed (since UL capability is the same as DL) |
| 15 | {48C, n48A} | non-contiguous | both | not allowed (UL contiguous and DL non-contiguous is invalid) |
| 16 | {48C, n48A} | both | absent | DL DC_(n)48CA, UL DC_(n)48AA DL DC_48C_n48A, UL DC_48A_n48A |
| 17 | {48C, n48A} | both | non-contiguous | DL DC_(n)48CA, UL DC_48A_n48A DL DC_48C_n48A, UL DC_48A_n48A |
| 18 | {48C, n48A} | both | both | DL DC_(n)48CA, UL DC_(n)48AA DL DC_(n)48CA, UL DC_48A_n48A DL DC_48C_n48A, UL DC_48A_n48A |
| 19 | {48A, 48A, n48A} | absent (contiguous) | absent | DL DC_48A-(n)48AA, UL DC_(n)48AA |
| 20 | {48A, 48A, n48A} | absent (contiguous) | non-contiguous | DL DC_48A-(n)48AA, UL DC_48A_n48A |
| 21 | {48A, 48A, n48A} | absent (contiguous) | both | DL DC_48A-(n)48AA, UL DC_(n)48AA DL DC_48A-(n)48AA, UL DC_48A_n48A |
| 22 | {48A, 48A, n48A} | non-contiguous | absent | DL DC_48A-48A_n48A, UL DC_48A_n48A |
| 23 | {48A, 48A, n48A} | non-contiguous | non-contiguous | not allowed (since UL capability is the same as DL) |
| 24 | {48A, 48A, n48A} | non-contiguous | both | not allowed (UL contiguous and DL non-contiguous is invalid) |
| 25 | {48A, 48A, n48A} | both | absent | DL DC_48A-(n)48AA, UL DC_(n)48AA DL DC_48A-48A_n48A, UL DC_48A_n48A |
| 26 | {48A, 48A, n48A} | both | non-contiguous | DL DC_48A-(n)48AA, UL DC_48A_n48A DL DC_48A-48A_n48A, UL DC_48A_n48A |
| 27 | {48A, 48A, n48A} | both | both | DL DC_48A-(n)48AA, UL DC_(n)48AA DL DC_48A-(n)48AA, UL DC_48A_n48A DL DC_48A-48A_n48A, UL DC_48A_n48A |

# Conclusion

In this contribution, we have analyzed the suggested solution by RAN2, which can solve the issues RAN4 has been long discussing.

Proposal 1: Intra-band EN-DC configuration DC_48A-(n)48AA with UL DC_(n)48AA in 5.3B.1.3-2 (mixed case) should be moved to in 5.3B.1.2-1 (contiguous case).

# Reference

[1] R2-2304431 Reply LS on intraBandENDC-Support, RAN2#121-bis-e

[2] R4-2220837 LS on intraBandENDC-Support, RAN4#105

[3] R4-2219710 Open issues on intra-bandENDC-Support, Nokia, Nokia Shanghai Bell, RAN4#105