3GPP TSG RAN WG4 Meeting#107                          R4-2307510

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on L1/L2 inter-cell mobility delay requirements

Source:	China Telecom

Agenda item:			8.25.2.3

Document for:	Discussion

- Introduction

In the RAN4 #106-bis-e meeting, the L1/L2 based inter-cell mobility were discussed and WF [1] has been approved. In this paper, some issues on L1/L2 inter-cell mobility delay requirements are further discussed.

- Discussion

Issue 3-2-2: Procedure of cell switch

In last meeting, companies had different views on procedure of cell switch. The Way forward during the last meeting is duplicated as below [1]:

< Way forward >: FFS the following options - Option 1: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. - Option 2: Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch. - Option 3: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2). - Option 4: UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

In our understanding, when UE perform T/F fine tracking, the precondition is that the target cell is a known cell for UE or UE has searched the target cell. However, Tsearch is contained in Tinterruption, in which case, if tend to reduce the interruption time during cell switch by performing T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2), the target cell should be known cell.

Proposal 1: Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

Issue 3-3-4: Tprocessing,2

In last meeting, companies had different views on Tprocessing,2. The Way forward during the last meeting is duplicated as below [1]:

< Way forward >: FFS the following options - Option 1 (CTC, CMCC, Nokia, Huawei): The time for UE processing could be reduced if some procedures have been done before UE receive the cell switch command or for some scenarios. - Option 1a (CMCC): Tprocessing = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command - Option 1b (Nokia): - Some of the RRC processing can be done prior to the cell switch to reduce the delay. FFS what parts are done when the configuration arrives. - At least partial RRC processing is done before the cell switch command. Exact conditions can be FFS. - LTM-processing Should consider a scenario where the user plane does not need reset, and there is no need for further RRC processing. - Option 2 (Apple): Tprocessing in LTM shall include execution time CHO and the legacy Tprocessing in Tinterrupt. - Option 3 (QC): RAN4 to not assume UE can always finish a processing of RRC configurations for LTM cells before LTM handover command reception, e.g., the processing and loading the configuration before the LTM handover command reception can be limited to measurement related configurations of the LTM cells. And RAN4 to not assume the processing and loading the measurement configuration of the LTM cell before LTM handover command reception means the entire downlink configuration of the LTM target cell is processed and loaded. - Option 4 (MTK): - To avoid defining too much Tprocessing,2 values for different scenarios, suggest focusing only on the typical scenarios and classifying the scenarios into limited groups. - Categorize all the scenarios into at most four groups depending on if L2/L3 reconfiguration or L1 reconfiguration is needed: L2/L3 reconfiguration; L1 reconfiguration; Typical scenario. Group#1; Y; Y; - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch with L1 and L2/L3 reconfiguration. Group#2; N; Y; - intra-DU or Inter-DU, intra-frequency or inter-frequency cell switch without L2/L3 reconfiguration but with L1 reconfiguration: - including switch to active SCell without L2/L3 reconfiguration. Group#3; N; N; - intra-frequency cell switch without L1/L2/L3 reconfiguration, maybe intra-DU or inter-DU. Group#4; Y; N; - intra-frequency cell switch with L2/L3 reconfiguration, maybe intra-DU or inter-DU. - Tprocessing,2=20ms for intra-FR cell switch and Tprocessing,2=40ms for inter-FR cell switch when software processing for L2/L3 reconfiguration and L1 reconfiguration is needed. FFS: the value for other groups. - Option 5 (vivo): - In the cell switch delay requirements, T_processing is specified in a case-by-case manner based on the current state of target cell. - For the baseline requirement, RAN4 assume UE forms the complete configuration and applies it during the execution of cell switch command. Before cell switch command, only necessary L1 configuration are formed and applied. - Option 6 (Ericsson): - Processing delay is 3ms when delta configuration is used and L2 reset is not performed. - Processing delay is 0ms for the role change case. - For all other cases, RAN4 shall try to optimise the UE processing delay so that LTM benefits can be fully realised

As RAN2 assumed, the time for UE processing, before and after cell switch command, respectively. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc.

In our understanding, if some procedures have been done before UE receives the cell switch command, i.e. security update, the time for UE process could been reduced in certain case.

Proposal 2: The time for UE processing could been reduced if some procedures have been done before UE receive the cell switch command.

Issue 3-3-5: T/F fine tracking: TΔ and Tmargin

In last meeting, companies had different views on T/F fine tracking: TΔ and Tmargin. The Way forward during the last meeting is duplicated as below [1]:

< Way forward >: FFS the following options - Option 1: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms. FFS: whether TΔ and Tmargin can be 0 under certain conditions. - Option 1a: TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms. - Option 1b: TΔ and Tmargin can be 0 under the condition that DL synchronization for candidate cell(s) has been performed before cell switch command. - Option 1c: When TCI state to use for the target cell is already in the active TCI state list, it is also necessary for the UE to meet the Te requirement for an initial transmission, that is, at least one SSB is available at the UE during the last 160ms, then TΔ and Tmargin can be 0. - Option 1d: TΔ = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command - Option 1e: TΔ = 0 for the case that SSB based fine synchronization for candidate cell(s) is performed before cell switch command - Option 2: FFS

TΔ is the time for fine time tracking and acquiring full timing information and Tmargin is the time for SSB post-processing, the procedure is same as L3 HO. So, we think the requirements for L3 HO could be reused as baseline. In addition, for L1/L2 mobility, we think the DL synchronization for candidate cell(s) could be performed before cell switch command, in which case, TΔ can be 0.

Proposal 3: Reuse the requirements for L3 HO as baseline, i.e., TΔ=1 Tfirst-RS, Tmargin = 2ms.

- Under the condition that DL synchronization for candidate cell(s) has been performed before cell switch command, TΔ can be 0.

Issue 3-3-14: Tinterruption

In last meeting, companies had different views on Tinterruption. The Way forward during the last meeting is duplicated as below [1]:

< Way forward >: FFS the following options - Option 1: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd - Option 2: There is almost no interruption during cell switch procedure when target Pcell/SCell is current SCell/PCell. - Option 3: - For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU. - For RACH-less cell switch, T_interruption at least include T_processing,2

When UE processes the cell switch command, it could remain the connection with source cell. So it should be excluded from Tinterruption. In which case, the components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 4: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Issue 3-4-1: known cell conditions

In last meeting, companies had different views on known cell conditions. The Way forward during the last meeting is duplicated as below [1]:

< Way forward >: FFS the following option - Use the following known cell condition as a baseline for further study: - The target cell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the cell switch command: - the UE has sent a valid L1 or [L3] measurement report for the target cell and - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3, - One of the SSBs measured from the target cell also remains detectable during the cell switch delay according to the cell identification conditions specified in clause 9.3. - otherwise it is unknown.

We agree to add the L1 measurement report for the target cell in known case.

Proposal 5: Agree with the above option as known cell conditions.

- Summary

In this paper, we provide our views on L1/L2 inter-cell mobility delay requirements. From this discussion we have derived the following proposals:

Proposal1: Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

Proposal 2: The time for UE processing could been reduced if some procedures have been done before UE receive the cell switch command.

Proposal 3: Reuse the requirements for L3 HO as baseline, i.e., TΔ=1 Tfirst-RS, Tmargin = 2ms.

- Under the condition that DL synchronization for candidate cell(s) has been performed before cell switch command, TΔ can be 0.

Proposal 4: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 5: Agree with the above option as known cell conditions.

- References

[1] R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.