3GPP TSG-RAN WG4 Meeting #107	 R4-2307527

Incheon, South Korea, 22– 26 May, 2023

Source:	Huawei, HiSilicon, Deutsche Telekom AG

Title:	TP for TR 38.718-03-01: to include band combination CA_n3A-n75A-n78A

Agenda item:			7.11.2

Document for:	Approval

1. Introduction

A text proposal for TR 38.718-03-01to add CA_n3A-n75A-n78A configuration as specified in WID [1].

2. Text Proposal

---Start of changes---

5.x	CA_n3A-n75A-n78A

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz |  |  | FDD |
| n75 | N/A | 1432 MHz | – | 1517 MHz | SDL |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz |  |  | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n75+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n75A-n78A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n75-n78, the TIB,c and RIB,c values are reused from DC_2-12_n41and are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n75-n78 | 0.6 | - | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n75-n78 | 0.2 | - | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

---End of changes---

Reference

[1]		RP-222834, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2), ZTE