3GPP TSG-RAN WG4 Meeting # 107		R4-2307594

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.14.5.2

Source: 	CMCC

Title: 			 Discussion on UE demodulation and CSI requirements for ATG scenario

Document for:	Discussion

- Introduction

In last meeting, RAN4 has agreed to at least cover AWGN channel model for both DL and UL demodulation requirements, and some test parameters have been agreed. We list the agreements for information

Issue 1-1: Scenario Agreement: - Focus on ATG deployment scenario which aligned with co-existence session, other scenarios (e.g., the scenario with large Doppler jumping in an ATG cell) can be considered in the future enhancement when needed. Issue 1-2: UE assumption Tentative agreements: - Take ATG UE pre-compensation on UL frequency shift and UL timing shift for ATG demodulation as baseline. - Take ATG UE pre-compensation on DL frequency shift for ATG demodulation as baseline. Issue 1-3-1: Channel model Agreement: - At least single path AWGN channel with doppler Issue 1-3-2: Doppler shift assumption: Agreement from GTW: - Align with RF session agreement on frequency error requirements with below values as starting point - For FDD channel model, set Doppler as [220Hz] - For TDD channel model, set Doppler as [400/500Hz]. FFS: - For FDD channel model, - Option 1: set Doppler as 220Hz - Option 2: Set Doppler as 200Hz for UL, 220Hz for DL - For TDD channel model: - Option 1: set Doppler as 400Hz. - Option 2: set Doppler as 500Hz Issue 2-6: DMRS Agreement: - 1 front loaded DMRS + 1 Additional DMRS position for both FDD and TDD test cases Issue 2-7: koffset Agreements: - koffset value 2ms, the unit and value can be revised after RAN2 confirm whether SIB19 can be reused or not.

In this contribution, we further provide our views on test scope and test setup.

- PDSCH requirements

Issue 2-2: TDD pattern

- Option 1: Define a new TDD pattern 30D4S6U which only applied for ATG scenario.

- Option 2: Proceed with typical TDD pattern for requirement, further check the performance if the new TDD pattern is agreed to be introduced

  - RF, RRM and Demod sessions should keep the consistency on the agreement for the necessary of new TDD pattern for ATG scenario.

  - Start with FDD requirement configuration if companies cannot get consensus on TDD pattern issue easily

- Option 3: If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only.

For ATG scenario, the distance between CPE and ATG BS may achieve 250km, a large guard period should be configured to prevent interference from DL to UL, the GP length should at least cover 2*propagation delay ≈ 1.7ms. Besides, in ATG network, the demand of downlink throughput is much more than uplink throughput. Therefore, we propose to define a new TDD pattern 30D4S6U, which is applied for ATG UE only.

Applying the new TDD pattern in ATG test cases can also verify that the ATG UE could support this new TDD pattern, which is different from common TDD pattern in TN network.

Proposal 1: Define a new TDD pattern 30D4S6U which only applied for ATG scenario.

To support this new TDD pattern, UE need to support the extended k1 and HARQ process. Such UE capabilities had been introduced in R17 NTN WI by RAN1. We copy it as follows:

| 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL | max-HARQ-ProcessNumber-r17 | BandNR | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26-10 | K1 range extension | 1.Support of extended K1 value range of (0..31) for unpaired spectrum | k1-RangeExtension-r17 | BandNR | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |

From our perspective, we think there is no technical issue for ATG UE supporting these two features. In fact, recall the discussion of RAN1 R17 NTN, RAN1 also explicitly agreed that such UE capabilities support ATG. However, due to no ATG operating bands at that time, the applicability of the UE capabilities didn’t include ATG scenario. In order to finish the work for ATG, LS should be sent to RAN1 to add these two features for R18 ATG.

Proposal 2: Send a LS to RAN1, to add the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ to R18 ATG.

Issue 2-3: Bandwidth & SCS

- For TDD:

  - Option 1: 40MHz for TDD 30kHz.

  - Option 2: 100MHz for TDD 30kHz.

- For FDD:

  - Option 1: 20Mhz for FDD 15kHz

  - Option 2: 10MHz for FDD 15kHz

Considering the high demand of channel capacity, we propose to use larger bandwidth. For TDD, we support 100MHz. For FDD, although 20MHz is preferred, we can compromise to the agreements in RAN4#106, configure 10MHz for FDD.

Proposal 3: Use 100MHz bandwidth for TDD 30kHz, use 20MHz or 10MHz for FDD 15kHz.

Issue 2-4: Antenna Configuration

- 2T2R

- FFS:

  - 2T4R (for UE supporting 4R)

  - 4T4R (for UE supporting 4R)

It is typical to use 4R to achieve better performance if UE supporting 4R. Therefore, we think 4R should be covered.

Proposal 4: For antenna configuration, cover 2T4R test case which only applied for UE supporting 4R.

Issue 2-5: MCS&Rank

- Cover rank 1, 16QAM and 64QAM

- FFS: Whether to cover 256 QAM

- Companies are encouraged to bring simulation results in next meeting.

In our simulation results [2], we use MCS 13 in table 1 for 16QAM, MCS 22 in table 1 for 64QAM. The SNR at 70% TP is in the test range.

Besides, our simulation results shows that the SNR at 70% TP is in the test range for 256QAM. Considering the high channel capacity demand, and lack of rank 2 supporting, we propose to cover 256QAM, the MCS 24 in table 2 can be used.

Observation 1: For MCS 13 in table 1, MCS 22 in table 1 and MCS 24 in table 2, the SNRs at 70% TP are in the test range.

Proposal 5: Cover 256QAM for ATG demod.

Issue 2-1: Test scope

- FFS For PDSCH:

  - Option 1: Introduce PDSCH requirements with AWGN+Doppler channel model

  - Option 2: FFS

    - Decide whether to introduce new PDSCH requirements after channel model and other parameters (e.g., bandwidth, TDD pattern and so on) are agreed.

    - If no new PDSCH requirements for ATG, further decide which legacy PDSCH requirements shall be applied for ATG UE.

Considering there is no PDSCH requirements with AWGN+fixed doppler channel model and new TDD pattern, we support to introduce PDSCH requirements for ATG UE.

Proposal 6: Introduce new PDSCH test cases for ATG including:

| FDD 10 or 20MHz 15kHz SCS | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |
|  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

| TDD 100MHz 30kHz SCS | 30D4S6U | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  | 7D1S2U | 2T2R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

- PDCCH requirements

Issue 2-1: Test scope

- FFS For PDCCH:

  - Option 1: Introduce PDCCH requirements with AWGN+Doppler channel model

  - Option 2: No need to introduce additional PDCCH requirements. But the legacy PDCCH mandatory requirements shall be met.

  - Option 3: FFS

    - Decide whether to introduce new PDCCH requirements after channel model is agreed.

    - If no new PDCCH requirements for ATG, whether and which legacy PDCCH requirements shall be applied for ATG UE

Since there is no legacy PDCCH requirements with AWGN+Doppler channel model, we propose to also define new PDCCH requirements which only applicable ATG UE.

Proposal 7: Introduce PDCCH requirements with AWGN+Doppler channel model which only applicable for ATG UE.

Aggregation level

Considering the AWGN channel model, and 200Hz doppler for FDD and 500Hz doppler for TDD, we propose to configure aggregation level 4 and 8 as the starting point for simulation assumption.

CORESET duration

For CORESET duration, we propose to take 1 and 2 symbols as the starting point.

For above parameters, further down selection can be done based on SNR simulation results.

Proposal 8: For simulation assumption, take aggregation level 4 and 8, CORESET duration 1 and 2 symbols as the starting point, and do the further selection according to SNR simulation results.

- CSI reporting requirements

Issue 2-1: Test scope

- For CSI reporting:

  - Do not define new CSI reporting requirements for ATG UE.

  - FFS

    - Option 1: legacy CSI reporting requirements shall be met.

    - Option 2: FFS whether and which legacy UE CSI reporting requirements shall be applied for ATG UE.

If ATG specific requirements for PDSCH and PDCCH will be introduced, then the UE capability of doppler handling will be verified, we are fine to reuse the legacy CSI reporting requirements under AWGN propagation condition.

Proposal 9: For CSI reporting, if ATG specific requirements for PDSCH and PDCCH will be introduced, reuse the legacy CSI reporting requirements under AWGN propagation condition.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: Define a new TDD pattern 30D4S6U which only applied for ATG scenario.

Proposal 2: Send a LS to RAN1, to add the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ to R18 ATG.

Proposal 3: Use 100MHz bandwidth for TDD 30kHz, use 20MHz or 10MHz for FDD 15kHz.

Proposal 4: For antenna configuration, cover 2T4R test case which only applied for UE supporting 4R.

Observation 1: For MCS 13 in table 1, MCS 22 in table 1 and MCS 24 in table 2, the SNRs at 70% TP are in the test range.

Proposal 5: Cover 256QAM for ATG demod.

Proposal 6: Introduce new PDSCH test cases for ATG including:

| FDD 10 or 20MHz 15kHz SCS | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |
|  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

| TDD 100MHz 30kHz SCS | 30D4S6U | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  | 7D1S2U | 2T2R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

Proposal 7: Introduce PDCCH requirements with AWGN+Doppler channel model which only applicable for ATG UE.

Proposal 8: For simulation assumption, take aggregation level 4 and 8, CORESET duration 1 and 2 symbols as the starting point, and do the further selection according to SNR simulation results.

Proposal 9: For CSI reporting, if ATG specific requirements for PDSCH and PDCCH will be introduced, reuse the legacy CSI reporting requirements under AWGN propagation condition.

- Reference

[1] R4-2305899, WF for ATG demodulation requirements, CMCC.

[2] R4-2307595, Simulation results for ATG UE demodulation, CMCC

- Appendix

The draft LS to RAN1 is captured below:

3GPP TSG RAN WG4 Meeting #107		R4-23XXXXX

Incheon, KR, May 22 – May 26, 2023

Title:	LS on UE feature for NR ATG

Release:	Rel-18

Study Item:	NR_ATG

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Shiyuan Wang

E-mail Address:	wangshiyuan@chinamobile.com

Attachments: None

1. Overall Description:

In ATG deployment scenario, the distance between CPE and ATG BS may achieve 250km, a large guard period should be configured to prevent interference from DL to UL, the GP length should at least cover 2*propagation delay ≈ 1.7ms. Besides, in ATG network, the demand of downlink throughput is much more than uplink throughput. Therefore, a new TDD pattern 30D4S6U is needed. However, to support HARQ under this new TDD pattern, the range of k1 and number of HARQ process should be extended.

RAN4 notice that such UE features had been introduced in R17 NTN WI by RAN1, which are 26-5‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ as specified in TS38.822. Recall the discussion of RAN1 R17 NTN, RAN1 also explicitly agreed that such UE capabilities can support ATG scenario. However, due to no ATG operating bands at that time, the applicable scenario of the features didn’t include ATG.

Considering all the information above, RAN4 would like to respectfully ask RAN1 to check that if the UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ could be used for Rel-18 NR ATG. If yes, RAN4 kindly ask RAN1 to add the same UE feature as 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ to NR ATG in RAN1 feature group.

2. Actions:

To RAN1

ACTION: 	RAN4 would like to respectfully ask RAN1 to check that if the UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ could be used for Rel-18 NR ATG. If yes, RAN4 kindly ask RAN1 to add the same UE feature as 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ to Rel-18 NR ATG in RAN1 feature group.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #108 	21 Aug - 25 Aug 2023		Toulouse, FR

TSG-RAN WG4 Meeting #108-bis 	9 Oct - 13 Oct 2023	               Xiamen, CN