3GPP TSG-RAN WG4 Meeting # 107	R4-2307605

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.30.3.2

Source: MediaTek Inc.

Title: Discussion on R18 MIMO for Timing requirements for UL multi-DCI multi-TRP with two TAs

Document for: Discussion

- Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into three agenda items to be discussed: (1) RRM requirements impacts, (2) Timing requirements for UL multi-DCI multi-TRP with two TAs, (3) Unified TCI framework. The discussion in this paper focus on the “Timing requirements for UL multi-DCI multi-TRP with two TAs”.

- Discussion

Based on our preliminary study, we suggest RAN4 can further study following topics.

- Scenarios for multi-TRP with 2TAs

- DL reference timing for UL multi-DCI multi-TRP with two TAs

- MTTD

2.1 Scenarios for multi-TRP with 2TAs

In last meeting, there’re some discussion [2] below on scenarios for multi-TRP with 2TAs. RAN4 agree to discuss the MTTD requirement with two TAGs only for mDCI scenarios.

Issue 2-1-2: MTTD requirements applicability Agreement: - RAN4 to discuss the MTTD requirement with two TAGs only for mDCI. Issue 2-1-5: TAG management for multi-TRP with 2 TA - Proposals: - Option 1: (Apple) - RAN4 can do some study on TAG management for multi-TRP with 2 TAs. - For example: Once it is about to exceed the limit UE can support, UE can send some indication to network so that network can e.g. indicate UE to fall back to single TA to avoid waste of UL resource since UE anyway cannot maintain two UL soon. - Option 2: use LTE CA requirements as baseline (Ericsson, Huawei, ZTE) - Option 3: FFS (vivo, MediaTek, Samsung)

For issue 2-1-5, in our understanding, there’s similar issue already existed in legacy CA/DC scenarios. For CA/DC, no specific requirements or UE behavior are defined. If the actual MTTD exceeds the maximum value in CA/DC, it would be up to UE implementation. Here, we suggest following the same mechanism as CA/DC.

Regarding the example in option 1, we doubt whether UE can report the actual TTD level to NW in time as it is very possible that the actual TTD is larger than CP and then smaller than CP now and then. So, the following proposal is suggested.

Proposal 1: When the actual TTD between two TAGs for multi-panel UL mTRP exceed the capability UE can support, not define additional requirements. It’s up to UE implementation on how to handle this case.

2.2 DL reference Timing for UL multi-DCI multi-TRP with two TAs

In last meeting, there’re some discussion [2] below on DL reference timing for UL multi-DCI multi-TRP with two TAs.

Issue 2-1-3: Reference timing - Proposals: - Option 1: (Nokia, Ericsson) - The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission. - Specify for each UL/joint TCI state the DL RS the UE must use for DL time tracking. - Option 2: (Apple, Nokia, MediaTek, ZTE, vivo, Huawei, Samsung) - In UL timing requirements, some clarification needs to be added to accommodate - Two DL reference timings are supported where each DL reference timing is associated with one TAG. - Two TAGs associated with different UL/joint TCI state. - Option 3: (MediaTek) - Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing - FFS whether RAN4 needs distinguish same or different DL timing reference in the discussion for 2 TAGs. - Option 4: (Huawei) - For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission. Note: FFS on whether single reference shall be allowed or not. More RAN1 input is expected for further RAN4 discussion. Reference from RAN1 agreement [3] Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured. Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH AP/SP/P SRS Proposal 2.0 from RAN1 #112bis-e For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the baseline feature is revised as follows: - UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

From current RAN 1 progress, the association of TAG and DL reference timing is still not very clear to us. We think much more RAN1 output is necessary. Here is our current understanding:

- One TAG implies one DL reference timing.

- Typically, Two TAGs imply two DL reference timing. For two TAGs with the same DL reference timing, we are not clear about the applicable scenarios. We are open to know the deployment detail before further requirement discussion.

From above analysis, the following proposal is suggested.

Observation 1: Each TAG is allowed to have its own DL reference timing. Typically, two TAGs may have different DL reference timing

Proposal 2: FFS whether RAN4 needs distinguish same or different DL timing reference in the discussion for 2 TAGs.

2.3 MTTD

In last meeting, there’re some discussion [2] on MTTD requirement for UE not support RTD > CP.

Issue 2-1-1: What is the assumption on M1/M2 for MTTD for UE not capable of supporting RTD>CP? - Proposals: - MTTD for UE not capable of supporting RTD > CP - Option 1: (MediaTek) - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=0 and M2=0 - Option 2: (Nokia, Samsung, Xiaomi, ZTE, Huawei, QC, Ericsson, vivo, Apple) - If UE supports sTxMP - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=1.6us and M2=0.5 us - If UE doesn’t support STxMP - Wait for RAN1 further progress for gap/scheduling restriction - No MTTD requirements for this case.

For option 2, the first bullet is ok to us. If UE supports STxMP, we can define the margin M1(1.6 us)/M2(0.5 us) for the MTTD requirements. If UE not capable of STxMP, UE may only have one FFT/RF to transmit multi-UL on multi-TRP in TDM. Therefore, some switching time is needed between the transmissions of the two TRPs. We understand that RAN1 is discussing whether to introduce a gap or scheduling restriction as shown below. We are fine not to define MTTD requirements if UE does not support STxMP with a condition that a time gap is introduced between two UL transmissions associated with two different TA values.

Proposal 3: For UE not capable of RTD>CP, if UE does not support STxMP, a switching time is necessary between two UL transmissions associated with two different TA values, and RAN4 does not need to define MTTD requirements

Reference from RAN1 agreement Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e: - Alt 1: Introducing a time gap X between two UL transmissions associated with two different TA values - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused - FFS: How X is determined - Alt 2: Reduce the overlapping duration of one of the two UL transmissions - Alt 3: Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap - Other alternatives are not precluded TBD: how to capture the downselected alternative(s) in the specifications in case specification impact is deemed needed.

- Summary

In this paper, the discussion of R18 MIMO is provided. We have the following proposal:

Proposal 1: When the actual TTD between two TAGs for multi-panel UL mTRP exceed the capability UE can support, not define additional requirements. It’s up to UE implementation on how to handle this case.

Observation 1: Each TAG is allowed to have its own DL reference timing. Typically, two TAGs may have different DL reference timing

Proposal 2: FFS whether RAN4 needs distinguish same or different DL timing reference in the discussion for 2 TAGs.

Proposal 3: For UE not capable of RTD>CP, if UE does not support STxMP, a switching time is necessary between two UL transmissions associated with two different TA values, and RAN4 does not need to define MTTD requirements

- Reference

- R4-2306362, WF on R18 NR MIMO RRM requirements, Samsung, RAN#106bis-e

- R4-2306254, Topic summary for [106bis-e][223] NR_MIMO_evo_DL_UL, Samsung, RAN#106bis-e

- RP-230102, 3GPP TSG RAN Meeting #99	, Rotterdam, Netherlands, March 20-23, 2023