3GPP TSG-RAN WG4 Meeting # 107	R4-2307611

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 5.25.3

Source: MediaTek Inc.

Title: Discussion on NR-DC with selective activation of cell groups via L3 enhancements

Document for: Discussion

- Introduction

RAN4#106bis-e had some discussion on the RRM requirements for selective activation of cell groups. There is only one open issue left [1]. In this contribution, we would like to share our view on the remaining issue.

- Discussion

Last meeting, RAN4 had some discussion on whether UE would evaluate the execution condition of other candidate PSCells while executing CPC but without consensus.

Issue 1-6: whether to consider UE is not evaluating the execution condition of other candidate PSCells while executing CPC - Candidate solutions: - Option 1: Yes. Event_DU for the subsequent change can be set to zero, if the execution condition for another PSCell is fulfilled while the UE is completing the PSCell change. - Option 2: No. According to TS37.340 and RAN2 agreements, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

To us, it is very clear that UE is not required to evaluate the execution condition of other candidate PSCells while executing CPAC according to 37.340.

From 37.340: Section 10.6

Proposal 1: UE is not required to evaluate the execution condition of other candidate PSCells while executing CPAC according to 37.340.

- Summary

In this paper, we have some discussion on NR-DC with selective activation of cell groups. We have the following proposal:

Proposal 1: UE is not required to evaluate the execution condition of other candidate PSCells while executing CPAC according to 37.340.

4 Reference

R4-2306396, “WF on NR Mobility Enhancements RRM requirements (part 2)”, Apple, RAN4#106bis e-meeting, April 17th – 26th, 2023.