3GPP TSG-RAN WG4 Meeting #107									              R4-2307644

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.11.2.4

Source:	Apple

Title:	On BWP w/o restriction - RRM core requirements for option B-1-2

Document for:	Discussion

# Introduction

In RANP#99 a new work item on BWP without restriction was approved [1]. Objectives are duplicated here for information:

- For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

RAN4 TU starts from April meeting. RAN4 had initial discussion and reached some agreement in RAN4#106-bis-e [1]. In this contribution, we focus on the remaining issues of option B-1-2.

# Discussion

The most controversial issue is about interruption design:

## Issue 5-2: Interruption requirements for supporting option B-1-2

<Way forward >:

- Proposals

  - Option 1:

    - Introduce symbol level interruption requirement for Option B-1-2, which is equal to the symbol length of MG for CD-SSB.  The portion of interrupted symbols would be decided by the SSB periodicity for BM/RLM/BFD.

  - Option 2:

    - Define explicit interruption length without mentioning interruption ratio.

  - Option 3:

    - Support NW to control the interruption location. Interruption ratio are not defined. Define requirements on length of each interruption. Use the following values as baseline

      - FR1: 0.5ms

      - FR2: 0.25ms

  - Option 4:

    - Interruption requirements for UE supporting option B-1-2 are specified with probability of missed ACK/NACK.

      - Alt 1: For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP) measurements based on SSB outside active BWP.

      - Alt 3: Specify 0.5% probability of missed ACK/NACK as the interruption requirements during BM/RLM/BFD measurements based on SSB outside the active BWP.

  - Option 5:

    - Define interruption ratio and interruption length for each interruption

      - Alt 2: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. And the length for each interruption shall not exceed the RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

  - Option 6:

    - RAN4 shall leverage the interruption requirements (NCSG and NFG) from L3 measurements to define the interruption requirements for RLM/BFD/BM measurements.

Regarding detailed interruption requirements, we see two alternatives:

- Define explicit interruption length without mentioning interruption ratio.

- Define interruption ratio in a general way.

For alterative 1), typical example could be 0.5ms in FR1 and 0.25ms in FR2. Note that how frequently UE shall perform L1 operation is configurable from network point of view. Once the configuration of L1-RS, SMTC and measurement gap is provided, the L1 operation interval is determined, at least from RAN4 requirement point of view network shall not the interruption ratio. Therefore, it is feasible to only define explicit interruption length without mentioning interruption ratio.

Alternatively, RAN4 can also clearly specify interruption in the format of equation so that it can cover all the configurations. For instance, the ratio can be calculated by X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is [0.5ms] in FR1 and [0.25ms] in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

Proposal 1: the following options can be considered to define interruption for option B-1-2:

- Option 1: define explicit interruption length without mentioning interruption ratio.

- Option 2: X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is [0.5ms] in FR1 and [0.25ms] in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

# Conclusion

In this contribution, we provide discussion on BWP without restriction – option B-1-2. After discussion, the following conclusions are provided:

Proposal 1: the following options can be considered to define interruption for option B-1-2:

- Option 1: define explicit interruption length without mentioning interruption ratio.

- Option 2: X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is [0.5ms] in FR1 and [0.25ms] in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

# References

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo

- R4-2303313, WF on BWP_withoutRestriction, vivo