3GPP TSG-RAN WG4 Meeting #107									              R4-2307645

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.25.1

Source:	Apple

Title:	Discussion on RAN1 LS on LTM

Document for:	Discussion

# Introduction

In RAN1#112bis-e, an LS [1] was approved to RAN4 on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, in which some RAN4 action is expected. RAN4 is asked to provide the feedback on the issue of time gap between a PDCCH order and the corresponding PRACH transmission.

In this contribution, we will provide our view on the related issues.

# Discussion

Related context from RAN1 LS is duplicated here:

- Time gap between a PDCCH order and the corresponding PRACH transmission RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM. RAN1 believes that this will require that the time gap is increased at least for the following scenario - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell RAN1 relies on RAN4: - to verify the need for the above additional latency and, if so, the corresponding value is needed. - to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH - to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed Potential RAN1 spec update will be based on RAN4’s feedback.

There are three actions for RAN4:

- Verify the need for the above additional latency and, if so, the corresponding value is needed.

There are two scenarios mentioned in the additional latency. Scenario 1 is that target candidate cell is not a current serving cell with PUCCH/PUSCH. Scenario 2 is that target candidate cell is inter-frequency with the current serving cell.

For both scenario 1 and 2, additional time for UE to apply the RRC RACH configuration. It was agreed in RAN2 [2] that RACH related configuration is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration:

RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch).

It is possible that network pre-configures RACH configuration for multiple candidate cells for potential early TA acquisition. UE does not need to apply the related parameters until it is indicated to perform RACH to target cell.

For scenario 1, some extra time is needed for UE to finish fine time tracking before sending the RACH preamble. Since the UL timing of RACH preamble transmission to target candidate cell is supposed to meet Te requirement, fine time tracking is necessary. As for scenario 2, given that target candidate cell is one of the current serving cells, UE is supposed to maintain fine time tracking. Therefore, extra time is not necessary.

Proposal 1: additional latency is needed for UE to apply RRC RACH configuration.

Proposal 2: additional latency is needed for fine time tracking if target candidate cell that is not a current serving cell.

- To investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH

This issue has already been raised during last RAN4#106-bis-e. The latest agreement can be found in [3]:

Issue 1-3-3: Interruption due to PDCCH ordered RACH before cell switch command < Agreement>: - Further discuss whether and how to define interruption and/or scheduling restriction requirements due to PDCCH ordered RACH before cell switch command once corresponding RAN1 design is stable.

Technically, interruption and/or scheduling needs to be introduced on source serving cells. RF tuning/retuning can be expected if target candidate cell is on a different carrier. Besides, from baseband capability point of view, as baseline, UE shall not be required to support simultaneous transmission of RACH to candidate cell and UL to source cell(s). The interruption/scheduling restriction would happen not only on RACH transmission occasion, but also on fine time tracking occasion (e.g. due to RF tuning/retuning). Exact interruption requirement can be discussed once the RACH procedure is stable enough.

Proposal 3: inform RAN1 that RAN4 will specify requirements of interruption and/or scheduling restriction on source serving cell(s) for PDCCH ordered RACH to candidate cell.

Proposal 4: exact requirements for interruption/scheduling can be discussed in RAN4 once the RACH procedure design is stable enough in RAN1/2.

- To verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed.

For information, definitions of each component in existing RACH delay is duplicated here from RAN1 spec:

If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec, where - NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - ∆BWPSwitching=0 if the active UL BWP does not change and ∆BWPSwitching is defined in [10, TS 38.133] otherwise - ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2 - Tswitch is a switching gap duration as defined in [6, TS 38.214] For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N2 assuming SCS configuration μ=0. For single cell operation or for operation with contiguous carrier aggregation in a same frequency band or for operation with non-contiguous carrier aggregation in a same frequency band if the UE is not provided with intraBandNC-PRACH-simulTx-r17, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot with respect to the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

If above propose 1 regarding additional time to apply RRC RACH configuration is agreed, then ΔBWPSwitching is no longer needed. Otherwise, it is still needed. ΔDelay is caused by RF tuning. It shall remain the same.

Proposal 5: ΔBWPSwitching can be 0 if additional latency is allowed for UE to apply RRC RACH configuration.

Proposal 6: ΔDelay shall remain the same.

# Conclusion

In this contribution, we provide analysis on time gap between a PDCCH order and the corresponding PRACH transmission. After discussion, the following conclusions are provided:

Proposal 1: additional latency is needed for UE to apply RRC RACH configuration.

Proposal 2: additional latency is needed for fine time tracking if target candidate cell that is not a current serving cell.

Proposal 3: inform RAN1 that RAN4 will specify requirements of interruption and/or scheduling restriction on source serving cell(s) for PDCCH ordered RACH to candidate cell.

Proposal 4: exact requirements for interruption/scheduling can be discussed in RAN4 once the RACH procedure design is stable enough in RAN1/2.

Proposal 5: ΔBWPSwitching can be 0 if additional latency is allowed for UE to apply RRC RACH configuration.

Proposal 6: ΔDelay shall remain the same.

# References

- R1-2304276, LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, Fujitsu, MediaTek

- R2-2304553, Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM, Fujitsu, CATT

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R4-2303175, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.