3GPP TSG-RAN WG4 Meeting #107									              R4-2307653

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.26.2.1

Source:	Apple

Title:	Discussion on general aspects of R18 MUSIM

Document for:	Discussion

# Introduction

R18 MUSIM - General aspects were discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues.

In this contribution, we continue discussing the open issues.

# Discussion

The first remaining issue is about the scope:

Issue 1-1-1: Clarification on the scope

- Proposals

  - P1: Add the following note for the sentence “Case 2: Collisions between MUSIM gap and SMTC”

    - Note: The scope collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements

  - Support (Qualcomm Huawei Nokia MTK xiaomi vivo)

  - Not support (Ericsson)

  - FFS (Apple)

Agreements: No

Candidate options:

Recommendations: Discuss at next meeting

Issue 1-1-5: General rule to handle NW-A and NW-B procedures

- Proposals

  - P1: RAN4 to define the priorities for each procedure in either NW-A or NW-B in descending order as follow. The gaps or resources for higher priority procedures should be kept once the collision happens.

    - Level 1: One-shot RRM mobility procedures in NW-A, such as Handover/SCell activation/SI update;

    - Level 2: Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading;

    - Level 3: Measurements procedures for both NW-A and NW-B

Agreements: No

The simplest solution is to follow legacy gap design. However, we do see some issue on the approach of reusing existing methodology. The difference here compared to the legacy gap scenario includes:

- As more and more gaps can be configured, it becomes more and more challenging for NW A to avoid collision between some one-shot procedures e.g. SCell activation and measurement gaps. Considering two concurrent gaps in NW A and additional three periodic MUSIM gaps for NW B, there are already five periodic gaps.

- Dropping NW A procedure due to MUSIM gaps may not bring any benefit to NW A. This is unlike legacy gap, wherein at least mobility performance in NW A can be guaranteed in case of collision. If NW A may degrade too much due to MUSIM gaps, NW A may lose interest in providing MUSIM gaps to the UE.

Too avoid complicated design, one possible compromise could be that: from requirement point of view, RAN4 confirms that the scope of collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements. Besides, RAN4 clarifies in high level in our spec that during some one-shot procedure, MUSIM gaps are not expected to be enabled unless the one-shot procedure is not interrupted and can be completed in time. For instance, during SCell activation, UE is not expected to enable MUSIM gaps unless the existing SCell activation delay can still be met.

Proposal 1: from requirement point of view, RAN4 confirms that the scope of collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements.

Proposal 2: add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met.

Next issue is about MUSIM overhead:

Issue 1-1-2: MUSIM overhead

- Proposals:

  - Option 1: Do not define overhead cap for MUSIM gaps (Qualcomm vivo CMCC Ericsson Huawei Nokia Apple)

  - Option 2: Define overhead cap for MUSIM gaps. (xiaomi oppo)

    - Option 2a: Measurement requirement does not apply when more than one MUSIM gap is configured with MGRP = [20] ms (xiaomi)

Agreements: No

Candidate options:

Recommendations: Discuss at next meeting

This is different from concurrent gaps design, in which gap configuration is purely up to network assuming the configured patterns are supported by UE. However, in MUSIM gap configuration procedure, both UE and network involved. If UE doesn’t want to be configured with certain combo of MUSIM gaps, it would not request them in the first place. From network side, it can also refuse to configure MUSIM with very short MGRP to avoid throughput degradation.

Proposal 3: MUSIM overhead cap is unnecessary.

Next issue is about mandatory gap patterns:

Issue 1-1-4: Mandatory MUSIM gap patterns

- Proposals

  - P1: No need to discuss further whether to introduce mandatory MUSIM gap patterns (Qualcomm vivo oppo Apple MTK Huawei)

  - P2: RAN4 to define the mandatory MUSIM gap patterns (Ericsson Nokia Chapter CMCC)

Agreements: No

Candidate options:

Recommendations: Discuss at next meeting

We don’t think it is necessary to define any mandatory MUSIM gap patterns. Although the MUSIM gap patterns are requested by UE, it is not purely up to UE on which patterns to request. Purpose of MUSIM gaps are for network B operation, so the patterns requested by UE need to match the signal transmission configuration from network B. Different network may have different configuration e.g. in paging and SI transmission. We cannot assume some mandatory MUSIM gap patterns can cover all the scenarios.

Proposal 4: No need to discuss further whether to introduce mandatory MUSIM gap patterns.

# Conclusion

In this contribution, we provide further discussion on improvement on R18 MUSIM - general aspects. After discussion, the following conclusions are provided:

Proposal 1: from requirement point of view, RAN4 confirms that the scope of collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements.

Proposal 2: add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met.

Proposal 3: MUSIM overhead cap is unnecessary.

Proposal 4: No need to discuss further whether to introduce mandatory MUSIM gap patterns.

# References

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo