3GPP TSG-RAN WG4 Meeting #107	R4-2307659

Incheon, Korea, 22th ~ 26th May 2023

Title: 	On MRTD/MTTD requirement applicability for EN-DC/NE-DC

Source: 	Apple

Agenda item:	4.4

Document for:	Discussion

# Introduction

Issue 1-2-4: MRTD/MTTD requirement applicability for Rel-15/16/17 UE:

- Proposals

  - FFS the applicability of existing MRTD/MTTD requirements for different UE capabilities, i.e. UE supporting interBandMRDC-WithOverlapDL-Bands-r16 or not.

  - FFS whether to have further clarification in the spec.

In previous RAN4 meeting, the MRTD/MTTD requirement applicability for Rel-15/16/17 UE was discussed based on [1, 2], but no consensus has been reached. The following open issue [3] needs to be further discussed in RAN4#107. This contribution will present our views and give proposals on how to solve the confusion.

# Discussion

  - MRTD/MTTD requirement in Rel-15

The discussion on applicability of MRTD/MTTD requirement was initiated in relation to the intra-band non-collocated CA/EN-DC in Band 42/n77/n78 [1] and further discussed based on [2]. As mentioned, the discussion on the necessity of non-collocated inter-band EN-DC with overlapping DL bands deployment can date back to Rel-15. The agreement in Rel-15 is as shown in the WF [4], where it is stated for TDD-TDD intra-band that,

- Synchronous DC operation as well as NR DL/UL resource assignment to avoid DL/UL interference with LTE TDD is needed for intra-band LTE-NR DC.

- It is proposed to only consider collocation scenario for intra-band LTE-NR DC in Rel-15.

- UE shall support the synchronous TDD-TDD LTE-NR intra-band DC provided that the MRTD at the UE does not exceed values…

For inter-band EN-DC, the MTTD/MTTD requirement is specified for inter-band asynchronized operation and synchronous operation in 7.5.2/7.6.2 assuming non-collocated deployment (e.g. propagation time difference is 30us for synchronous case). The EN-DC_42-n77/78 combination has been introduced in Rel-15 [5]. These EN-DC combinations are like intra-band case although they are classified as inter-band in terms of band number. But the MTTD/MRTD requirement applicability was not clearly mentioned for TDD-TDD inter-band EN-DC with overlapping frequency. Synchronous DC operation between LTE and NR should be considered for such combination to avoid DL/UL interference. So, the requirement for asynchronous operation is not applicable to TDD-TDD EN-DC with overlapping frequency. In addition, the non-collocated case is not discussed in Rel-15. Hence, only MTTD/MRTD requirement for intra-band collocated EN-DC is applicable for Rel-15 UE supporting TDD-TDD inter-band EN-DC with overlapping frequency.

Observation 1: For TDD-TDD EN-DC with overlapping frequency, the MTTD/MRTD requirement for asynchronous operation in 7.5.2/7.6.2 is not applicable.

Observation 2: For Rel-15 UE supporting TDD-TDD EN-DC with overlapping frequency, only the MTTD/MRTD requirement for intra-band collocated case in 7.5.3/7.6.3 is applicable.

Proposal 1: It is proposed to clarify the requirement applicability in Observation 1&2 for MRTD/MTTD requirement from Rel-15.

  - MTTD/MRTD for inter-band non-collocated EN-DC with overlapping frequency

In Rel-16, type 2 UE was discussed in relation to FDD-FDD EN-DC_20-n28. The signaling interBandMRDC-WithOverlapDL-Bands-r16 was specified in 38.306 according to the outcome of that discussion. The type 2 UE is pointed to MRTD requirement in 7.6.2/7.6.5 which is based on asynchronous operation without differentiating FDD and TDD. It should be noted that the TDD-TDD inter-band non-collocated EN-DC/NE-DC combination with overlapping frequency was added to the signaling without discussion for any example TDD-TDD band combination. However, the asynchronous requirement is not applicable for TDD-TDD inter-band EN-DC/NE-DC with overlapping frequency due to cross link interference. Hence, the description needs to be updated according to the requirement applicability as discussed in 2.1.

Proposal 2: It is proposed to apply the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.5.5.1/7.6.2.1/7.6.5.1 for TDD inter-band EN-DC/NE-DC with overlapping frequency from R16.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| interBandMRDC-WithOverlapDL-Bands-r16 the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |

Proposal 3: It is proposed to update the description for interBandMRDC-WithOverlapDL-Bands-r16 as following:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| interBandMRDC-WithOverlapDL-Bands-r16 if the capability is reported，it indicate one of the following for FDD and TDD respectively - the UE supports FDD-FDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5], MTTD according to 7.5.2/7.5.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE) according to [4]. - the UE supports TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2.1/7.6.5.1 in 38.133 [5], MTTD requirement according to clause 7.5.2.1/7.5.5.1 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE) according to [4]. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |

# conclusion

This contribution discussed the MRTD/MTTD requirement applicability for EN-DC. The following observations and proposals are concluded.

Observation 1: For TDD-TDD EN-DC with overlapping frequency, the MTTD/MRTD requirement for asynchronous operation in 7.5.2/7.6.2 is not applicable.

Observation 2: For Rel-15 UE supporting TDD-TDD EN-DC with overlapping frequency, only the MTTD/MRTD requirement for intra-band collocated case in 7.5.3/7.6.3 is applicable.

Proposal 1: It is proposed to clarify the requirement applicability in Observation 1&2 for MRTD/MTTD requirement from Rel-15.

Proposal 2: It is proposed to apply the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.5.5.1/7.6.2.1/7.6.5.1 for TDD inter-band EN-DC/NE-DC with overlapping frequency from R16.

Proposal 3: It is proposed to update the description for interBandMRDC-WithOverlapDL-Bands-r16 as following:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| interBandMRDC-WithOverlapDL-Bands-r16 if the capability is reported，it indicate one of the following for FDD and TDD respectively - the UE supports FDD-FDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.27.6.5 in 38.133 [5], MTTD according to 7.5.2/7.5.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE) according to [4]. - the UE supports TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2.1/7.6.5.1 in 38.133 [5], MTTD requirement according to clause 7.5.2.1/7.5.5.1 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE) according to [4]. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |

# Reference

[1] R4-2219286, Discussion on RRM requirements for intra-band non-collocated EN-DC/NR-CA deployment, Samsung, RAN4#105

[2]	R4-2300239, On MRTD/MTTD requirement applicability for EN-DC/NE-DC, Apple

[3]	R4-2220398, WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA, Huawei, HiSilicon, Samsung, RAN4#105

[4] R4-1711964, WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15, Ericsson, MediaTek, RAN4#105

[5] 38.101-3, version fj1.