3GPP TSG-RAN WG4 Meeting #107 	R4-2307691

# Incheon, South Korea,  22nd May  - 26th May, 2023

Source:	Ericsson

Title:	Simulation results for FR1 4Tx demodulation requirements

Agenda item:	8.5.3.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have agreed on most parameters for the FR1 4Tx demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth 15kHz SCS: 5MHz and 50MHz, FFS on 20MHz 30kHz SCS: 10MHz and 100MHz, FFS on 40MHz |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Channel model and MCS |  | TDLB100-400: MCS2 TDLC300-100: MCS12 TDLA30-10: MCS17 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2.1-2 Simulation results with 70% of maximum throughput for 30kHz SCS, 40MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 1+1 | TDLB100-400 | 2 | A | -1.00 |
|  |  |  |  |  | B | -1.01 |
|  |  |  | TDLC300-100 | 12 | A | 12.52 |
|  |  |  |  |  | B | 12.64 |
|  |  |  | TDLA30-10 | 17 | A | 16.24 |
|  |  |  |  |  | B | 16.24 |
| 4 | 8 |  | TDLB100-400 | 2 | A | -4.43 |
|  |  |  |  |  | B | -4.41 |
|  |  |  | TDLC300-100 | 12 | A | 5.26 |
|  |  |  |  |  | B | 5.28 |
|  |  |  | TDLA30-10 | 17 | A | 8.71 |
|  |  |  |  |  | B | 8.71 |

# Conclusions

Simulation results for FR1 4Tx demodulation requirements are delivered.

# References

[1]	R4-2305890, WF on FR1 4Tx demodulation requirement, Ericsson, RAN4#106bis-e