3GPP TSG RAN WG4 Meeting #107	R4-2307710

Incheon, KR, May 22nd – May 26th, 2023

Agenda item:	8.9.3

Source: 	vivo

Title: 	Discussion on RRM core requirements for FR1-FR1 NR-DC

Document for:	Discussion

- Introduction

In the RAN4#106bis-e meeting, basically remain aspects of RRM core requirements for FR1-FR1 NR-DC were discussed and came to agreements. There are few issues left to be discussed. In this paper, we will provide our view on them.

- Discussion

The following issue was captured in the WF on R18 eFeRRM – RRM requirements for FR1-FR1 NRDC [1].

- RAN4 are to discuss the RRM requirements for FR1-FR1 NRDC - Whether to indicate the beam failure during SCG activation procedure (Issue 1-1-5) - General aspects related to BFD and RLM (Issue 1-1-1)

2.1 SCG activation/deactivation

- Whether to indicate the beam failure during SCG activation procedure (Issue 1-1-5) - Option 1: UE report beam failure of the PSCell during PSCell activation (after receiving the SCG activation message) - Option 2: Deprioritize this issue due to a corner case when BFD happens during activation procedure - Option 3: Ask RAN2 for clarifications - General aspects related to BFD and RLM (Issue 1-1-1) - Option 1: - A UE which has not detected either BFD or RLF on the deactivated PSCell, need no additional measurements at PSCell activation. - A UE which has detected either BFD or RLF on the deactivated PSCell is allowed additional measurements at PSCell activation. - Option 2: If UE has detected either BFD or RLF on the deactivated PSCell for RACH-less SCG Activation, UE fall back to RACH based activation.

In the last meeting, the case of beam failure during SCG activation procedure was raised. In our understanding, this is more likely a corner case which BF/RLF occurred after UE received the activation command since if the PSCell is known, the activation delay is quite short so that there is little chance of failure during this period. Even the failure is occurred, considering UE may not be able to complete RLM/BFD measurement within such shore duration, there has no chance to detect failure. However, from some companies’ perspective, more clarification from RAN2 is still necessary to solve this ambiguity for this case even though the activation period is quite short. The ambiguity is embodied in the fact that, During RACH-less activation procedure, if BF or RLF occurs after UE receive the RRC activation command, there have two interpretation here:

- If SCG to be activated is still regarded as at the deactivated state, BF will be indicted to NW via MCG.

- If SCG to be activated is regarded as at the activated state, UE will fall back to RACH-based activation.

From point of view of UE behaviour, our preference is to follow the first interpretation since UE needs to monitor PDCCH if SCG is at activated state whereas UE is not required to do that during SCG activation procedure. Yet considering there may have different understanding between UE and NW for this case and uncertainty on UE behaviour, we are fine to check with RAN2 for more clarification.

Proposal 1: RAN4 to ask RAN2 for clarification on the case that BFD happens during SCG activation procedure

- Conclusions

Proposal 1: RAN4 to ask RAN2 for clarification on the case that BFD happens during SCG activation procedure

- Reference

[1] R4-2306322 WF on RRM requirements for FR1+FR1 NR-DC, OPPO