3GPP TSG-RAN WG4 Meeting # 107	R4-2307756

# Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.858: Self-interference analysis for FR1 Wide Area BS

Agenda Item:	8.20.2.2.1

Document for:	Approval

- Introduction

SBFD feasibility study and RF impact from BS aspects were discussed in last meeting and WF for the feasibility from BS aspect was approved in [1].

Agreement:

- The TR section “10.2 Feasibility of FR1 Wide Area BS aspects” shall be further broken-down to harmonize the common understating from RAN4 if possible and summarize the input from companies.

In this contribution, we provide some text proposal for the section 10.2 Self-interference analysis based on our input in [2] and [3].

- Text Proposal

## 10.2 Feasibility of FR1 Wide Area BS aspects

### 10.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 10.2.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

| FR1 |  |  |  | Huawei |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS example 1 | Wide Area BS example 2 |
| BS TX Power dBPTX = ① dBm |  |  |  | 47 | 53 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 | 45 |
|  |  | Frequency isolation techniques used |  | DPD | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 | 80 |
|  |  | Spatial isolation techniques used |  | Spatial separation between TX/RX panel with absorbing material and choke structure. | Spatial separation between TX panel with absorbing material and choke structure. |
|  | Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Less than 0.5 dB loss | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -94 | -88 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A | 10 |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A | Analog filter is put after LNA |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -43 | -37 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | digital filter: 60-80 dB | sub-band analog filter: 10 dB digital filter: 60-80 dB |
|  |  | Frequency isolation techniques |  | digital filtering | sub-band analog filter and digital filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -10 | 0 |
|  |  |  | Rx IM3 contribution (dBm) | -109 | -121 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | ADC noise: -109 reciprocal phase noise mixing:-112 | ADC noise: -113 reciprocal phase noise mixing:-116 |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -105 | -111 |
|  | Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Less than 0.5 dB loss | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 | 15 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | -150.6 | -155.3 |
| Noise floor ⑩dBm |  |  |  | -96 dBm/20 MHz | -96 dBm/20 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm | -102 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 149 | 155 |
| SBFD configuration |  |  |  | DUD [40, 20, 40] | DUD [40, 20, 40] |
| Guardband assumption (if exist) |  |  |  | Existing SU | Existing SU |
| bandwidth over which suppression is achieved |  |  |  | Several hundred MHz | Several hundred MHz |

### 10.2.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

### 10.2.1.2.1	Huawei

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.2.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

For FR1 our analysis and evaluation is sumarized in Table 2.1-1, we show two examples for Wide Area BS to consider different max TX power. One is 47 dBm max TX power and the other is 53 dBm. The major difference for WA example 2 is the adoption of analog filter to counteract the higher RX blocking from TX sub-band.

Analogue filter prior to the LNA would introduce also insertion loss which will cause sensitivity loss. Hence it is not suitable. Meanwhile putting analogue filter after LNA could tolerance the high insertion loss. The filter can be RF analog filter in the front-end or base-band analogue filter before the ADC. In the example we use RF analog filter which is put after LNA. The filter performance can be found in Figure below. A Q-value of 1500 and 5 poles are assumed in the simulator. It can be found that the insertion loss is less than 5 dB and ~15 dB suppression is achieveable.

Figure 10.2.1.2-1: performence of an example sub-band fitler

As shown in the example below, putting analogue filter after LNA, since the front-end LNA can provide substantial gain on the wanted signal, the impact to overall noise figure is negligible (2.21dB vs 2.24 dB), and the RX IIP3 prior to the filter is high enough to cope with high blocking level, such as ~ -30 dBm.

Table 2.2.-1: Cascaded NF and IIP3

| Receiver | Band Filter | LNA | ATT | subband Filter |
| --- | --- | --- | --- | --- |
| GAIN(dB) | -1.20 | 25.00 | -2.00 | -5.00 |
| NF(dB) | 1.20 | 1.00 | 2.00 | 5.00 |
| OIP3(dBm) | 100.00 | 34.00 | 100.00 | 100.00 |
| C_gain(dB) | -1.20 | 23.80 | 21.80 | 16.80 |
| C_NF(dB) | 1.20 | 2.20 | 2.21 | 2.24 |
| C_OIP3(dBm) | 100.00 | 34.00 | 32.00 | 27.00 |
| C_IIP3(dBm） | 101.20 | 10.20 | 10.20 | 10.20 |

Analogue sub-band filter after LNA can provide the needed suppression for the receiver parts after the filter, and the impacts to RX sensitivity due to insertion loss is negligible. If analogue sub-band filter is adopted in the solution, the blocking performance can be improved at least 10 dB.

### 10.2.1.2.2	[Company Name]

### 10.2.1.2.3	[Company Name]

### 10.2.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

- Conclusion

In this contribution, we provide text proposal on RSIC analysis.

- Reference

[1] R4-2306005, “WF on implementation feasibility of SBFD”, Samsung

[2] R4-2301426,	“Feasibility study from BS aspect”, Huawei, HiSilicon

[3] R4-2305302, “Discussion on the feasibility of FR1 BS aspects”, Huawei, HiSilicon