3GPP TSG-RAN4 Meeting #107	R4-2307817

Incheon, KR, May 22 – May 26, 2023

Source:	Samsung

Title:	discussion and simulation results on 8Rx SDR requirements

Agenda item:	8.5.3.1.3

Document for:	Discussion

- Introduction

In RAN4 #106 and RAN4 #106bis-e meeting, general test scope, simulation assumptions for FR1 8Rx SDR requirements have some agreements in WF [1][2]. In this contribution, we share our views on open issues of FR1 SDR requirements.

- Discussion

  - Simulation assumptions

Agreements for simulation assumptions in WF [1] is as below Table2-2. Simulations in this document are based on this simulation assumptions.

Table 2-2: Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1005} for 6 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

  - Maximum MCS for 256QAM 8 layers

About the 8Rx SDR feasibility of 256QAM, below agreement is achieved in RAN4#106bis-e [2].

Issue 3-4: Maximum MCS and MIMO layers for 256QAM - 2 and 4 layers: reuse the requirements defined in Table 5.5A-5 of TS 38.101-4 - Candidate options for maximum achievable MCS for 8 layers - Option 1: MCS22 - Option 2: MCS24

Simulation results for 256QAM 8 MIMO layers SDR test are as below. Considering the maximum achievable SNR for 64QAM is 30.4dB, and the RF impairment margin, it is more suitable to define MCS25 as the maximum achievable MCS for 256QAM 8 MIMO layers. Therefore, we prefer to define MCS25 as the maximum achievable MCS for 256QAM 8 MIMO layers, and we could also accept other proposals with lower MCS level than MCS25.

Proposal 1: Define the maximum achievable MCS for 256QAM 8Rx 8 layers as no more than MCS25.

  - Maximum MCS and MIMO layers for 1024QAM

About the 8Rx SDR feasibility of 1024QAM, below agreement is achieved in RAN4#106bis-e [2].

Issue 3-5: Maximum MCS and MIMO layers for 1024QAM - Introduce SDR requirements for 1024QAM at least for 2 MIMO layers - Candidate options - Option 1: Only 2 layers: - Option 1a: MCS23 - Option 1b: MCS24 - Option 2: Both 2 and 4 layers: - Option 2a: MCS23 - Option 2b: MCS24

Simulation results for 1024QAM 2 and 4 MIMO layers SDR test are as below. Considering the maximum achievable SNR for 1024QAM is 32dB, and the RF impairment margin, it is more suitable to define MCS24 as the maximum achievable MCS for 1024QAM 2 MIMO layers, and define MCS23 as the maximum achievable MCS for 1024QAM 4 MIMO layers.

Proposal 2: Propose the maximum achievable MIMO layers for 1024QAM 8Rx is layer 4.

Proposal 3: Define the maximum achievable MCS for 1024QAM 8Rx 2 layers as no more than MCS24.

Proposal 4: Define the maximum achievable MCS for 1024QAM 8Rx 4 layers as no more than MCS23.

  - MCS indexes for 8 MIMO layers

According WF [2] and the simulation results in section 2.2, and considering the theoretical upper bound MCS in table 2-1 [3] and Table 5.5A-5 in 38.101-4, we propose below MCS indexes for 8 MIMO layers.

Proposal 5: Define MCS indexes for 8 MIMO layers as

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2, 4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

  - MCS indexes for 1024QAM 8Rx

According the simulation results in section 2.3, and considering the theoretical upper bound MCS in table 2-1 [3] and Table 5.5A-6 in 38.101-4, we propose below MCS indexes for 8 MIMO layers.

Proposal 6: Define MCS indexes for 1024QAM 8Rx as

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 22 |
|  | 2 | 10 | 0.75 | 20 |
|  | 2 | 10 | 0.4 | 10 |
|  | 4 | 10 | 1 | 23 |
|  | 4 | 10 | 0.8 | 21 |
|  | 4 | 10 | 0.75 | 19 |
|  | 4 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Conclusion

In this contribution, we provide analysis and views on 8Rx SDR requirements for remaining open issues. The proposals could be summarized as:

Proposal 1: Define the maximum achievable MCS for 256QAM 8Rx 8 layers as no more than MCS25.

Proposal 2: Propose the maximum achievable MIMO layers for 1024QAM 8Rx is layer 4.

Proposal 3: Define the maximum achievable MCS for 1024QAM 8Rx 2 layers as no more than MCS24.

Proposal 4: Define the maximum achievable MCS for 1024QAM 8Rx 4 layers as no more than MCS23.

Proposal 5: Define MCS indexes for 8 MIMO layers as

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2, 4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Proposal 6: Define MCS indexes for 1024QAM 8Rx as

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 22 |
|  | 2 | 10 | 0.75 | 20 |
|  | 2 | 10 | 0.4 | 10 |
|  | 4 | 10 | 1 | 23 |
|  | 4 | 10 | 0.8 | 21 |
|  | 4 | 10 | 0.75 | 19 |
|  | 4 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Reference

[1] R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #106 meeting

[2] R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #106bis-e meeting

[3] R4-2304901, “discussion and simulation results on 8Rx SDR requirements”, Samsung, 3GPP RAN4 #106bis-e meeting

- Appendix

SDR related simulation results

For 2 and 4 MIMO layers of 256QAM 8Rx cases, the simulation results are as below.