3GPP TSG-RAN4 Meeting #107	R4-2307818

Incheon, KR, May 22 – May 26, 2023

Source:	Samsung

Title:	discussion on advanced receiver assumption and NWA signaling for MU-MIMO

Agenda item:	8.19.1.1

Document for:	Discussion

Introduction

Last RAN4 #106bis-e meeting is the second meeting for MU-MIMO advanced receiver WI, in which reference receiver assumption, required information, signaling for network assistant information (NWA) and simulation assumption for MU-MIMO scenario have some agreements in WF [1]. Due to the timeline schedule, NWA related topics especially for DCI related signaling topics should be prioritized to get agreements in Q2.

In this contribution, we share our views on open issues of MU-MIMO required information.

### Sub-topic 1-1 Reference receiver assumptions Issue 1-1-1: Reference receiver assumption for R-ML Candidate options: - Option 1: UE perform RML algorithm for serving and all co-scheduled UEs in the cell - Option 2: R-ML receiver in terms of total layer (serving + interfering) and modulation order - Option 3: UE performs joint detection on layers of one additional co-scheduled UE in addition to its own layers on the same frequency and time resource as its own allocation - Option 4: Limit the number of co-scheduled UE is no more than 1 and the number of interference layers are no more than 2 Way forward - This issue is highly related to how UE could obtain each requirement information and how NWA is designed. - Discuss how to obtain each of the needed parameters. Issue 1-1-2: Reference receiver Candidate options: - Option 1: Down-select R-ML as a candidate reference receiver to define phase II requirements - Option 2: To be decided later Way forward - To be discussed in the next meeting. #### Sub-topic 1-2-2: Information required for both E-MMSE-IRC and R-ML Issue 1-2-2-1: The presence of co-scheduled UE - Proposals on how this information could be obtained by the UE: - Option 1: Blind detection should be considered - Option 2: By assistant information signalling - Option 3: Through signalling of other parameters or by blind detection to the DMRS port information to be discussed - Proposals on how the NWA is signalled (if introduced): - Option 1: DCI-based signalling jointly with modulation order and DMRS port - Option 2: DCI-based signalling jointly with only modulation order - Option 3: 1 bit RRC signalling to indicate whether the DMRS port is used for the co-scheduled UE Way forward - Postpone this issue after the agreements of other information are reached. Issue 1-2-2-2: The DMRS sequence information for the co-scheduled UE Way forward - For the DMRS configuration parameter including: DMRS type, DMRS additional position, maximum length: - Restriction already exists in RAN1 specification (TS38.214), thus signaling is not needed. - For the scrambling ID and nSCID information: - Assume same as that of the target UE agreed as RAN4 default assumption - FFS on the signaling to inform UE if RAN4 default assumption not valid for any of the co-scheduled UE: - Option 1: 1-bit RRC signaling - Option 2: Implied by DCI signaling on modulation order (if introduced) - Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits Issue 1-2-2-3: The DMRS port information for the co-scheduled UE GTW agreements on Apr 17th: - Dedicated DCI signaling is not preferred for the DMRS port information - FFS whether assistant RRC signalling can be introduced to reduce the BD complexity and/or maintain reasonable CE performance for target UE - Companies are encouraged to further evaluate BD performance including the detailed assumption: - Number of co-scheduled UE for BD - Time/frequency location of co-scheduled UE Way forward - Study the BD performance together with FDRA information as recommended in Issue 1-2-2-8. - FFS whether to introduce additional assistant RRC signalling to restrict the BD complexity. Issue 1-2-2-4: PRB bundling size for the co-scheduled UE Way forward - UE needs to know the PRB bundling size of co-scheduled UEs if different from target UE - How could be obtained - Assume the PRB bundling size of co-scheduled UEs is same with that of target UE - FFS on the signaling to inform UE if RAN4 default assumption not valid: - Option 1: 1-bit RRC signaling - Option 2: No signaling is required. - Option 3: Implied by DCI signaling on modulation order (if introduced) Issue 1-2-2-5: DMRS power boosting for the co-scheduled UE Way forward - DMRS power boosting should be the same for both target and the co-scheduled UE. - FFS on the signaling to inform UE if RAN4 default assumption not valid: - Option 1: 1-bit RRC signaling - Option 2: Implied by DCI signaling on MO (if introduced) - Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits - Option 3: No signaling is required. Issue 1-2-2-7: Time domain resource allocation information of the co-scheduled UE Way forward - UE assumes the same PDSCH symbols are allocated to the target and the co-scheduled UEs - FFS on the signaling to inform UE if RAN4 default assumption not valid, by: - Option 1: 1-bit RRC signalling - Option 2: Implied by DCI signaling on MO (if introduced) - Option 2A: If “no co-schedule UEs are presented in the allocated resource to the target UE” is signaled in DCI, combine this information in the same signaling without additional bits - Option 3: No signaling is required Issue 1-2-2-8: Frequency domain resource allocation information of the co-scheduled UE Way forward - UE should know the FDRA information of the co-scheduled UE - Study the R-ML performance with BD to the DMRS port information as well as the FDRA information of the Co-UE (Low priority for the May meeting). Evaluation assumptions are as below: - 1 Co-UE - Detection granularity – up to UE implementation - Select the following two cases based on the agreed simulation assumption: - Case 2 (rank 1+1 2T2R QPSK interference TDLC300-100 random precoding) - Case 9 (rank 2+2 4T4R 64QAM interference TDLA30-10 orthogonal precoding) - Two cases for the FDRA of the co-UE: - Full CHBW allocation (52PRBs) - Partial CHBW allocation (0~24 PRBs) - Note: Assume that the R-ML has known all the other required information and all the agreed default assumptions are valid - Companies are encouraged to bring simulation results. #### Sub-topic 1-2-3: Additional information required for R-ML Issue 1-2-3-1: The modulation order information of the co-scheduled UE Way forward - The following additional assumptions to the R-ML receiver can be agreed: - Within each PRB/PRG, UE applies R-ML to all interference layers with prior information that all interference layers have same modulation order - FFS whether to consider the case with interference layers have different modulation orders within one or more PRBs. - Evaluation assumptions of the MO BD study: - 1 Co-UE - Detection granularity – up to UE implementation - Following cases: - Rank 1+1, 2T2R, MCS 13 for the target UE, QPSK interference, TDLC300-100, random precoding - Rank 2+2, 4T4R, MCS 17 for the target UE, 16QAM interference, TDLA30-10, orthogonal precoding - Rank 1+1, 2T2R, MCS 13 for the target UE, 16QAM interference TDLC300-100 random precoding (Optional) - Full CHBW allocation (52PRBs) FDRA of the co-UE: - Note: Assume that the R-ML also needs to perform DMRS port and FDRA information BD and all the agreed default assumptions are valid. - Companies are encouraged to bring simulation results for the next meeting. - With this MO BD study, the following is not precluded: - The possibility of full signalling of modulation order and/or other information. - The possibility of non-dynamic NWA signalling (i.e., non-DCI) solutions. - For this MO BD study, companies are encouraged to take all proposals from Issue 1-2-3-2 into consideration. Issue 1-2-3-3: RS location information of the co-scheduled UE Way forward - UE can assume the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE - FFS whether to consider RRC signalling to inform UE whether the default assumption is needed Issue 1-2-3-2: Content of the network signalling on modulation order (for information) - Proposal 1: Signalling Overhead; DCI signalling Info.; Antenna Port = 1000 + DMRS Port (P). 2 bits; maxMIMO-Layers = 2; 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM; P = {0, 1}. 6 bits; maxMIMO-Layers = 4; For 3 interfering ports in cyclic order from desired layer, each 2 bits represent as 00: No interference presence 01: Interference with QPSK 10: Interference with 16QAM 11: Interference with 64QAM or 256 QAM; P = {0, 1, 2, 3}. - Proposal 2: Use examples listed in below as start point - Example 1: Bitfield; coUE modulation order; UE behavior. 000; No UE which has same DMRS sequence with target paired; SU-MIMO (MMSE-IRC). 001; PRB aligned (Single coUE modulation per PRB); UE fallback to MMSE-IRC or R-ML with blind detection with only 4 hypotheses (UE dependent). 010; Bandwidth aligned, QPSK; R-ML without modulation order detection. 011; Bandwidth aligned, 16-QAM; R-ML without modulation order detection. 100; Bandwidth aligned, 64-QAM; R-ML without modulation order detection. 101; Bandwidth aligned, 256-QAM; R-ML without modulation order detection. 110; Bandwidth aligned, 1024-QAM; R-ML without modulation order detection. 111; PRB not aligned (Multiple coUE modulations per PRB); UE fallback to MMSE-IRC or R-ML with blind detection (UE dependent). - Proposal 3: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs, or signal the maximum MCS index table (64QAM, 256QAM or 1024QAM table) used among all the (potential) co-scheduled UEs. - Proposal 4: Include blind detection opportunities for modulation order detection using semi-static DUT resource allocation, where DUT is not scheduled with PDSCH, but co-scheduled UEs are scheduled with PDSCH. - Proposal 5: Introduce DCI signaling to assist blind detection, such as 1 bit modulation order set information for each co-scheduled UE to reduce search space, for example, when gNB decide 16QAM for a co-scheduled UE, then gNB transmit 1 bit to indicate the modulation order of this co-scheduled UE is in the set {16QAM, 64QAM} Way forward - To be discussed in the next meeting if introduced #### Sub-topic 1-2-4: Signalling for network assistant information if introduced Issue 1-2-4-1: Signalling for the network assistant information (If introduced) Candidate options: - Option 1: Only consider RRC or MAC-CE based network assistance signalling - Option 2: DCI - Option 3: FFS once it is agreed which information is to be signalled - Option 4: Some of the information could be carried by DCI and others carried by higher layer Way forward - Discuss separately for each parameter Issue 1-2-4-2: Granularity of the network assistant signalling (If introduced) Candidate options: - Option 1: The granularity of the network assistant signalling should be the wideband - Option 2: FFS until it is agreed which information is to be signalled Way forward Further discuss

Discussion

Considering more than one information and network assistant signaling are required for MU-MIMO advanced receiver (E-MMSE-IRC or R-ML), and the MU-MIMO advanced receiver is not mandatory feature for UE, thus it is better to notify the network if this UE support MU-MIMO advanced receiver or not in case the network waste resource to send unnecessary signaling. If UE has MU-MIMO advance receiver capability, network could transmit MU-MIMO related network assistant signaling to this UE to assist the interference reduction, otherwise, network has no need to transmit MU-MIMO related network assistant signaling to this UE.

Proposal 1: Introduce new UE capability about MU-MIMO advanced receiver, in case the network transmit MU-MIMO related NWA to the UE without MU-MIMO advanced receiver capability.

Reference receiver assumptions

Reference receiver assumption for R-ML

At least for the DL MU-MIMO algorithm in Rel-15 product, the number of co-scheduled UE could be more than 1 and the number of interference layers are not limited to 2, thus it sees for Rel-18 option 4 (Limit the number of co-scheduled UE is no more than 1 and the number of interference layers are no more than 2) is a technology retrogression. Similarly, if we only focus on one additional co-scheduled UE as option 3 mentioned, the blind detection of DMRS ports may could not reflect the reality of more than one co-scheduled UEs scenarios. Option 2 is realizable assuming blind detection for DMRS ports and scrambling IDs, but with concern about the blind detection performance, we prefer option 1 as the assumption for R-ML with the necessary signaling introduced.

Proposal 2: Use option 1 (UE perform R-ML algorithm for serving and all co-scheduled UEs in the cell) as reference receiver assumption for R-ML if necessary signaling introduced.

Reference receiver

Based on the simulation results summary in [2], R-ML algorithm has better performance than E-MMSE-IRC algorithm, while R-ML algorithm with more information required than E-MMSE-IRC algorithm. Thus if we target to define requirements in phase II for only one advanced receiver, we propose to accept option 1. But if we don’t have limit of the number of requirements, in case some UE could not support R-ML or could not receive modulation order related information which is need by R-ML correctly, considering E-MMSE-IRC algorithm could also get some benefit compared with legacy IRC algorithm, we could consider to define two sets of requirements for MU-MIMO advanced receivers in phase II.

Proposal 3: Support option 1 (Down-select R-ML as a candidate reference receiver to define phase II requirements) if we plan to define requirements in phase II for only one advanced receiver.

Proposal 4: Do not down-select candidate reference receivers if we consider to define two sets of requirements for MU-MIMO advanced receivers in phase II.

Information required for both E-MMSE-IRC and R-ML

The presence of co-scheduled UE

For both E-MMSE-IRC and R-ML advanced receiver, there is no doubt that UE should know the presence of MU-MIMO. But it seems no need to introduce explicit network assistant information signaling for the presence of co-scheduled UE, as the presence of co-scheduled UE could be implicitly indicated by other network assistant information signaling, for example DMRS port information or modultion order information of co-scheduled UEs. If introduced DMRS port or modultion order information is for each co-scheduled UE, we not only could get the information of the presence of co-scheduled UE, but also could get the number of co-scheduled UEs. As the pairing is changing every DL slot, it is more suitable to use DCI-based signaling.

Proposal 5: Through DCI-based signaling of other parameters if introduced to get the presence of co-scheduled UE.

The DMRS sequence information for the co-scheduled UE

RAN1 specification has restrict the actual number of front-loaded DMRS symbols, the actual number of additional DMRS, the DMRS symbol location and DMRS configuration type should be same for paired UEs. In order to get the DMRS sequence of the co-scheduled UE, the DMRS sequence initialization seed nSCID ∈ {0, 1} and the DMRS sequence scrambling IDs NID0,NID1 should be known by target UE. DMRS sequence initialization seed only need 1 bit DCI signaling for each co-scheduled UE, but each DMRS sequence scrambling ID need 16 bits RRC signaling for each co-scheduled UE. Considering the overhead, it is impossible to use explicit network assistant information signaling for the DMRS sequence initialization seed and scrambling ID(s) information. But current RAN1 specification don’t define restriction for the DMRS sequence initialization seed and scrambling ID(s) directly like other parameters (the actual number of front-loaded DMRS symbols, the actual number of additional DMRS, the DMRS symbol location and DMRS configuration type), which may generate different DMRS sequences for paired UEs. If paired UEs use different scrambling ID(s) and sequence initialization seed for DMRS, all the discussions/solutions about enhanced receiver for MU-MIMO is meaningless. Therefore, in order to ensure the feasibility of advanced receiver for MU-MIMO, we prefer to introduce configuration restriction in RAN1 “The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the DMRS sequence initialization seed and scrambling ID(s)”. If RAN1 has concern about this configuration restriction, we could consider introducing default assumption which assume the same DMRS sequence initialization seed and scrambling ID(s) for paired UEs in RAN4 and 1 bit RRC signaling to let UE know if this default assumption is applied by network or not.

Proposal 6: Prefer to introduce configuration restriction in RAN1 “The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the DMRS sequence initialization seed and scrambling ID(s)”. If RAN1 has concern about this configuration restriction, we could consider introducing default assumption which assume the same DMRS sequence initialization seed and scrambling ID(s) for paired UEs in RAN4 and 1 bit RRC signaling to let UE know if this default assumption is applied by network or not.

The DMRS port information for the co-scheduled UE

UE should know the DMRS port information for the co-scheduled UEs, thus UE could get the DMRS sequence of the co-scheduled UE. But it seems dedicated DCI signaling for DMRS port information is not accepted, then UE should do blind detection for the DMRS port information. If the number of paired layers is limited to 4, the complexity of blind detection is acceptable. If the number of paired layers could reach 8, the complexity of blind detection will be increased. In order to reduce the overhead of DCI signaling, we prefer to introduce the DMRS ports related information signaling of the co-scheduled UEs which have the same frequency domain resources with the target UE. In this case, total 2*N bits DCI information is enough, in which N is the number of the co-scheduled UEs which have the same frequency domain resources with the target UE.

Proposal 7: Propose to introduce the DMRS ports related information signaling of the co-scheduled UEs which have the same frequency domain resources with the target UE.

Precoding granularity for the co-scheduled UE

Considering the channel estimation matrix of DMRS also include the precoding matrix W in PRB level, thus both the signal from the primary UE and the interference from co-scheduled UE could be estimated without knowing of precoding matrix, which means precoding matrix is transparent to UE. Furthermore, UE need to know the PRG (Precoding resource block group) in order to get better channel estimation, in which the UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs. Since RAN1 has defined the same PRG are used for DMRS ports in the same CDM group in 38.214 as below

Therefore, it is straight forward to assume the PRG-level grid of co-scheduled UEs is same with that of target UE, and if this default assumption is not applied by the network, 1 bit RRC signaling should be informed to the target UE, then the target UE could use the minimum PRG granularity for co-scheduled UEs to do channel estimation.

Proposal 8: Define default assumption that PRG-level grid of co-scheduled UEs is same with that of target UE, and if this default assumption is not applied by the network, 1 bit RRC signaling should be informed to the target UE, then the target UE should use the minimum PRG granularity (actually it is 2) for co-scheduled UEs to do corresponding channel estimation.

DMRS power boosting for the co-scheduled UE

For the DMRS power boosting, TS38.214 section 4.1 has description as: For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Clause 5.1.6.2. The DM-RS scaling factor  specified in Clause 7.4.1.1.2 of [4, TS 38.211] is given by.

Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

Thus we know the DMRS power boosting could be decided by ‘Number of DM-RS CDM groups without data’, which means the REs of DMRS CDM group(s) could not be reused by PDSCH. For example, if ‘Number of DM-RS CDM groups without data’ is 1, it means the REs of DMRS CDM group 0 could not be shared by PDSCH, while the REs of DMRS CDM group 1 could be shared by PDSCH. If we look into Table 7.3.1.2.2-1- Table 7.3.1.2.2-4 in TS 38.212 carefully, we could find out once the total layer of MU-MIMO is known by network, the ‘Number of DM-RS CDM groups without data’ is also decided. We assume gNB prefer to set the same number of CDM groups for paired UEs even there is no such restriction. Thus we think it is not necessary to introduce RRC signaling for this assumption.

Proposal 9: Define the same DMRS power boosting used for target UE and co-scheduled UEs as default assumption, and no need to introduce RRC signaling for this default assumption.

Time and Frequency domain resource allocation information of the co-scheduled UE

If current UE could get the time and frequency domain resource allocation information of all co-scheduled UEs, the better result of interference cancellation could be more expectable. But considering the overhead to transmit the frequency domain resource allocation information for each co-scheduled UE, and network usually prefers to pair UEs with similar frequency resource requirement, we would like to propose UE assumes the same OFDM symbols for PDSCH and the same frequency domain resource allocation for the target and the co-scheduled UEs, and introduce simple signaling to notify target UE if default assumption is not valid.

Proposal 10: Define default assumption to assume that the same time domain resource allocation for the target UE and the co-scheduled UEs, and 1 bit RRC signaling to inform the target UE if default assumption is not valid.

Proposal 11: Define default assumption to assume that the same frequency domain resource allocation for the target UE and the co-scheduled UEs, and 1 bit DCI signaling to inform the target UE if default assumption is not valid, then blind detection should by applied for the frequency domain resource.

Additional information required for R-ML

The modulation order information of the co-scheduled UE

UE with R-ML should know the modulation order information for each co-scheduled layer. Generally we have three methods to obtain this modulation order information: the first one is by blind detection, but the blind detection complexity grows rapidly with the growth of co-scheduled layers; the second one is by assistance information signaling with modulation order information, if the default assumptions (co-scheduled UEs and target UE have the same DMRS sequence, the same precoding granularity, the same DMRS power boosting, the same time and Frequency domain resource allocation), and 1024QAM is not considered as a candidate modulation order for paired UEs, it is feasible to transmit modulation order information of each co-scheduled UE directly as NWA signaling (2 bits for each co-scheduled UE per slot); the third one is to introduce some signaling to reduce the seach space, this method is more implenmentable with lower overhead, for each co-scheduled UE, 1 bit signaling indicating modulation order set to assit the modulation order detection.

Proposal 12: UE needs to know the modulation order related information by assistant information signaling, such as full signaling of modulation order of each co-scheduled UE or partial signaling of modulation order set of each co-scheduled UE.

RS location information of the co-scheduled UE

Proposal 13: UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE, and no RRC signaling is needed for this default assumption.

Content of the network signaling on modulation order

Actually the content of the network signaling on modulation order is related to the decisions about DMRS port information, time and frequency domain resource allocation information. Before we get conclusion of these parameters, as we believe network prefer to pair UEs required similar number of PRBs rather than pair UEs with the same modulation order, we prefer to define DCI signaling to assist modulation order detection, such as 2 bits modulation order information for each co-scheduled UE or 1 bit modulation order set information for each co-scheduled UE to reduce search space, for example, when gNB decide 16QAM for a co-scheduled UE, then gNB transmit 1 bit to indicate the modulation order of this co-scheduled UE is in the set {16QAM, 64QAM}.

Proposal 14: DCI signaling to assist modulation order detection: 2 bits modulation order information for each co-scheduled UE or 1 bit modulation order set information for each co-scheduled UE to reduce blind detection complexity, the total number of introduced DCI bits is 2*N or N for the whole scheduled bandwidth of target UE, in which N is the number of co-scheduled UEs.

Signaling for network assistant information

Signaling for the network assistant information

Due to the discussion in section 2.2 and section 2.3, we need to decide to use DCI or other signaling carried by higher layer for each introduced parameter separately, therefore we support option 4 and we suggest to move this topic into separate parameter discussions in information required sections.

Proposal 15: Support option 4 (Some of the information could be carried by DCI and others carried by higher layer) and move this topic into separate parameter discussions in information required sections.

Granularity of the network assistant signaling

Considering the overhead limitation, the granularity of the network assistant signaling should be the wideband.

Proposal 16: Support option 1 (The granularity of the network assistant signaling should be the wideband).

- Conclusion

In this contribution, we provide analysis and views on MU-MIMO required information for remaining open issues.

Proposal 1: Introduce new UE capability about MU-MIMO advanced receiver, in case the network transmit MU-MIMO related NWA to the UE without MU-MIMO advanced receiver capability.

Proposal 2: Use option 1 (UE perform R-ML algorithm for serving and all co-scheduled UEs in the cell) as reference receiver assumption for R-ML if necessary signaling introduced.

Proposal 3: Support option 1 (Down-select R-ML as a candidate reference receiver to define phase II requirements) if we plan to define requirements in phase II for only one advanced receiver.

Proposal 4: Do not down-select candidate reference receivers if we consider to define two sets of requirements for MU-MIMO advanced receivers in phase II.

Proposal 5: Through DCI-based signaling of other parameters if introduced to get the presence of co-scheduled UE.

Proposal 6: Prefer to introduce configuration restriction in RAN1 “The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the DMRS sequence initialization seed and scrambling ID(s)”. If RAN1 has concern about this configuration restriction, we could consider introducing default assumption which assume the same DMRS sequence initialization seed and scrambling ID(s) for paired UEs in RAN4 and 1 bit RRC signaling to let UE know if this default assumption is applied by network or not.

Proposal 7: Propose to introduce the DMRS ports related information signaling of the co-scheduled UEs which have the same frequency domain resources with the target UE.

Proposal 8: Define default assumption that PRG-level grid of co-scheduled UEs is same with that of target UE, and if this default assumption is not applied by the network, 1 bit RRC signaling should be informed to the target UE, then the target UE should use the minimum PRG granularity (actually it is 2) for co-scheduled UEs to do corresponding channel estimation.

Proposal 9: Define the same DMRS power boosting used for target UE and co-scheduled UEs as default assumption, and no need to introduce RRC signaling for this default assumption.

Proposal 10: Define default assumption to assume that the same time domain resource allocation for the target UE and the co-scheduled UEs, and 1 bit RRC signaling to inform the target UE if default assumption is not valid.

Proposal 11: Define default assumption to assume that the same frequency domain resource allocation for the target UE and the co-scheduled UEs, and 1 bit DCI signaling to inform the target UE if default assumption is not valid, then blind detection should by applied for the frequency domain resource.

Proposal 12: UE needs to know the modulation order related information by assistant information signaling, such as full signaling of modulation order of each co-scheduled UE or partial signaling of modulation order set of each co-scheduled UE.

Proposal 13: UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE, and no RRC signaling is needed for this default assumption.

Proposal 14: DCI signaling to assist modulation order detection: 2 bits modulation order information for each co-scheduled UE or 1 bit modulation order set information for each co-scheduled UE to reduce blind detection complexity, the total number of introduced DCI bits is 2*N or N for the whole scheduled bandwidth of target UE, in which N is the number of co-scheduled UEs.

Proposal 15: Support option 4 (Some of the information could be carried by DCI and others carried by higher layer) and move this topic into separate parameter discussions in information required sections.

Proposal 16: Support option 1 (The granularity of the network assistant signaling should be the wideband).

- Reference

[1] R4-2305914, “WF on advanced receiver for MU-MIMO scenario”, China Telecom, 3GPP RAN4 #106bis-e meeting

[2] R4-2304687, “simulation result collection for advanced receiver for MU-MIMO”, China Telecom, 3GPP RAN4 #106bis-e meeting