3GPP TSG-RAN4 Meeting #107	R4-2307819

Incheon, KR, May 22 – May 26, 2023

Source:	Samsung

Title:	discussion on absolute physical layer throughput with link adaptation

Agenda item:	8.19.2

Document for:	Discussion

- Introduction

In last RAN4 #106bis-e meeting, requirement specification, test parameters and simulation assumptions for absolute physical layer throughput with link adaption requirement have agreements in WF [1]. In document [2], we provide our simulation results based on WF [1]. In this contribution, we share our views on open issues of physical layer throughput requirement specification.

- Discussion

Based on the agreement of physical layer throughput tests methodology in WF [1] and simulation results summary in [3], we have following analysis and views.

  - Phy Layer TP requirement specification

Issue 2-1 Phy Layer TP test metric - Average SNR of impairments results to achieve T% of maximum throughput + X dB margin - Use Gspan = [2.5] dB to check if the results are aligned - The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs. Issue 2-1-1: Test point T (%) selection - Candidate test pool based on alignment simulation results in R4-2304257 Test Case; Candidate test point 1: T % (SNR, Gspan); Candidate test point 2: T % (SNR, Gspan). FR1 FDD 2x2; 10% (3.2 dB, 1.6 dB) 15% (6.5 dB, 2.1 dB); 40% (18.5 dB, 2.4 dB). FR1 TDD 2x2; 10% (3.6 dB, 1.9 dB) 15% (7.1 dB, 1.8 dB); 35% (17.5 dB, 2.4 dB). FR1 FDD 2x4; 15% (3.3 dB, 1.9 dB); 50% (15.2 dB, 2.5 dB). FR1 TDD 2x4; 15% (3.6 dB, 2.2 dB); 50% (15.4 dB, 2.8 dB) 35% (10.7 dB, 2.3 dB). FR2-1 (TDD 2x2); 15% (4.4 dB, 2.1 dB); 40% (15.1 dB, 1.8 dB). - Issue A. # of test point(s) - Option 1. Two test points for 2Rx and 4 Rx. - Option 2. Two test points for 2Rx and 1 test point for 4 Rx. - Option 3. Single test point for 2Rx and 4 Rx - Issue B. T(%) and outlier removal for Gspan <= 2.5 dB Issue 2-1-2: X dB margin - Option 1: Apply X dB margin per modulation order to impairment results X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM Note that the modulation order would be based on median value over companies’ own median statistics on CQI. - Option 2: Apply single X dB margin to impairment results (e.g. X=0.5 dB or else) WF for Issue 2-1-1 and 2-1-2 - Discuss in the next meeting with the following aspect based on the updated results if any. - SNR options considering uniqueness of test SNR coverage - T (%) based on alignment results considering Gspan - Final SNR with X dB margin is within range of [0 20] for FR1 and [0 16] for FR2. - It does not preclude the possibility of T(%) adjustment with [+- 5% steps] or removal of outlier results from Gspan perspective.

We already have the agreement on physical layer throughput test methodology “Average SNR of impairments results to achieve T% of maximum throughput + X dB margin” in RAN4 #105 meeting. Based on the simulation results summary [3], and considering the requirement is with link adaptation, it is benefit to cover more SNR test points, thus we prefer two test points for both 2Rx and 4Rx cases.

Proposal 1: For # of test points, we prefer option 1 (two test points for both 2Rx and 4Rx cases).

From current simulation results in [3], we could observe that the span at some T% of max TP points are much higher than 2.5dB. If we eliminate the minimum and maximum SNR values for the T% of max TP points, we could observe that the span will be limited in 2.5dB and the average SNR is almost same as before.

Observation 1: The span at some T% of max TP points are much higher than 2.5dB. If we eliminate the minimum and maximum SNR values for the T% of max TP points, we could observe that the span will be limited in 2.5dB and the average SNR is almost same as before.

Proposal 2: Test points: SNRs at 10% and 35% max TP for 2Rx cases (both FR1 and FR2); SNRs at 20% and 50% max TP for FR1 4Rx cases.

Proposal 3: For the SNR average and span calculation at T% of max TP, eliminate the minimum and maximum SNR values at T% of max TP.

For the X dB margin, as RAN4 requirements is using 0.5dB for QPSK/16QAM and 0.8dB for 64QAM/256QAM, we’d like to use these values for ATP with link adaptation requirements.

Proposal 4: For X dB margin, use option 1 (0.5dB for QPSK/16QAM and 0.8dB for 64QAM/256QAM).

  - Applicability and release independency

Issue 2-2: Applicability and release independence - Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15 - Option 1.a: Proposal considering declaration - Optional for Rel-15 and Rel-16 UEs based on declaration - Mandatory for all Rel-17 and forward UEs.. - Option 2: The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

Proposal 5: support Option 1a for applicability and release independency.

- Conclusion

In this contribution, we provide analysis and views on absolute physical layer throughput with link adaptation requirements for remaining open issues. Proposals and observations could be summarized as:

Proposal 5: Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15.

Proposal 1: For # of test points, we prefer option 1 (two test points for both 2Rx and 4Rx cases).

Observation 1: The span at some T% of max TP points are much higher than 2.5dB. If we eliminate the minimum and maximum SNR values for the T% of max TP points, we could observe that the span will be limited in 2.5dB and the average SNR is almost same as before.

Proposal 2: Test points: SNRs at 10% and 35% max TP for 2Rx cases (both FR1 and FR2); SNRs at 20% and 50% max TP for FR1 4Rx cases.

Proposal 3: For the SNR average and span calculation at T% of max TP, eliminate the minimum and maximum SNR values at T% of max TP.

Proposal 4: For X dB margin, use option 1 (0.5dB for QPSK/16QAM and 0.8dB for 64QAM/256QAM).

Proposal 5: support Option 1a for applicability and release independency.

- Reference

[1] R4-2305915, “WF on absolute physical layer throughput requirement”, Intel, 3GPP RAN4 #106bis-e meeting

[2] R4-2304904, “simulation results on absolute physical layer throughput with link adaptation”, Samsung, RAN4 #106bis-e meeting

[3] R4-2304257, “Summary of simulation results for NR UE ATP requirements”, Intel, 3GPP RAN4 #106bis-e meeting