3GPP TSG-RAN4 Meeting #107	R4-2307820

Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0338 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on link adaptation absolute physical layer requirements for FR1 TDD 2Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add requirements for FR1 TDD 2Rx labsolute physical layer requirement with link adaptation |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - add requirments in Clause 5.6 5.6.1 5.6.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No requirements for FR1 TDD 2Rx absolute physical layer requirement with link adaptation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 5.6.1 5.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

5.6	PDSCH absolute physical layer throughput with link adaptation requirements

5.6.1	1Rx requirements

(Void)

5.6.2	2Rx requirements

5.6.2.1	FDD

5.6.2.2	TDD

The performance requirements are specified in Table 5.6.2.2-3, with the addition of test parameters in Table 5.6.2.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purpose are specified in Table 5.6.2.2-1.

Table 5.6.2.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE absolute phycial layer performance with link adaptation | 1-1, 1-2 |

Table 5.6.2.2-2: Test parameters

| Parameter |  | Unit | Test 1-1 | Test 1-2 |
| --- | --- | --- | --- | --- |
| Test metric (T% of max throughputnote1 at target SNR) |  |  | [10%] | [35%] |
| CSI measurement channelsnote2 |  |  | As specified in Table A.4-2: Rank 1: TBS.2-3 Rank 2: TBS.2-4 |  |
| Tx EVM |  |  | 3% |  |
| TDD Slot Configuration |  |  | 7D1S2U S:6D+4G+4U |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 2 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 9 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |  |
|  | CodebookSubsetRestriction |  | Not configured |  |
|  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 5.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Note: Other common test parameters are defined in Section 6.1.2 Note1: The maximum throughout is defined as with TBS corresponding to CQI index 15 with rank 2. Note2: For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission |  |  |  |  |

Table 5.6.2.2-3: Minimum requirements for link adaption absolute physical layer

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | TDLA30-5 | ULA Low 2x2 | [10] | [TBD] |
| 1-2 | 40 / 30 | TDLA30-5 | ULA Low 2x2 | [35] | [TBD] |

5.6.3	4Rx requirements

5.6.3.1	FDD

5.6.3.2	TDD

<End of changes>