3GPP TSG-RAN WG4 Meeting #107                                                            R4-2307831

Incheon, Korea, 22 – 26 May 2023

Agenda Item:	8.19.1.2

Source:	Ericsson

Title:	On parameter assumptions for inter-user interference cancellation

Document for:	Discussion

# 1	Introduction

After last RAN4 #106bis-e meeting’s discussion online, several parameter configurations and assumption have been agreed for companies to do the phase I evaluations. Agreements have been captured in the agreed Way forward [1]. Thus, in this meeting, companies are encouraged to bring simulation results to compare the performance, and to see how much gain can be found for different candidate receivers.

In this contribution, we provided our views on down selecting some options listed in the Way forward [1]. Observations and proposals were captured in the summary section.

# 2	Discussion

The number of co-scheduled UEs

From demodulation processing point of view, there is no difference between having one or more co-scheduled UEs. From the previous discussion and evaluations, companies have observed enough gain from one or two reference receivers over the baseline no matter with configuring rank combination 1+1 and 2+2. In this case, we see it is sufficient to only consider 1 co-scheduled UE to move forward. We are open for the study on more than one co-scheduled UEs in phase I. However, we propose to only consider 1 co-scheduled UE for defining requirements in phase II.

Proposal 1: Consider only 1 co-scheduled UE in phase II

MCS for the target UE

As for the target UE, it was agreed to consider MCS13 and MCS17 for initial simulation. Here we listed the agreement in the agreed WF[1]:

Issue 2-1-4: MCS for the target UE Way forward - Cover MCS 13 for rank 1 and rank 2 for initial simulation - Cover MCS 17 for rank 2 for initial simulation - Further discuss whether to cover MCS 4 for rank 1 in the next meeting - The assumption can be updated later based on available results.

For MCS13, there were sufficient results to justify its validity from the previous meetings.

As for the MCS17, it was agree to have some initial evaluations. We shared our simulation results below:

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Baseline | E-IRC | RML |
| 4 | 10 / 15 | MCS17 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 17.1 | 16.7 | 16.2 |
| 5 | 10 / 15 | MCS17 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 17.1 | 16.5 | 16.1 |

From the simulation results we can observe that for scenario rank 2+2, the performance of MCS17 with 2x2 ULA low can have x.xdB gain for E-IRC receiver and x.xdB gain for R-ML receiver.

Observation 1: For scenario rank 2+2 the performance of MCS17 with 2x2 ULA low can have x.xdB gain for E-IRC receiver and x.xdB gain for R-ML receiver

Modulation order for co-scheduled UE

As for the modulation for the co-scheduled UE, following agreements have been made in the last meeting according to the agreed WF [1]:

Issue 2-1-5: Modulation order for the co-scheduled UE Way forward - Maintain last meeting status from the modulation order perspective: For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose) for initial simulation - For rank 1+1: QPSK (high priority) - For rank 2+2: 64QAM (high priority) - For rank 2+2: QPSK (high priority) - For rank 1+1 (64QAM) +1 (QPSK) (lower priority) - For rank 1+1 (64QAM) +1 (16QAM) (lowest priority) - Other options on the modulation order for co-scheduled UE are not precluded. - These assumptions can be updated in the next meeting based on available simulation results.

Comparing with last meeting, there are additional cases for configuring QPSK for the modulation order for the co-scheduled UE. Here we shared our simulation results:

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Baseline | E-IRC | RML |
| 2 | 10 / 15 | 16QAM, 0.48 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 12.9 | 12.8 | 12.6 |
| 3 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 12.9 | 11.7 | 10.3 |
| 4 | 10 / 15 | MCS17 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 17.1 | 16.7 | 16.2 |
| 5 | 10 / 15 | MCS17 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 17.1 | 16.5 | 16.1 |

From the simulation results we can observe that configuring QPSK for the co-scheduled UE can have more gain comparing wih 64QAM for the co-scheduled UE. Thus, we suggest to consider QPSK for the co-scheduled UE for scenario: rank 2+2.

Observation 2: Configuring QPSK for the co-scheduled UE can have more gain comparing wih 64QAM for the co-scheduled UE

Proposal 2: Consider QPSK for the co-scheduled UE for scenario: rank 2+2

# 3	Summary

In summary, we provided our views on aspects listed above. Meanwhile, we also shared suggested receiver assumptions for candidate advanced receivers based on some initial simulation results. We summarized our observations and proposals as follows:

Proposal 1: Consider only 1 co-scheduled UE in phase II

Observation 1 : For scenario rank 2+2 the performance of MCS17 with 2x2 ULA low can have x.xdB gain for E-IRC receiver and x.xdB gain for R-ML receiver

Observation 2 : Configuring QPSK for the co-scheduled UE can have more gain comparing wih 64QAM for the co-scheduled UE

Proposal 2: Consider QPSK for the co-scheduled UE for scenario: rank 2+2

# References

[1]	R4-2305914: WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106bis-e