3GPP TSG-RAN WG4 Meeting #107                                                            R4-2307833

Incheon, Korea, 22 – 26 May 2023

Agenda Item:	8.19.1

Source:	Ericsson

Title:	TP to TR 38.878: Link level simulation results for inter-user interference cancellation

Document for:	Approval

# 1	Introduction

This TP provided the “Link level simulation results” section for Inter-user interference cancellation for MU-MIMO.

# References

[1]	R4-2305914: WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106bis-e

START OF TEXT PROPOSAL

### 4.4.2	Link level simulation results

In this sub-clause, link level simulation results from different companies are collected for analysis of PDSCH performance in scenario with inter-user interference for MU-MIMO. The link level analysis of PDSCH performance is performed under assumptions from sub-clause 4.4.1.

The detailed simulation results from different companies are provided in the attached file ‘Attachment 1 - R4-2304687 Simulation result collection for advanced receiver for MU-MIMO’

Table 4.4.2-1 provides the summary of simulation results from different companies.

Table 4.4.2-1: Summary of simulation results

| Co-scheduled UE number | Rank |  | MCS target UE (MCS Table 1) | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | FDRA of the Co-UE | SPAN |  |  | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | target UE | Co-scheduled UE |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) | Gain of R-ML | Gain of E-IRC |
| 1 | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) |  |  |  |  |  |
|  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  |  |  |  |  |  |
|  |  |  | 17 | 64QAM |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 for each Co-scheduled UE | 13 | 64QAM and QPSK | 4Tx 4Rx XPL medium | TDLC300-100 | random |  |  |  |  |  |  |
|  |  |  |  | 64QAM and 16QAM |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Partial CHBW allocation (0~24 PRBs) |  |  |  |  |  |
|  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal | Full CHBW allocation (52PRBs) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Partial CHBW allocation (0~24 PRBs) |  |  |  |  |  |
|  | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) |  |  |  |  |  |
|  | 2 | 2 | 17 | 16QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  |  |  |  |  |  |
|  | 1 | 1 | 13 | 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 | random |  |  |  |  |  |  |

END OF TEXT PROPOSAL