3GPP TSG-RAN WG4 Meeting #107                                                            R4-2307834

Incheon, Korea, 22 – 26 May 2023

Agenda Item:	8.14.5.2

Source:	Ericsson

Title:	On UE demodulation requirements for ATG network

Document for:	Discussion

# 1	Introduction

In last RAN4 #106bis-e meeting, companies had several discussions on the scope and the general issues for UE demodulation requirements for ATG network. A Way Forward [1] was agreed and carried all the agreements and open issues. Followings are agreed assumptions during last meeting’s discussion:

Issue 1-1: Scenario Agreement: - Focus on ATG deployment scenario which aligned with co-existence session, other scenarios (e.g., the scenario with large Doppler jumping in an ATG cell) can be considered in the future enhancement when needed. Issue 1-2: UE assumption Tentative agreements: - Take ATG UE pre-compensation on UL frequency shift and UL timing shift for ATG demodulation as baseline. - Take ATG UE pre-compensation on DL frequency shift for ATG demodulation as baseline. Issue 1-3-1: Channel model Agreement: - At least single path AWGN channel with doppler Issue 1-3-2: Doppler shift assumption: Agreement from GTW: - Align with RF session agreement on frequency error requirements with below values as starting point - For FDD channel model, set Doppler as [220Hz] - For TDD channel model, set Doppler as [400/500Hz]. Issue 2-6: DMRS Agreement: - 1 front loaded DMRS + 1 Additional DMRS position for both FDD and TDD test cases Issue 2-7: koffset Agreements: - koffset value 2ms, the unit and value can be revised after RAN2 confirm whether SIB19 can be reused or not. Issue 2-9: Switching point Agreements: - Do not consider switching point for ATG UE demodulation evaluation and requirements Issue 2-10: alternatives to evaluate the Doppler shift tracking ability - For UL: There is no necessity to evaluate the Doppler shift tracking ability of a receiver - For DL: - Do not consider switching point for ATG UE demodulation evaluation and requirements

Except the agreements above, however, some of issues are listed with several options in this Way Forward for further studies. In this contribution, we discussed first the scope of UE demodulation requirement. Then, we analyzed and gave our proposals for the parameter assumptions of PDSCH requirements if introduced.

# 2	Discussion

## 2.1 Test scope

Based on the agreements companies made in the RAN4 #106 meeting, PDSCH requirements will be introduced. We are fine to keep this previous agreement and consider the propagation condition as: AWGN + Doppler that agreed in the last meeting.

For CSI reporting, the specification has included requirements for CQI reporting under AWGN condition already, and ATG scenario brings no obvious difference to that. Meanwhile, we won’t introduce new codebook or plan to implement advanced codebooks under AWGN condition, which gives no reason for defining new PMI requirements. Same circumstance for RI reporting requirement, only rank 1 has been agreed to consider. In this case, we propose not to define any new CSI reporting requirements, the ATG UE shall meeting legacy UE CSI reporting requirements, including both AWGN and fading channel. There is no need to consider new PDCCH requirement either.

Proposal 1: Do not define new requirement for ATG UE CSI reporting, the ATG UE shall meet legacy UE CSI reporting requirements

Proposal 2: Do not define new PDCCH requirements, the ATG UE shall meet legacy PDCCH requirements

## 2.2 Parameter assumptions for PDSCH requirement

Bandwidth & SCS

It’s agreed to apply 15kHz SCS for FDD and 30kHz SCS for TDD.

Issue 2-2-3: Bandwidth & SCS Agreement: - Apply 15kHz SCS for FDD, 30kHz SCS for TDD. - For FDD 15kHz - 10MHz - For TDD 30kHz, - 40MHz or 100MHz

As for the corresponding bandwidth, we are fine with consider 10MHz bandwidth for FDD 15kHz SCS and 40MHz bandwidth for TDD 30kHz to keep the consistency with legacy PDSCH requirements. In addition, we propose to also consider large bandwidth for defining PDSCH requirements, since it is practical to apply higher bandwidth for the ATG (CPE) UE that always has high SINR and needs to serve a large traffic capacity as it is distributing data within an aircraft.

Considering the ATG operating bands, n3, n34 and n41 were newly included into the WID.  After overviewing all the bands for ATG, including n1, n3, n34, n41, n78 and n79, 20MHz can be supported by all the FDD bands and 100MHz can be supported by all the TDD bands except n34. In this case, we propose to have additional test cases for 20MHz bandwidth for FDD and 100MHz bandwidth for TDD.

Proposal 3: Consider additional test cases for 20MHz bandwidth for FDD and 100MHz bandwidth for TDD

MCS & Rank

For the MCS & rank, following options are listed in the WF:

Issue 2-5: MCS&Rank - Cover rank 1, 16QAM and 64QAM - FFS: Whether to cover 256 QAM Companies are encouraged to bring simulation results in next meeting. - Companies are encouraged to bring simulation results in next meeting.

As for the MCS selection, it was agreed to cover 16QAM and 64QAM. Besides, we think 256QAM should be considered since there is a need for high capacity for ATG service.

As for the link budget, we found it is optimal for applying 256QAM, the SNR at receiver side for 2GHz and 4GHz are shown below:

Figure 2.2-1 Received SNR per RB for 2GHz

Figure 2.2-2 Received SNR per RB for 4GHz

Detailed analysis can be found in our discussion paper in section 2.3[2].

Observation 1: 256QAM can be supported from the link budget point of view

From the performance point of view, we also evaluated two different MCS values of 256QAM to see the SNR point of 70% and 90% of max Tput. Followings are results and plots for cases of FDD configuring 256QAM [3]:

Table 2.2-1 Performance of MCS20

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS index and MCS Table | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-5 | 10 / 15 | 20 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 12.49 |
| 1-6 | 20 / 15 | 20 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 12.41 |

Figure 2.2-3 FDD, 256QAM(MCS20) with 10MHz and 20MHz bandwidth

Table 2.2-2 Performance of MCS27

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS index and MCS Table | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-7 | 10 / 15 | 27 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 19.51 |
| 1-8 | 20 / 15 | 27 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 19.53 |

Figure 2.2-4 FDD, 256QAM(MCS27) with 10MHz and 20MHz bandwidth

From the simulation results above, we can observe the SNR point of 90% max Tput for MCS20 is around 13dB and for MCS27 is around 20dB. Comparing with link budget results, we see that:

- For FDD 20MHz CBW, 256QAM can be used for DL when distance is less than 140km.

- For TDD 100MHz CBW, 256QAM can be used for DL when distance is less than 70km

Observation 2: There are obvious margins in link budget to support 256QAM in ATG scenario

Thus, we propose to consider 256QAM for the modulation order if there will be new PDSCH demodulation requirements.

Proposal 4: Consider 256QAM for defining PDSCH demodulation requirements (if introduced)

Applicability rule

Three options were listed for the discussion of applicability rule:

Issue 2-8: Applicability rule - Option 1: RAN4 to further evaluate applicability rules between ATG and NTN UEs only after the scope of ATG UE demodulation is agreed (QC) - Option 2: Applicability rule can be defined that if UE supporting both NTN and ATG feature has passed the NTN performance requirements, then it can skip ATG cases with the same MCS and rank configurations. (HW) - Option 3: Needed to have the applicability rule. Introducing a new UE feature is preferable so that we can easily define the applicability rule if we are going to define new ATG requirements into the TS38.101-4. (Ericsson) Tentative agreements: - Further discuss the applicability rule after channel model, test scope and test parameters are converged.

If new PDSCH requirements will be introduced, especially within the TS38.101-4, a new UE feature is needed since, unlike other UEs, ATG UEs need to support the pre-compensation. Meanwhile, the applicability of ‘nrofHARQ-Processes-v1700’ should be supported if the new TDD pattern is introduced.

Therefore, we should also define the corresponding applicability rule. Here we provide an example as follows:

5.1.1.12 Applicability of requirements for ATG The performance requirements in Table 5.1.1.12-1 shall apply for UEs which support optional feature supportOfATG. UE capability; Test type; Test list. ATG with 2RX; FR1 FDD; PDSCH; All tests in Clause 5.2.1.1.x. PDCCH; All tests in Clause 5.3.2.1.2. FR1 TDD; PDSCH; All tests in Clause 5.2.1.2.x. PDCCH; All tests in Clause 5.3.2.2.2. ATG with 4RX; FR1 FDD; PDSCH; All tests in Clause 5.2.2.1.x. PDCCH; All tests in Clause 5.3.3.1.2. FR1 TDD; PDSCH; All tests in Clause 5.2.1.2.x. PDCCH; All tests in Clause 5.3.3.2.2.

Proposal 5: New UE feature for ATG scenario shall be introduced

Proposal 6: Applicability rule for ATG UE shall be introduced

# 3	Summary

In summary, we discussed the necessity of introducing UE demodulation and CSI reporting requirement. Based on the analysis above and agreements made in the previous meeting, we proposed the following:

Proposal 1: Do not define new requirement for ATG UE CSI reporting, the ATG UE shall meet legacy UE CSI reporting requirements

Proposal 2: Do not define new PDCCH requirements, the ATG UE shall meet legacy PDCCH requirements

Proposal 3: Consider additional test cases for 20MHz bandwidth for FDD and 100MHz bandwidth for TDD

Observation 1: 256QAM can be supported from the link budget point of view

Observation 2: There are obvious margins in link budget to support 256QAM in ATG scenario

Proposal 4: Consider 256QAM for defining PDSCH demodulation requirements (if introduced)

Proposal 5: New UE feature for ATG scenario shall be introduced

Proposal 6: Applicability rule for ATG UE shall be introduced

# References

[1]	R4-2305899, WF for ATG demodulation requirements, CMCC, RAN4 #106bis-e, Online

[2]	R4-2307688, Discussion on general issue for ATG demodulation, Ericsson, RAN4 #107

[3]	R4-2307835, Simulation results for ATG PDSCH demodulation, Ericsson, RAN4 #107