# 3GPP TSG- RAN WG4 Meeting # 107	R4-2307842

# Incheon, Korea, 22th May–26th May, 2023

# Source: 	ZTE Corporation

# Title: 	Discussion on added FRCs for less than 5MHz NR channels

# Agenda item:	8.15.3

# Document for:	Approval

# 1 Introduction

During RAN#99-e meeting, a revised Work Item [1] has been approved on introduction of NR support for dedicated spectrum less than 5MHz for FR1. In this contribution, we want to share some further views on added FRCs.

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n106, n26, n28 and n85:

  - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum.

  - Minimize impact on RF requirements:

    - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

    - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n106, n26, n28 and n85.

  - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

# 2 Discussion

2.7 FRC: Add FRCs from R4-2304575 as a baseline. Agreement: FRC: Add FRCs from R4-2304575 as a baseline

  - 2.1 FRC for receiver requirements for QPSK

Table 2.1 Fixed reference channel for receiver requirements (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 3 |
| Subcarrier spacing | kHz | 15 |
| Subcarrier spacing configuration μ |  | 0 |
| Allocated resource blocks |  | 15 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 8 |
| MCS Index |  | 4 |
| MCS Table for TBS determination |  | 64QAM |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 984 |
| Transport block CRC | Bits | 16 |
| LDPC base graph |  | 2 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 |
| Binary Channel Bits per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 3240 |
| Max. Throughput averaged over 1 frame | Mbps | 0.787 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |

  - 2.2 FRC for maximum input level for 64QAM

Table 2.2 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 64QAM)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 3 |
| Subcarrier spacing | kHz | 15 |
| Subcarrier spacing configuration μ |  | 0 |
| Allocated resource blocks |  | 15 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 8 |
| MCS Index |  | 24 |
| MCS Table for TBS determination |  | 64QAM |
| Modulation |  | 64 QAM |
| Target Coding Rate |  | 3/4 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 7296 |
| Transport block CRC | Bits | 24 |
| LDPC base graph |  | 1 |
| Number of Code Blocks per Slot |  |  |
| For Slot 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 |
| Binary Channel Bits per Slot |  |  |
| For Slot 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 9720 |
| Max. Throughput averaged over 1 frame | Mbps | 5.837 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |

  - 2.3 FRC for maximum input level for 256 QAM

Table 2.3 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 256QAM)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 3 |
| Subcarrier spacing | kHz | 15 |
| Subcarrier spacing configuration μ |  | 0 |
| Allocated resource blocks |  | 15 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 8 |
| MCS Index |  | 23 |
| MCS Table for TBS determination |  | 256QAM |
| Modulation |  | 256 QAM |
| Target Coding Rate |  | 4/5 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 9992 |
| Transport block CRC | Bits | 24 |
| LDPC base graph |  | 1 |
| Number of Code Blocks per Slot |  |  |
| For Slot 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 |
| Binary Channel Bits per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 12960 |
| Max. Throughput averaged over 1 frame | Mbps | 7.994 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |

  - 2.4 FRC for maximum input level for 1024 QAM

Table 2.4 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 1024QAM)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 3 |
| Subcarrier spacing | kHz | 15 |
| Subcarrier spacing configuration μ |  | 0 |
| Allocated resource blocks |  | 15 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 8 |
| MCS Index |  | 23 |
| MCS Table for TBS determination |  | 1024QAM |
| Modulation |  | 1024 QAM |
| Target Coding Rate |  | 0.78 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 12808 |
| Transport block CRC | Bits | 24 |
| LDPC base graph |  | 1 |
| Number of Code Blocks per Slot |  |  |
| For Slot 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 |
| Binary Channel Bits per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 16200 |
| Max. Throughput averaged over 1 frame | Mbps | 10.246 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |

Proposal 1: It is proposed to define added FRCs as Table 2.1, Table 2.2, Table 2.3 and Table 2.4.

# 3 Conclusion

In this contribution, we want to share some views on added FRCs and the proposal is made as following:

Proposal 1: It is proposed to define added FRCs as Table 2.1, Table 2.2, Table 2.3 and Table 2.4.

# 4 Reference

- RP-230186, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Anterix, T-Mobile USA