3GPP TSG-RAN WG4 # 107	R4-2307857

Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.19.2

Source:	MediaTek Inc.

Title:	Discussion on Application Layer Data Throughput

Document for:	Discussion

# Introduction

In RAN4#106-bis-e, the Way Forward document was approved [1]. Also, in RAN4#106-bis-e simulation results with and without HARQ retransmissions were already collected in [2]. We provide discussion with proposals for the Rel-18 work in this contribution.

# Discussion

# 2.1 Background

In the following Table 1-1 some main parameters of baseline simulation assumptions of the WF document [1] are listed.

Table 1-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation.

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
| Maximum number of HARQ transmission |  |  | 4 | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |  |  |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| The Outer Loop Link Adaptation (OLLA) |  |  | Disabled (Follow CQI) |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

We have provided simulation results in the previous RAN4#106 meeting in separate document [3] that are already included in simulation result collection document [2]. Our simulation assumptions for the previous meeting are still valid and we do not need to update our previous results.

# 2.2 Remaining open issues

In this chapter we discuss remaining open items.

Issue 2-1-1: Test point T (%) selection

- Candidate test pool based on alignment simulation results in R4-2304257

| Test Case | Candidate test point 1: T % (SNR, Gspan) | Candidate test point 2: T % (SNR, Gspan) |
| --- | --- | --- |
| FR1 FDD 2x2 | 10% (3.2 dB, 1.6 dB) 15% (6.5 dB, 2.1 dB) | 40% (18.5 dB, 2.4 dB) |
| FR1 TDD 2x2 | 10% (3.6 dB, 1.9 dB) 15% (7.1 dB, 1.8 dB) | 35% (17.5 dB, 2.4 dB) |
| FR1 FDD 2x4 | 15% (3.3 dB, 1.9 dB) | 50% (15.2 dB, 2.5 dB) |
| FR1 TDD 2x4 | 15% (3.6 dB, 2.2 dB) | 50% (15.4 dB, 2.8 dB) 35% (10.7 dB, 2.3 dB) |
| FR2-1 (TDD 2x2) | 15% (4.4 dB, 2.1 dB) | 40% (15.1 dB, 1.8 dB) |

- Issue A. # of test point(s)

  - Option 1. Two test points for 2Rx and 4 Rx.

  - Option 2. Two test points for 2Rx and 1 test point for 4 Rx.

  - Option 3. Single test point for 2Rx and 4 Rx

- Issue B. T(%) and outlier removal for Gspan <= 2.5 dB

Issue 2-1-2: X dB margin

  - Option 1: Apply X dB margin per modulation order to impairment results

X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM

X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM

Note that the modulation order would be based on median value over companies’ own median statistics on CQI.

  - Option 2: Apply single X dB margin to impairment results (e.g. X=0.5 dB or else)

WF for Issue 2-1-1 and 2-1-2

- Discuss in the next meeting with the following aspect based on the updated results if any.

- SNR options considering uniqueness of test SNR coverage

- T (%) based on alignment results considering Gspan

- Final SNR with X dB margin is within range of [0 20] for FR1 and [0 16] for FR2.

- It does not preclude the possibility of T(%) adjustment with [+- 5% steps] or removal of outlier results from Gspan perspective.

From alignment simulation results in R4-2304257 [2], we have selected aligned 15% and 35% throughput test point for all tests and collected averaged SNRs and corresponding SNR spans in Table 1-1. With this selection we have 2 tests with larger span than target 2.5dB. To move forward we suggest selecting aligned 2 test points as shown in Table 1-1 rather than trying to hand pick smallest spans and resolve span issue by adding reasonable extra margin to requirements. We are also fine to remove outliers to achieve target span. We would prefer to define margin independently of modulation order for simplicity.

Observation #1: We can select aligned 2 test points for all tests with reasonable span.

Observation #2: We can resolve large span issue with extra margin in requirements, and we are also fine to remove outliers.

Proposal #1: For Issue 2-1-1 Test point T (%) selection Issue A, we support Option 3. We can find 2 reasonable test points for each test.

Proposal #2: For Issue 2-1-1 Test point T (%) selection Issue B, we support resolving large span issue with extra margin in requirements, and we are also fine to remove outliers.

Proposal #3: For Issue 2-1-2 X dB margin, we prefer Option 2. It would be more straightforward to use single margin value. We can also compromise to Option 1 if there are strong concerns from other companies.

Table 1-1: Proposed tests

| Test Case | Candidate test point 1: T % (SNR, Gspan) | Candidate test point 2: T % (SNR, Gspan) |
| --- | --- | --- |
| FR1 FDD 2x2 | 15% (6.5 dB, 2.1 dB) | 35% (16.7 dB, 3.7 dB) |
| FR1 TDD 2x2 | 15% (7.1 dB, 1.8 dB) | 35% (17.5 dB, 2.4 dB) |
| FR1 FDD 2x4 | 15% (3.3 dB, 1.9 dB) | 35% (10.6 dB, 2.8 dB) |
| FR1 TDD 2x4 | 15% (3.6 dB, 2.2 dB) | 35% (10.7 dB, 2.3 dB) |
| FR2-1 (TDD 2x2) | 15% (4.4 dB, 2.1 dB) | 35% (13.4 dB, 1.7 dB) |

Issue 2-2: Applicability and release independence

- Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15

- Option 1.a: Proposal considering declaration

- Optional for Rel-15 and Rel-16 UEs based on declaration - Mandatory for all Rel-17 and forward UEs.

- Option 2: The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

In this Rel-18 WI we are using the latest performance to define requirements. Therefore, we do not see reasonable to make these tests mandatory starting from Rel-17 UEs.

Proposal #4: For Issue 2-2 Applicability and release independence we support Option 2. We still think that Rel-18 test should be defined as applicable from Rel-18 because we are using the latest performance to define requirements.

# Conclusion

In this paper we provided the view on the application layer data throughput. The following observations and proposals are made:

Observation #1: We can select aligned 2 test points for all tests with reasonable span.

Observation #2: We can resolve large span issue with extra margin in requirements, and we are also fine to remove outliers.

Proposal #1: For Issue 2-1-1 Test point T (%) selection Issue A, we support Option 3. We can find 2 reasonable test points for each test.

Proposal #2: For Issue 2-1-1 Test point T (%) selection Issue B, we support resolving large span issue with extra margin in requirements, and we are also fine to remove outliers.

Proposal #3: For Issue 2-1-2 X dB margin, we prefer Option 2. It would be more straightforward to use single margin value. We can also compromise to Option 1 if there are strong concerns from other companies.

Proposal #4: For Issue 2-2 Applicability and release independence we support Option 2. We still think that Rel-18 test should be defined as applicable from Rel-18 because we are using the latest performance to define requirements.

# Reference

- R4-2305915, “Way forward on physical layer throughput requirements”, Intel

- R4-2304257, “Summary of simulation results for NR UE absolute physical layer throughput requirements with link adaptation”, Intel

- R4-2301864, “Simulation results for Application Layer Data Throughput”, MediaTek