3GPP TSG-RAN WG4 Meeting # 107	R4-2307859

# Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on UE RF requirements for IoT NTN

Agenda Item:	9.7.4

Source:	MediaTek Inc.

Document for:	Approval

- Introduction

In RAN4#106bis-e meeting, regarding IoT NTN UE, the network signalling remark NS_02N does not clearly specify the value.

Based on our study, if the NS_02N’s value is not specified, RAN5 could assume network signalling NS_02N/2N (i.e., remark/value) would be used. However, the value for NS_02N should be integer value based on RAN2 rule. Therefore, the violation is expected.

In this contribution, the proposal about useful integer value for NS_02N is provided for NTN IoT specification moving forward.

- UE RF requirements open issues

2.1	Integer value for Network signalling remark NS_02N

In TS 36.102 clause 6.2A.3, RAN4 has defined the additional maximum power reduction(A-MPR) requirements, Network signaling value NS_02N has been introduced in TS 36.102, based on which RAN5 derives specific message content, However RAN5 finds the NS_02N doesn’t not apply a valid value for IE additionalSpectrumEmission

Taking Band255 A-MPR Network signaling value NS_02N for category M1 UE as an example,

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | ≤ 3.5 |

RAN5 sets IE additionalSpectrumEmission as 2N (NS_02N) in the systeminformationblockType2 as part of the cell broadcast message, this exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2N |  |  |

But IE AdditionalSpectrumEmission value was defined as Integer in TS 36.331 clause 6.3.4

AdditionalSpectrumEmission information element

-- ASN1START

AdditionalSpectrumEmission:: =		INTEGER (1..32)

AdditionalSpectrumEmission-v10l0:: =	INTEGER (33..288)

-- ASN1STOP

Therefore, to clean the violation about using 2N for remark NS_02N below can be helpful as modified below.

Table 6.2A.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | Change 2N to 2 |  |  |

Proposal 1: Based on RAN2 and RAN5 rules about IE additionalSpectrumEmission, to consider the integer-value 2 for network signalling remark NS_02N for usage.

Proposal 2: Based on proposal 1, to consider agreeing the modification indicated in [1].

- Conclusion

In the contribution, proposals of IoT NTN UE RF requirements for residual topics are provided.

Proposal 1: Based on RAN2 and RAN5 rules about IE additionalSpectrumEmission, to consider the integer-value 2 for network signalling remark NS_02N for usage.

Proposal 2: Based on proposal 1, to consider agreeing the modification indicated in [1].

- References

[1] R4-2307849, “CR to 36.102 for NTN IoT UE RF requirements corrections”, MediaTek Inc.