3GPP TSG-RAN WG4 Meeting #107	R4-2307876

Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	Discussion on mobility requirements for ATG RRM

Agenda item:	8.14.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on mobility requirements for ATG RRM.

Discussion

Location/Distance based Mobility

In RAN4 #106bis-e meeting [1], following agreements were made.

Issue 2-1-1-1: Cell re-selection mechanism - Option 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation. - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority. - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection. - Option 4: - If ATG UE altitude higher than certain threshold, ATG UE perform measurement - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement - Option 5: No need to specify distance, altitude or speed-based enhancements to cell re-selection mechanism during Release 18.

As listed in above, cell re-selection mechanism is discussed in the RAN4 #106bis-e meeting. For option 1, we think there is a technical benefit for ATG UE operation, and since the similar mechanism is already defined in NTN, the distance-based mechanism can be used in ATG.

- Proposal 1. Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

  - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

  - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

For option 4, some companies have concern that the technical benefit of the mechanism. However, we think that the mechanism in option 4 should be introduced to guarantee the measurement performance of ATG UE. As shown in Fig. 1, interference from TN can affect the ATG UE when ATG UE is located certain low altitude. Thus, in RRC_IDLE mode, the measurement performance of ATG UE can affected by interference from TN.

Figure 1. ATG Networks

- Proposal 2. to guarantee the measurement performance of ATG UE, we propose altitude-based measurement triggering condition as

  - If ATG UE altitude higher than certain threshold, ATG UE perform measurement,

  - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement

Conclusion

In this contribution, our proposals are provided as follows:

- Proposal 1. Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

  - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

  - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

- Proposal 2. to guarantee the measurement performance of ATG UE, we propose altitude-based measurement triggering condition as

  - If ATG UE altitude higher than certain threshold, ATG UE perform measurement,

  - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement

Reference

[1] R4-2306344, “WF on NR ATG RRM requirement”, CMCC