3GPP TSG-RAN WG4 Meeting #107	R4-2307879

Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	Discussion on improvement on SCell/SCG setup delay

Agenda item:	8.25.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on improvement on SCell/SCG Setup delay.

Discussion

Clarification on ‘Early measurement’

Issue 2-1-5: clarification on ‘Early measurement’ - Candidate solutions: - FFS: Early measurements can be considered at RRC connection setup regardless of whether they originate from IDLE/INACTIVE mode or from CONNECTED mode before the UE entered IDLE/INACTIVE mode. (Nokia)

We think there is no technical benefit to consider measurement results which originate from connected mode before the UE entered IDLE/INACTIVE mode due to UE mobility and accuracy. So, we prefer to follow the legacy definition of early measurements.

- Proposal 1: for definition of early measurement, the legacy definition can be used as “Early measurements are performed during IDLE/INACTIVE mode”

Overall solution for UE which is configured with EMR measurement

Issue 2-2-1: overall solution for UE which is configured with EMR measurement. - Candidate solutions: - Option 1: - Availability and validity status should be introduced into existing EMR reports to indicate to the network whether EMR measurement results are valid. - Option 1a: - For EMR capable UE, introduce a new indication in existing EMR report to allow UE to indicate network whether and which EMR measurement results are valid upon UE returning connected mode. - Option 1c: - For EMR capable UE, the UE reports the indication of valid EMR results in RRC setup/resume Complete message (Msg5). - Option 2: FFS

We support to introduce a new indication for validity, but, we think the final decision on validity shall be made by the NW, and the UE can provide additional information to help of NW decision through the new indication.

- Proposal 2: Introduce a new indication for validity. But the final decision on validity should be made by the NW, and the UE can provide additional information to help NW decision through the new indication. Additional information could be

  - information about when the measurement result was obtained

  - Information about the serving cell to which the UE belongs when the measurement results were obtained.

Definition of ‘valid’ in solution based on existing measurement

Issue 2-2-3: definition of ‘valid’ in solution based on existing measurement - Agreements: - Candidate criteria for measurements validity definition - A) the measurement are performed within the last [X] seconds before it is reported - B) the reported measurement results satisfy measurement accuracy - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB - FFS whether a single or several criteria should be used for measurements validity definition.

- Proposal 3: For definition of valid, following criteria can be used

  - combination of A and B or

  - single criteria C

where A, B and C can be defined as

    - A) the measurements are performed within the last [X] seconds before it is reported

    - B) the reported measurement results satisfy measurement accuracy

    - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

Number of samples, including whether Rx beam sweeping is needed

Issue 2-3-8: number of samples, including whether Rx beam sweeping is needed - Proposals: - Option 1: (QC) - Enhanced measurement requires at least 8 SSB samples and additional [X] samples of SSB burst. (FFS on X, e.g 2) - Option 2: (Nokia) - Number of samples UE needs to measure can be depending on UE radio conditions and measurement conditions. - During re-evaluation/validation measurements, UE is not expected to perform full beam-sweeping and hence, the scaling factor associated with the beam sweeping can be reduced. - Option 3: (LG) - During the additional measurement, for the further validity check, reduced samples and (or) reduced beam sweeping factors can be considered for the results obtained within the last [X] seconds. Reduced samples or reduced beam sweeping factors may not be considered for results obtained more than [X] seconds ago. Also, for the results obtained within the last [Y] (Y<X) seconds, it can be treated as valid results without additional measurements. - Option 4: (vivo) - Both introducing UE capability for lower Rx beam sweeping factor like Rel-17 positioning and using the previous beam information obtained in the early measurement can be the candidate options. - Option 5: (OPPO) - As baseline, RAN4 shall not reduce the scaling factor of Rx beam sweeping when defining requirements for the new measurement during RRC connection setup/resume. - Option 5a: (HW) - To guarantee the measurement accuracy, the measurement samples are not supposed to be reduced. Not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure.

During the additional measurement, reduced samples and (or) reduced beam sweeping factors can be considered for the further check of measurement results obtained in the last [X] seconds.

- Proposal 4: During the additional measurement, for the further validity check, reduced samples and (or) reduced beam sweeping factors can be considered for the results obtained within the last [Y] seconds. Reduced samples or reduced beam sweeping factors may not be considered for results obtained more than [Y] seconds ago. Also, for the results obtained within the last [X] (Y>X) seconds, it can be treated as valid results without additional measurements.

Conclusion

In this contribution, we provide our views as follows:

- Proposal 1: for definition of early measurement, the legacy definition can be used as “Early measurements are performed during IDLE/INACTIVE mode”

- Proposal 2: Introduce a new indication for validity. But the final decision on validity shall be made by the NW, and the UE can provide additional information to help NW decision through the new indication. Additional information could be

  - information for when the measurement result was obtained

  - Information for the serving cell to which the UE belongs when the measurement result was obtained.

- Proposal 3: For definition of valid, following criteria can be used

  - combination of A and B or

  - single criteria C

where A, B and C can be defined as

    - A) the measurements are performed within the last [X] seconds before it is reported

    - B) the reported measurement results satisfy measurement accuracy

    - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

- Proposal 4: During the additional measurement, for the further validity check, reduced samples and (or) reduced beam sweeping factors can be considered for the results obtained within the last [Y] seconds. Reduced samples or reduced beam sweeping factors may not be considered for results obtained more than [Y] seconds ago. Also, for the results obtained within the last [X] (Y>X) seconds, it can be treated as valid results without additional measurements.

Reference

[1] R4-R4-2306356, WF on NR Mobility Enhancements RRM requirements (part 2), Apple