3GPP TSG-RAN WG4 Meeting # 107		R4-2307884

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			5.2.8.4

Source:	MediaTek Inc.

Title:	Discussion on measurement procedure requirements in NTN

Document for:	Discussion

# Introduction

In this contribution, our views on MGRP for the concurrent gap in inter-frequency measurement over NTN.

# Missing sharing factor for concurrent gap in inter-frequency measurement over NTN

Table 9.3.4-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(8 * Kgap) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured. |  |

One remaining issue on inter frequency measurement is the MGRP, as Issue 1-5 in WF [1].  The legacy Note 3 in 9.3.4 for TN can be a starting point (as listed below). However, it can be ambiguous for NTN because 2 gaps can be associated to the same frequency in NTN. As illustrated in Figure 1, MG#1 (MGRP = 40 ms) and MG#2 (MGRP=160ms) will lead to different delay requirements.

Figure 1: Ambiguity on the delay requirement of NTN concurrent gap.

Observation 1: The association between SMTC and MG is ambiguous, and it will lead to different delay requirements.

As now this WI is during maintenance phase, One simple solution without backward capability issue is to take the MG with the longer MGRP to define the delay requirement.

Proposal 1: Regarding NTN inter-frequency measurement delay requirement, for a UE supporting concurrent gaps, the MRGP is the MRGP of the measurement gap with the longer MGRP associated with the target frequency layer to be measured.

# Summary

Observation 1: The association between SMTC and MG is ambiguous, and it will lead to different delay requirements.

Proposal 1: Regarding NTN inter-frequency measurement delay requirement, for a UE supporting concurrent gaps, the MRGP is the MRGP of the measurement gap with the longer MGRP associated with the target frequency layer to be measured.

# Reference

- R4-2303147, “WF on Rel-17 NR NTN RRM Performance requirements”, Xiaomi, RAN4#106 meeting, Mar. 2023.