3GPP TSG RAN WG4 Meeting #107	R4-2307905

Incheon, Republic of Korea, May 22 – May 26, 2023

Source: 	Ericsson

Title:  	RRM aspects for tunnel deployment scenario

Agenda Item:	8.13.4.4

Document for:	Discussion

# Introduction

WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) [1] is introduced in Rel-18 release. In last meeting, issues on tunnel deployment are discussed and the agreements are captured in WF [2].

In this contribution, we present our viewpoints and proposals on the issues left in the WF.

# Discussion

## Sub-topic #2-1: Tunnel deployment

Issue 2-1-1: Deployment assumptions for Scenario#1

Agreement: - Consider both uni-directional and bi-directional deployments in the tunnel for Sceanrio#1 (single-panel reception UE and DPS transition scheme). Way forward: - FFS additional assumptions on tunnel deployment: - Option 1: In uni-directional scenarios assume the at least one open-space RRH is deployed close to railway, e.g., with tunnel deployment parameters, and orientations of RRH panels are the same in open-space and in the tunnel. - Option 2: No need to taking exit/entrance of the tunnel into account of tunnel scenario and channel.

The time duration of when traveling through exit/entrance may only a few seconds or shorter, even there’re some special propagation conditions but we don’t think set of RRM requirements need to be regulated to deal with their various characteristics in short time period. On other hand, we agree that some special RRH implementations can optimize beam management or mobility performance when UE’s moving across exit/entrance.

Proposal 1: No need to take exit/entrance of the tunnel into account of tunnel scenario and channel.

Issue 2-1-3: Solution to the mobility issue in the tunnel

Way forward: - FFS possible solution to the mobility issue inside the tunnel when CPE is travelling in the direction opposite to the serving beam: - Option 2: Solutions that allow network to trigger early handover/beam switch - Option 2a: Enabling CHO with special settings next to the RRH - Option 3: UE-initiated beam selection/activation based on beam measurement - Option 5: No need to introduce new mechanism for mobility issue when the train is travelling opposite to the serving beam orientation - Option 6: Consider bi-directional deployment with simultaneous multi-panel reception in the tunnel scenario to alleviate the mobility issues.

Besides of criteria based on signal strength , assistance information isn’t reliably utilized by network to determine the proper configurations to trigger TCI state update. Given that, shortening the disruption starting from determining dropping of signal strength to completing new TCI state is a feasible solution.

In the manner, we expect UE initiated TCI state switch can facilitate beam management and mitigate the negative effects of the issue. In UE initiated TCI state switch, the delay of UE signal strength reporting to the network and the delay of ACK for TCI state update command can be skipped, compared to normal TCI state switch.

More detailly, based on beam measurements, UE can select one (i.e., beam selection) or more (i.e., beam activation) TCI states as active and reports it/them to NW. Since the TCI state(s) is selected/activated by UE, UE is responsible to remember the QCL properties of DL RS(s) associated with the selected TCI state(s). Thus, the measurement in reporting is not needed anymore. Considering predictable stable and simple connection in HST FR2 scenario, UE can always assume network follows UE’s selection without the need of further confirmation between network and UE.

Figure 1 UE-initiate TCI state switch

Proposal 2: Support Option 2a enabling CHO.

Proposal 3: Support Option3: UE-initiate beam selection/activation based on beam measurement.

# Conclusion

Proposal 1: No need to take exit/entrance of the tunnel into account of tunnel scenario and channel.

Proposal 2: Support Option 2a enabling CHO.

Proposal 3: Support Option3: UE-initiate beam selection/activation based on beam measurement.

# References

- RP-222272 ‘New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)’, Samsung

- R4-2306398 ‘WF on NR FR2 HST UL Timing Adjustment Solutions and Tunnel Deployment’, Nokia, Nokia Shanghai Bell