3GPP TSG RAN WG4 Meeting #107	R4-2307906

Incheon, Republic of Korea, May 22 – May 26, 2023

Source: 	Ericsson

Title:  	CA requirements for enhanced HST FR2

Agenda Item:	8.13.4.2

Document for:	Discussion

# Introduction

WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) [1] is introduced in Rel-18 release. In last meeting, issues on multi-panel reception and CA relevant are discussed and the agreements are captured in [2].

In this contribution, we present our viewpoints and proposals on the issues relevant to CA  in aforementioned WF.

# Discussion

## Sub-topic #2-2 Definition of HST FR2 CA requirements

(1) PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements Agreements and way forwards: - The enhanced FR2 HST requirements for intra-frequency detection in Rel-17 FR2 HST WI are also applied for SCells inter-frequency cell detection if introduced. - FFS RAN4 need to specify the requirements for inter-frequency PSS/SSS detection and - FFS RAN4 need to specify the requirements for time index detection.

Proposal 1: We prefer specifying the requirements for inter-frequency PSS/SSS detection.

Proposal 2:  We understand time index detection in the issue is for inter-frequency.  We’re open to not define requirements on time index detection, it is likely that time index for inter-frequency aligns with serving PCell.

### SCell activation delay

(6) SCell activation delay Way forward: - Check SCell activation delay and applicable conditions in HST scenario.

SCell  activation delay can be 3ms provided below conditions are fulfilled as per legacy specifications:

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided - the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

It is very likely SCell activation delay in HST FR2 scenario doesn’t need this kind of limitation based on the premise that SCell is co-located with PSCell and share same transceiver.

Proposal 3: SCell activation delay with 3ms Tactivation_time  can be applied without legacy limitation in HST FR2 scenario.

# Conclusion

Proposal 1: We prefer specifying the requirements for inter-frequency PSS/SSS detection.

Proposal 2:  We understand time index detection in the issue is for inter-frequency.  We’re open to not define requirements on time index detection, it is likely that time index for inter-frequency aligns with serving PCell.

Proposal 3: SCell activation delay with 3ms Tactivation_time  can be applied without legacy limitation in HST FR2 scenario.

# References

- RP-222272 ‘New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)’, Samsung

- R4-2306341 ‘WF on intra-band CA for FR2 HST PC6 UE’, Ericsson