3GPP TSG-RAN WG4 Meeting #107	R4- 2307916

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title:	MSD for PC2 FDD band n8

Agenda Item:	7.22.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

The channel BW=25MHz has been added to band n8. It was agreed in the last meeting [1] that:

On n8 25MHz: Cover PC2 for n8 25MHz in FDD PC2 HPUE WI, but this does not prevent/delay completion for other potential completed CBWs.

In this paper, we present our analysis on the PC2 MSD for this new BW.

# 2	Discussion

Following the methodology used in our previous contribution [2], the PC2 MSD for 25MHz BW are estimated as in the table below:

Table 1: REFSENS degradation for PC2 (SCS=15kHz)

| Band | Tx arch | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n8 | 1Tx | 1.2 | 1.2 | 1.4 | 3.3 | 3.4 | - | 3.8 | - | - | - |
|  | 2Tx | 3.1 | 3.2 | 3.8 | 6.2 | 6.5 | - | 7.1 | - | - | - |

# 3	Conclusion

Regarding PC2 MSD for 25MHz BW, we have the following proposal:

Proposal 1: It’s proposed to set PC2 1Tx MSD=3.4dB and 2Tx MSD=6.5dB for 25MHz BW for band n8.

# References

- R4-2306544 WF on FDD HPUE Basket WIs, China Unicom, RAN4#106bis-e

- R4-2300715 MSD for PC2 FDD bands, Huawei, HiSilicon, RAN4#106

- RP-230352 Revised WID: high power UE (power class 2) for NR FR1 FDD single band, China Unicom, RAN#99