3GPP TSG-RAN WG4 Meeting # 107                                    R4-2307928

Incheon, Korea, 22th - 26th May 2023

Agenda item:	8.25.5

Source: 	ZTE Corporation

Title: 	Discussion on Enhanced CHO configurations

Document for:	Approval

Introduction

According to the agenda, RAN4 should continue to discuss the impact of RAN4 on the following objectives in WID [1] at this meeting:

- For CHO including target MCG and target SCG in NR-DC [RAN3]: - to specify data forwarding optimizations; and - to specify, if needed, a solution to avoid unnecessary signaling exchange between source MN and target SN. - To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline - To specify RRM core requirements for the following, as necessary [RAN4]: - L1/L2-based inter-cell mobility - Enhanced CHO configurations addressed by this WI

At the last meeting, an agreement was reached on the scope of RRM requirements for the enhanced CHO configuration [2], considering both scenarios in objectives 3 and 4 are considered in RAN4 requirements and Both FR1+FR2 and FR1+FR1 NR-DC are in scope of RRM requirements for enhanced CHO configurations.

Issue 3-1-1: whether both scenarios in objectives 3 and 4 are considered in RAN4 requirements: - Agreements: - Define requirements for the following scenarios: - Scenario 1: CHO including target MCG and target SCG in NR-DC. (obj. 3) - Scenario 2: CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

Based on this, we discuss the RRM requirements for CHO including target MCG and target SCG in NR-DC (obj. 3) and, CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4) in this contribution.

Discussion

According to the agreements from RAN2, RAN4 can discuss objective 3 and objective 4. For objective 3, RAN2 supports CHO including target MCG and target SCG in Rel-17. However, the corresponding RRM requirement has never been defined in RAN4. Therefore, we reached an agreement in RAN4#106bis that the requirements for CHO including target MCG and target SCG (objective 3) also need to be defined. Meanwhile, for objective 4, RAN2 is discussing the order of CHO evaluation and CPC/CPA evaluation, that is, whether they are concurrent or sequential. Based on our understanding, RAN2 has agreed to support the simultaneous evaluation of CHO and CPC in Rel-18. Next, we will discuss PCell handover delay and PSCell handover delay for objective 3 and 4, respectively

2.1 CHO including target MCG and target SCG in NR-DC (obj. 3)

In the last meeting, the companies held different views on this issue and way forward is duplicated as below [1]:

Issue 3-2-1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) - Agreements: - PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as: - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution. - Definition of each component, except Tprocessing, is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2. - Definition of Tprocessing: - Option 1: same as that defined in requirements of handover with PSCell. - Option 2: same as that defined in requirements of CHO. Issue 3-2-2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) - Candidate solutions: - Companies are encouraged to further discuss the following issues: - When does UE execute PSCell addition/change - Option 1: after PCell CHO is executed but before it is completed. - Option 2: after PCell CHO is completed. - Whether Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell?check

- We have reached an agreement on the definition of PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC and the only ambiguity currently lies in the definition of Tprocessing. Some companies think that UE behavior is exactly same as handover with PSCell once conditions in CHO for PCell is met, so the value of Tprocessing should follow the defined in requirements of handover with PSCell. However, Other companies believe that the value should reuse definition in requirements of CHO because RAN4 has only defined HO with PSCell requirements for FR1+FR2 NR-DC in R17 with no SCG condition is defined. So for CHO with PSCell in FR1+FR2 NR-DC, the delay requirements for PCell DCHOwithPSCell_PCell is the same as CHO.

Proposal 1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution.

  - Definition of each component is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2.

Regarding the delay requirement for CHO with PSCell in NR-DC, we recommend using the existing results of R17 as a baseline. In our understanding, the CHO with PSCell in Rel-17 can be divided into two parts. The first part is to have a conditional handover delay from the Pcell, and the change/addition of the PScell can be executed after executing the PCell CHO but before it is completed. In other words, for the discussion of delay requirements for Rel-17 CHO to include a target SCG, we can refer to existing requirements in TS 38.133 and revise the partial composition.

Proposal 2: UE execute PSCell addition/change after PCell CHO is executed but before it is completed.

In our understanding, Tsearch_PCell_Conditional is not neeed in DCHOwithPSCell_PSCel. In this case, Tsearch_PCell_Conditional = 0 since the condition for target PCell has already been met and PCell time can be considered known to UE.

Proposal 3: Tsearch_PCell_Conditional is not neeed in DCHOwithPSCell_PSCel due to target PCell is known to UE.

Proposal 4: Combination of HO with PSCell and CPC. PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

DCHOwithPSCell_PSCell =  TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms

2.2 CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

- For R18 CHO with CPAC, RAN2 has discussed the order and agreed to support the simultaneous evaluation of CHO and CPC in Rel-18. Therefore, we need to further discuss RRM requirements on this basis.

Issue 3-3-1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) - Candidate solutions: - Option 1: same as that in obj. 3. - Option 2: DCHOwithCPAC = Max [DCHO, DCPAC] - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition - Option 3: same as CHO - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that - Tprocessing = 25 ms - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC. Issue 3-3-2: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) - Candidate solutions: - Option 1: - if conditions for CPC are met before conditions for CHO are met, follow objective 3 requirements. - If conditions for CPC are met after CHO is complete, reuse existing CPC requirements. - Option 2: DCHOwithCPAC = Max [DCHO, DCPAC] - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition - Option 3: - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

- Regardless parallel or sequential evaluation, the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled. Based on the precondition,

- In our understanding, there are three possible scenarios for the combination of CHO and CPAC.

- Case1: CPC condition is met first

- Case2: CHO condition is met first

- Case3: Both CPC condition and CHO condition are met

- According to RAN2 agreement in RAN2#121bis, both CHO and CPC cell change is executed when both CHO and CPC conditions are met. If the CPC condition is met first, the CPC could not be executed and need to wait until the CHO condition is met. Meanwhile, if the CHO condition is met first, the CHO could not be executed and need to wait until the CPC condition is met. In addition, the NW can provide both CHO with candidate SCGs configuration and CHO-only/R17 CHO with SCG configuration for the same candidate PCell, to ensure that the CHO can be triggered if CHO condition is met.

RAN2 agreement in RAN2#120: Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation) RAN2 agreement in RAN2#121: RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18 RAN2 agreement in RAN2#121bis: When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

- According to RAN2's baseline, CHO+CPC only supports case 3. So, the handover delay depends on which timeline is the longest for CHO and CPC and we prefer option 2 that DCHOwithCPAC = Max [DCHO,  DCPAC]. Because both CHO and CPC conditions need be met, TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change.

- Observation1: TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change.

Proposal 5: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is define as DCHOwithCPAC = Max [DCHO,  DCPAC],

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition except TEvent_DU

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition  except TEvent_DU

Proposal 6: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is define as DCHOwithCPAC = Max [DCHO,  DCPAC],

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition except TEvent_DU

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition except TEvent_DU

Conclusions

In this contribution, we put forward the following proposals on Enhanced CHO configurations:

Proposal 1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution.

  - Definition of each component is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2.

Proposal 2: UE execute PSCell addition/change after PCell CHO is executed but before it is completed.

Proposal 3: Tsearch_PCell_Conditional is not neeed in DCHOwithPSCell_PSCel due to target PCell is known to UE.

Proposal 4: Combination of HO with PSCell and CPC. PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

    - DCHOwithPSCell_PSCell =  TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms

- Observation1: TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change.

Proposal 5: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is define as DCHOwithCPAC = Max [DCHO,  DCPAC],

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition except TEvent_DU

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition except TEvent_DU

Proposal 6: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) is define as DCHOwithCPAC = Max [DCHO,  DCPAC],

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition except TEvent_DU

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition except TEvent_DU

References

- RP-223520 Revised WID on Further NR mobility enhancements, MediaTek Inc.

- R4-2306396, WF on NR Mobility Enhancements RRM requirements (part 2), Apple