3GPP TSG-RAN WG4 Meeting #107                                   R4-2307931

Incheon, Korea, May 22 – May 26, 2023

Source: 	MediaTek Inc.

Title: 	Discussion on FR2-1 UL 256QAM

Agenda Item:	8.7.2

Document for:	Discussion

- Introduction

In RAN4 #106bis, a WF for UL 256QAM has been approved [1]. However, some open issues require further discussion in subsequent meetings. In this contribution, we provide more completed parameters and initial simulated results for our proposal as promised in previous discussions. Moreover, we present our views on these unresolved issues as well.

- 2	Discussion

2.1 Phase noise issues

Regarding phase noise issues, there are some open issues are listed in the below.

Issue 1-1-1 Phase noise assumption - FFS. Issue 1-1-2 Phase noise profiles evaluation - Further evaluate different phase noise profiles based on the simulation assumptions in issue 1-1-3 and issue 1-1-4 in next meeting.

For the issue of “the phase noise assumption on EVM budget”, it is noteworthy that phase noise in the RF front end has a significant impact on the EVM performance. The phase noise impairments are frequency dependent and phase noise could increase by 6 dB every time when the operating frequency doubles. Therefore, the phase noise impact in  FR2 is much higher compared with FR1 even when the common phase error (CPE) impacts can be compensated.

There are two UE phase noise model are captured in TR38.803 in section 6.1.10 and section 6.1.11. [2]. However, both models are not well suited for UL 256QAM for FR2-1. Therefore, we can study a new UE phase noise profile based on the pole/zero model in TR38.803 with improved phase noise performance. The phase noise model used here is a generalization of the multi-pole/zero model with extended fractional orders, which is given by the following equation:

Sf=PSD0n=1N1+(ffz,n)αz,nm=1M1+(ffp,m)αp,m                   (1)

Where S is the single sideband phase noise power spectral density, f the offset frequency, fz,1 .. fz,N the zeros, fp,1 .. fp,M the poles, αz,1 .. αz,N the order of the zeros, and αp,1 .. αp,M the orders of the poles.

Our phase noise model parameters for UL256QAM for 29.55GHz, 39GHz and 45GHz are summarized in Table 1, Table 2 and Table 3 respectively, and Figure 1 shows the corresponding phase noise profiles.

Table 1 Proposed phase noise modelling parameters for UL 256QAM at 29.55GHz

| PSD0 | 860.4 (29.35 dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | αz,n | fp,m | αp,m |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 660e3 | 2.7 | 1.6e6 | 3.3 |
| 3 | 182e6 | 2.53 | 30e6 | 1 |

Table 2 Proposed phase noise modelling parameters for UL 256QAM at 39GHz

| PSD0 | 1498.7 (31.76) dB |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | αz,n | fp,m | αp,m |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 620e3 | 2.7 | 1.6e6 | 3.3 |
| 3 | 240e6 | 2.53 | 30e6 | 1 |

Table 3 Proposed phase noise modelling parameters for UL 256QAM at 45GHz

| PSD0 | 1995.3 (33 dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | αz,n | fp,m | αp,m |
| 1 | 3e3 | 2.37 | 1 | 3.3 |
| 2 | 550e3 | 2.7 | 1.6e6 | 3.3 |
| 3 | 280e6 | 2.53 | 30e6 | 1 |

Figure 1.  Proposed phase noise profiles for 29.55GHz, 39GHz and 45GHz.

Several candidates for phase noise profiles base on pole-zero model are summarized in issue 1-1-4 [1], which focus on 29GHz and 39GHz. To evaluate the impact of PTRS processing to EVM of these phase noise profiles, we perform the simulation based on issue1-1-3 [1]. The desired waveform was 256QAM with 120 kHz SCS. To isolate the EVM impact of phase noise, no other types of impairments were added to the waveform. Table 4 and Table 5 show the initial simulated results.

Table 4 Benefit of PTRS in 256QAM, 64RB, 100MHz, SCS= 120kHz, CP-OFDM @29GHz

| CP-OFDM w/ phase noise, 256QAM, 120kHz, 64RB, DMRS based CPE removal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Phase noise model | PTRS configuration | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| TR38.803: UE example 1 | L = 1, K =2 | -28.3 | -27.7 | 0.6 |
| Our proposal | L = 1, K =2 | -32.7 | -31.6 | 1.1 |
| Qualcomm’s proposal | L = 1, K =2 | -32.5 | -30.7 | 1.8 |

Table 5 Benefit of PTRS in 256QAM, 64RB, 100MHz, SCS= 120kHz, CP-OFDM @39GHz

| CP-OFDM w/ phase noise, 256QAM, 120kHz, 64RB, DMRS based CPE removal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Phase noise model | PTRS configuration | EVM (dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| TR38.803: UE example 1 | L = 1, K =2 | -25.7 | -25.3 | 0.4 |
| Our proposal | L = 1, K =2 | -29.6 | -28.7 | 0.9 |

From Table 4, we can find that the EVM performance with our proposed phase noise profile is very close to Qualcomm’s proposal at 29GHz after PTRS processing. Therefore, our proposal and Qualcomm’s proposal for the phase noise profile can be used in 256QAM MPR evaluation for FR2-1.

Observation 1: EVM performance with our proposed phase noise profile is very close to Qualcomm’s proposal at 29GHz after PTRS processing.

Proposal 1: Both Our proposal and Qualcomm’s proposal for the phase noise profiles can be used for 256QAM MPR evaluation.

2.2 Minimum EIRP requirements

Regarding minimum EIRP requirements, the remaining open issue is listed in the below.

- Down select from Option 1 and Option 2 in next meeting. - Option 1: The minimum EIRP for UL 256 QAM for EVM test could be relaxed by 14 dB based on the difference between the SNR of 256QAM (29.1dB) and the SNR of QPSK(15.1dB). Parameter; Unit; Level for PC1; Level for PC2; Level for PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 18; 1; 8. Operating conditions; Normal Conditions. NOTE 1: PTRS is configured for 256 QAM. - Option 2: Use a “-1dB/dB” relation to calculate the minimum EIRP requirement for 256QAM and consider 1dB correction factor. Parameter; Unit; PC1; PC2; PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 19.5; 2.5; 9.5.

The UE minimum EIRP for QPSK, 16QAM and 64QAM feature a -1dB/dB relation which is used to derive the minimum UE EIRP for 256QAM for the EVM budget of 3.5%. The maximum dynamic range is the difference between the minimum peak EIRP and the minimum EIRP requirements for EVM test. In the last meeting, the MPR limitation issue was discussed because of the limited maximum dynamic range. Therefore, considering the maximum dynamic range issue, we prefer option 1.

Observation 2: The maximum dynamic range is the difference between the minimum peak EIRP and the minimum EIRP requirements for EVM test. In the last meeting, the MPR limitation issue was discussed because of the limited maximum dynamic range. Therefore, considering the maximum dynamic range issue, we prefer option 1.

Proposal 2: RAN4 to consider option 1 as the minimum EIRP requirements for EVM test.

- 3	Conclusion

In this contribution, we have provided our views for the open issues on the WF [1] and provide more completed parametersh for our phase noise profiles proposal.

Observation 1: EVM performance with our proposed phase noise profile is very close to Qualcomm’s proposal at 29GHz after PTRS processing.

Proposal 1: Both Our proposal and Qualcomm’s proposal for the phase noise profiles can be used for 256QAM MPR evaluation.

Observation 2: The maximum dynamic range is the difference between the minimum peak EIRP and the minimum EIRP requirements for EVM test. In the last meeting, the MPR limitation issue was discussed because of the limited maximum dynamic range. Therefore, considering the maximum dynamic range issue, we prefer option 1.

Proposal 2: RAN4 to consider option 1 as the minimum EIRP requirements for EVM test.

- 4	Reference

- R4-2306603, “WF on UL 256QAM”, Xiaomi.

- TR 38.803