3GPP TSG-RAN WG4 Meeting #107	R4-2307938

# Incheon, KR, May 22 – May 26, 2023

Title:	On applicability decision tree for FR2 MIMO OTA

Source:	Samsung

Agenda item:	8.17.2

Document for:	Discussion

1.	Introduction

According to the time plan of FR2 MIMO OTA campaign, lab alignment campaign can start after RAN4#106-bis-e immediately. Now that all the 4 PADs are NSA UE but the EN-DC band combination decision tree has not been determined yet. It is urgent to finalize the decision tree for smooth proceeding of subsequent campaign.

The agreement of last meeting is as following [1], B66 was agreed as example LTE anchor band for n261 and decision tree is expected to be further discussed:

Sub-topic 2-4 EN-DC band combination selection for FR2 MIMO OTA Issues 2-4-1 and 2-4-2: Example LTE anchor band selection and Decision tree <Agreement>: - To make sure a consistent test condition for a particular UE across different labs, an example LTE anchor band along with a decision tree shall be defined. - LTE B66 is selected as the example LTE anchor band for n261 FR2 MIMO OTA test. For UEs that don’t support B66, use a decision tree to select the LTE anchor band. - Further discuss details of the decision tree at next meetings. - Note: UEs that support n261 with any LTE anchor are allowed for the FR2 band n261 MIMO OTA Measurement Campaign.

In this contribution, we further discuss the decision tree and propose to reuse the same decision tree as that for FR1 TRP TRS.

2. 	Discussion

About the decision tree of EN-DC band combination selection, the options discussed last meeting are as following [2]:

Issue 2-4-2: Decision tree to select the EN-DC band combination for FR2 MIMO OTA testing - Options - Option 1 (Samsung): Reuse the same criteria of FR1 TRP TRS in terms of uplink perspective, i.e., select the LTE band whose uplink is closest in frequency to the uplink of LTE band used in the example band combination corresponding to the selected NR carrier and which is supported by the UE in an EN-DC configuration with the chosen NR band. - Option 1a: Reuse the same decision tree as that for FR1 TRP TRS (additional clarification needed about uplink or downlink as indicated above). - Option 2 (Xiaomi): Not to reuse the mechanism of FR1 TRP TRS for FR2 MIMO OTA EN-DC band combination selection. - Option 2a: The lowest LTE frequency band which is supported for FR2 EN-DC can be selected. - Others

During discussion in last meeting, companies have the consensus that NR FR2 MIMO OTA performance are not sensitive to which LTE carrier to be used theoretically. With that, it seems not necessary to define a separate decision tree mechanism for MIMO OTA. It would be no harm to reuse the decision tree for FR1 TRP TRS for simplicity.

Proposal 1:	Reuse the same decision tree of FR1 TRP TRS for FR2 MIMO OTA to select EN-DC band combination.

All the PADs will be available for lab alignment test after this meeting, the supported NR FR2 bands are collected already but the supported LTE bands are not yet [1]:

| Provider | How many PADs can be provided | NSA (preferred) /SA | Current location | When will the PAD(s) be ready | Supported bands | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Huawei | One commercial device for lab alignment activity but not for simulation validation activity | NSA | Beijing, China | Ready now | n257 | Not to disclose any UE information except supported bands |
| Keysight | Two commercial devices | EN-DC | USA | Can be hand carried to May RAN4 meeting | PAD_1: n257, n258, n260, n261 PAD_2: n260, n261 | Not to disclose the manufacturers and models |
| Samsung | One commercial device | EN-DC | Korea | Can be hand carried to May RAN4 meeting | n260, n261 | Not to disclose the manufacturers and models |

We think it is also important to collect the supported LTE bands corresponding to the NR FR2 tested band (e.g. n261), and the tested EN-DC band combination should be recorded in test results.

Proposal 2:	Collect PAD’s information on the supported LTE bands corresponding to the NR FR2 tested band in lab alignment campaign, and the tested EN-DC band combination should be recorded in test results.

The decision tree is not only related with EN-DC band combination selection, but also related with SA&NSA applicability rule. The applicability rule adopted for FR1 TRP TRS is as following:

4.3 Applicability rules for testing of FR1 SA and NSA UEs The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following: - For each NR band supported by the device, test the UE in EN-DC mode using any one example configuration containing that NR band or configuration declaration decision tree as per recommended TRP/TRS test procedures in this specification. The applicability and test coverage rules for Standalone (SA) and NSA (EN-DC) capable devices shall include the following: - For each NR band in a device, test the UE in Standalone Mode as per the TRP/TRS test procedures in this specification. - This shall also fulfil coverage for all EN-DC FR1 minimum performance requirements for that NR band and need not be retested in EN-DC mode.

Although in the market majority FR2 UEs are NSA mode currently, the decision tree would be better to be future proof to also accommodate FR2 SA UEs. This issue is not urgent, so we are open to further discuss how to handle the applicability rule for both SA and NSA capable FR2 UEs.

Proposal 3:	RAN4 further discuss how to handle the applicability rule for both SA and NSA capable FR2 UEs.

3. 	Conclusion

Proposal 1:	Reuse the same decision tree of FR1 TRP TRS for FR2 MIMO OTA to select EN-DC band combination.

Proposal 2:	Collect PAD’s information on the supported LTE bands corresponding to the NR FR2 tested band in lab alignment campaign, and the tested EN-DC band combination should be recorded in test results.

Proposal 3:	RAN4 further discuss how to handle the applicability rule for both SA and NSA capable FR2 UEs.

4. 	References

- R4-2305910    “WF for Rel-18 MIMO OTA”, CAICT

- R4-2305999    “Email discussion summary for [106bis-e][330] NR_MIMO_OTA_enh”, CAICT

- TS 38.161