3GPP TSG-RAN4 Meeting #107 		R4-2307945

Incheon, KR, 22th May – 26th May, 2023

Title:	Discussion on L1 part enhancement for FR2 SCell activation

Source:	Xiaomi

Agenda Item:	8.9.2.2

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, RAN4 has some discussion on how to reduce L1 measurement delay for FR2 SCell activation, and approved the related WF in [1]. In this contribution, we further discuss L1 part enhancement for FR2 SCell activation and provide our proposals.

- Discussion

Enhancement for L1-RSRP

Issue 2-1-1: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation (for the case when no valid L3 measurement result is reported after SCell activation command)? FFS: - Option 1 (Apple, Nokia, CMCC, LGE, CTC, NTT DCM, OPPO, Xiaomi, ZTE, OPPO, Ericsson): - if L3 and L1 measurement are using same RS or QCLed type D RSs, skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting. - Option 1a (Apple, LGE, CTC): - if L3 measurement is performed without L3 part enhancement - Option 2 (Intel): - if L1-RSRP report can be re-used for L3 measurement results, two sets of requirement and applicability needs to be defined. - Option 3 (QC): - Defining union delay requirements for cell detection and L1 measurement instead of discussing whether skipping L1 measurement. E.g ) X*TSSB where X = total beam sweeping factor for both cell detection and L1 measurement, FFS : X value. - Option 4 (MTK): - L1-RSRP measurement of FR2 unknown SCell activation can be skipped if NW can trigger the UE to report the latest RSRP measurements from L3 part given that no enhancement is applied in L3 part. - Option 5 (vivo, HW): - If zero is one of the possible entry of UE capability for X2 om issue 1-2-1, the discussion of issue 2-1-2 of R4-2303228 can be merged into 1-2-1 and 2-1-1 of R4-2303228. - Option 5a (HW): - Not to discuss the explicit UE behavior on “skipping L1-RSRP” or whether UE shall derive L1-RSRP based on L3 measurement, which can already be reflected by capability indication of X2 if X2 = 0 is introduced. - Option 6 (ZTE): - When the following conditions are satisfied, L1-RSRP measurement can be ignored: - UE can identify Tx beam during L3 part – L3 part includes all same RS or QCLed Type D RS used by L1 part. - UE can identify Rx beam during L3 part –UE has swept all Rx beam needed by L1 part during L3 part - Option 7 (Intel): - if side condition of SNR>=-2dB can be satisfied, rough beam based L3 measurement can be performed during L3 sync steps, which can satisfy the measurement accuracy requirement. - The L3 measurement result obtained during L3 sync steps will be sent to NW by L1 report. L1 report can be triggered with Scell activation command or separate command. - If new L1 reporting method for L3 measurement result can be defined and there is available L3 measurement result, UE can skip both L1-RSRP measurement and L1-RSRP reporting in Scell activation procedure

If the UE has not reported the valid measurement results with SSB index after receiving SCell activation command, whether the L1-RSRP measurement can be skipped or not is FFS. In our understanding, if the UE has performed the cell search procedure and reported the L3 measurement result with SSB index, and the same RS which is QCL-ed type D used for L1-RSRP measurement, the L1-RSRP measurement and reported procedures can be skipped.

Proposal 1: L1-RSRP measurement can be skipped provided that the UE has reported the valid L3 measurement results with SSB index during cell search procedure and the same RS which is QCL-ed type D used for L1-RSRP measurement.

Issue 2-1-2: if L1-RSRP measurement can be skipped in issue 2-1-1, whether to skip L1-RSRP reporting or TCI indication of FR2 unknown SCell activation (for the case when no valid L3 measurement result is reported after SCell activation command)? FFS: - Option 1 (Nokia, ZTE, Apple): - L1-RSRP reporting cannot be skipped. It is needed to inform network the beam information of the SCell. - The UE shall indicate the L1-RSRP reporting is derived by skipping L1-RSRP measurement. - Revised Option 2 (Ericsson): - UE skips L1-RSRP report if it receives TCI state based on L3 results reported after cell search.

If the UE has reported the L3 measurement result with SSB index, and the same RS which is QCL-ed used for L1-RSRP measurement, the NW can configure TCI state based on the L3 measurement report, and no need to report L1-RSRP measurement used for TCI state activation.

Proposal 2: If L1-RSRP measurement is skipped, the configuration of TCI state activation is based on L3 measurement report and the L1-RSRP reporting can be skipped.

- Conclusion

In this contribution, we discuss the potential enhancement on L1 part for FR2 SCell activation delay reduction and provide our proposals as follows.

Proposal 1: L1-RSRP measurement can be skipped provided that the UE has reported the valid L3 measurement results with SSB index during cell search procedure and the same RS which is QCL-ed type D used for L1-RSRP measurement.

Proposal 2: If L1-RSRP measurement is skipped, the configuration of TCI state activation is based on L3 measurement report and the L1-RSRP reporting can be skipped.

- Reference

[1]	R4-2306315	WF on R18 RRM enhancement- FR2 SCell activation delay reduction,	Apple