3GPP TSG-RAN WG4 Meeting # 107                                   R4-2308020

# Incheon, May 22 – May 26, 2023

Agenda item:	8.35.3

Source:	China Telecom

Title:	RF aspects on SSB-less SCell operation for FR1 inter-band CA

Document for:	Discussion

- Introduction

In RAN4 #106-bis-e meeting, SSB-less SCell operation for inter-band CA for FR1 and co-located cells was discussed and the WF [1] was approved.

SSB-less SCell operation for FR1 inter-band CA can reduce network energy consumption as well as SCell activation delay, we present our views on RF aspects on SSB-less SCell operation for FR1 inter-band CA in this paper.

- Discussion

Status in the WF [1]:

Issue 2-1-1: Priorities for feasibility study - Agreement： - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell. Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation - Agreement: - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS.

As agreed in the WF [1], RAN4 prioritize the discussion on SSB-less SCell operation for inter-band CA for FR1 and co-located cell. Regarding the RF requirements for inter-band CA for SSB-less Scell operation, there is no impact to the legacy requirements of inter-band CA TAE defined in TS38.104, and whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS. We think the RTD requirements can be discussed in RRM session for some of CA combinations, and the definition of specific band combination can be discussed in RF session.

Proposal 1: RTD requirements can be discussed in RRM session for some of CA combinations, and the definition of specific band combination can be discussed in RF session.

Status in the Summary [2]:

Issue 2-2-1: TAE/RTD - Proposals - Option 1: TAE less than 1/2CP - Observation 4: The TAE between two carriers should be less than 1/2CP.（R4-2304212, CMCC） - Proposal 2: Less TAE is required to ensure time synchronization.（R4-2304212,CMCC） - Option 2: TAE less than 260ns - Observation 1: SSB-less operation for intra-band contiguous CA which TAE shall not exceed 260 ns, is supported by current specification.（R4-2304221,HW） - Observation 2: For inter-band CA in FR1 implemented by a shared transmitter, the TAE can be within 260 ns the same as that of intra-band contiguous CA.（R4-2304221, HW） - Observation 3: For inter-band CA in FR1 implemented by co-located but independent transmitters, at least for some combinations it is possible to have a delay deviation within 260 ns.（R4-2304221,HW） - Proposal 1: In SCell without SSB for inter-band FR1 unknown SCell activation scenario, for some inter-band CA combinations, e.g., CA_n5-n8, CA_n1-n3, the RTD between target SCell and inter-band active serving cell is able to be within 260ns under practical BS implementation.（R4-2305025,China Telecom） - Option 3: TAE optimized to at least 65ns - Proposal 1: to confirm that the achievable FR1 inter-band CA TAE requirement in co-located scenario could be optimized down to at least 65ns;（R4-2305529,ZTE） - Option 4: others Issue 2-2-3: AGC - Proposals - Option 1: In SCell without SSB for inter-band FR1 unknown SCell activation scenario, for some inter-band CA combinations, e.g., CA_n5-n8, CA_n1-n3, the reception power difference between target SCell and inter-band active serving cell is able to be within 6dB under practical scenarios.（R4-2305025,China Telecom） - Option 2: others Issue 2-2-5: Baseline for feasibility study - Proposal: As the current specification supports SSB-less operation for intra-band contiguous CA, it can serve as a baseline for the feasibility studies of FR1 co-located inter-band CA with such capabilities.（Huawei,R4-2304221）

In inter-band CA SSB-less SCell activation scenario, from our perspective, for some low frequency inter-band CA band combinations, e.g., CA_n5-n8, CA_n1-n3, the frequencies of the two bands in the band combination are close, thus the RTD between target SCell and inter-band active serving cell is able to be within 260ns under practical BS implementation such as common RRU implementation for the two bands. Since the inter-band CA TAE is defined as 260ns in LTE, it’s feasible for RTD of NR low frequency band combination with small frequency separation to achieve 260ns under practical base station implementation.

Proposal 2: In inter-band CA SSB-less SCell activation scenario, for some low frequency inter-band CA combinations, e.g., CA_n5-n8, CA_n1-n3, the RTD between target SCell and inter-band active serving cell is able to be within 260ns under practical BS implementation.

For above-mentioned low frequency band combinations, under some practical BS implementation such as common RRU implementation for the two bands, the PSD of transmitter power can be same or similar, the path loss and other loss are similar, furthermore, the power imbalance for AGC is defined as 6dB to guarantee UE implementation. Therefore, the reception power difference between target SCell and inter-band active serving cell should be limited within 6dB.

Proposal 3: The reception power difference between target SCell and inter-band active serving cell should be limited within 6dB.

In our views, conditions of intra-band case can be the baseline for feasibility studies of FR1 co-located inter-band CA scenario.

Proposal 4: Conditions of intra-band case can be the baseline for feasibility studies of FR1 co-located inter-band CA scenario

- Conclusion

The contribution presents our views on the RF aspects of Network energy savings for NR, with the following proposals:

Proposal 1: RTD requirements can be discussed in RRM session for some of CA combinations, and the definition of specific band combination can be discussed in RF session.

Proposal 2: In inter-band CA SSB-less SCell activation scenario, for some low frequency inter-band CA combinations, e.g., CA_n5-n8, CA_n1-n3, the RTD between target SCell and inter-band active serving cell is able to be within 260ns under practical BS implementation.

Proposal 3: The reception power difference between target SCell and inter-band active serving cell should be limited within 6dB.

Proposal 4: Conditions of intra-band case can be the baseline for feasibility studies of FR1 co-located inter-band CA scenario

- References

- R4-2306639, WF on RF feasibility study of Network Energy Savings for NR, Huawei, RAN WG4 Meeting # 106-bis-e, April 2023.

- R4-2306310, Topic summary for [106bis-e] [148] Netw_Energy_NR, Huawei, RAN WG4 Meeting # 106-bis-e, April 2023.