3GPP TSG-RAN WG4 Meeting # 107                                   R4-2308021

# Incheon, May 22 – May 26, 2023

Agenda item:	8.35.4

Source:	China Telecom

Title:	RRM requirements on SSB-less SCell operation for FR1 inter-band CA

Document for:	Discussion

- Introduction

In RAN4 #106-bis-e meeting, SSB-less SCell operation for inter-band CA for FR1 and co-located cells was discussed and the WF [1] was approved.

SSB-less SCell operation for FR1 inter-band CA can reduce network energy consumption as well as SCell activation delay, we present our views about the potential feasibility and solutions on SSB-less SCell operation for FR1 inter-band CA in this paper.

- Discussion

General

Status in the WF [1]:

< Agreement >Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA - Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis < Agreement >Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

As agreed in the WF [1], Scenario 1 is no SSB but with TRS transmission configured on the SSB-less SCell, whether to consider requirements based on TRS transmission during and/or after SCell activation is FFS. In intra-band case, SCell activation delay is 3 ms thus TRS isn’t used for fine timing tracking during SCell activation, however, TRS is used for QCL configuration during SCell activation and used for timing tracking after SCell activation. In inter-band case especially for the band combination with small frequency domain separation, the requirement and conditions for intra-band case can be the baseline for inter-band case. Therefore, at least for band combinations with small frequency domain separation, there is no need to consider requirements based on TRS transmission during SCell activation.

Proposal 1: For band combinations with small frequency domain separation, there is no need to consider requirements based on TRS transmission during SCell activation.

Scenario 2 has no SSB and no TRS transmission configured on the SSB-less SCell, which seems to be more beneficial for network energy saving. Scenario 2/2a can be discussed simultaneously with Scenario 1.

Proposal 2: Scenario 2 seems to be more beneficial for network energy saving. Scenario 2/2a can be discussed simultaneously with Scenario 1.

Regarding conditions for SSB-less inter-band CA SCell operation for Scenario 1, we think received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell is 260ns, and the difference of the reception power between the SSB-less SCell and the FR1 inter-band active serving cell is 6dB. As for TCI state, we think it can depend on NW decision, and it’s not necessary to define TCI state as a condition. The frequency domain separation or specific band combination can be defined in RF session, and we can define the corresponding RRM conditions and requirements with these RF definitions. We can use the wording like small/large frequency domain separation for conditions discussion since RF session hasn’t completed these definitions so far.

Proposal 3: For SSB-less inter-band CA SCell operation for Scenario 1, between the SSB-less SCell and the FR1 inter-band active serving cell, RTD is 260ns and reception power difference is 6dB.

Proposal 4: It’s not necessary to define TCI state as a condition.

Proposal 5: The frequency domain separation or specific band combination can be defined in RF session.

Regarding conditions for SSB-less inter-band CA SCell operation for Scenario 2/2a, we think RTD and reception power difference are 260ns/6dB respectively, and other conditions can be further discussed.

Proposal 6: For SSB-less inter-band CA SCell operation for Scenario 2/2a, RTD and reception power difference are 260ns/6dB respectively, and other conditions can be further discussed.

Feasibility of SSB-less operation

Status in the WF [1]:

Issue 2-2-1: Whether/how to carry out performance evaluation - Candidate Options: - Option 1 (Nokia): RAN4 shall investigate the behavior of BLER against SNR and/or throughput against SNR as the metrics to evaluate the performance of physical downlink channels. Furthermore, RAN4 may consider how the RTD would impact the BLER or throughput performances of those channels. - RAN4 to start the feasibility study by evaluating the performance of PDSCH, using metric proposed in Proposal 1. - RAN4 to investigate feasible frequency separation between the PCell and SCell, and under which conditions reuse of the time/frequency synchronization could be achieved. - RAN4 to investigate the impact of channels models, starting with TDL models, on the SSB-less SCell operation for inter-band carrier aggregation. - RAN4 to discuss about how to model time and frequency errors that SCell may experience in SSB-less SCell operation. - The outcome of the feasibility study could be identifying the frequency separation between the PCell and SCell (i.e., feasible band combinations) and any other conditions such that the demodulation performances of physical downlink channels transmitted by the SSB-less SCell are within a tolerable performance degradation. - RAN4 to agree on the radio parameter settings for PCell and SCell to be used in the performance evaluation for SSB-less SCell operation. RAN4 may suggest using the parameters used in TR 38.864 as much as possible for these evaluations.

In our understanding, at least for band combination with small frequency domain separation, the conditions on RTD and reception power difference for SSB-less inter-band CA SCell activation are same as conditions for SSB-less intra-band CA SCell activation, thus it’s no need to carry out the performance evaluation for this case. As for band combination with large frequency domain separation, it can be further discussed whether to carry out performance evaluation.

Proposal 7: For band combination with small frequency domain separation, no need to carry out the performance evaluation.

RRM requirements for SSB-less SCell activation

Status in the WF [1]:

< Agreement >Issue 2-3-1: Requirements for SSB-less SCell activation SSB-less based SCell activation delay is to be specified for inter-band CA for FR1 and co-located cells based on feasibility study. Issue 2-3-2: Requirements for FR1 SSB-less SCell activation, if feasibility is confirmed - Candidate Options: - Option 1(Huawei, CTC, ZTE): For FR1 co-located SSB-less inter-band CA, when the frequency and time synchronization information of active serving cell on FR1 inter-band is able to be applied for to-be-activated SCell, and the power difference is not larger than [TBD]dB, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms). - Option 2 (Vivo): If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is known - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms. Otherwise, the SSB-less SCell is unknown - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

As agreed in the WF [1], SSB-less based SCell activation delay is to be specified for inter-band CA for FR1 and co-located cells based on feasibility study. If feasibility is confirmed, requirements for FR1 SSB-less SCell activation delay can be reduced to 3ms at least for certain band combinations.

Proposal 8: For FR1 co-located SSB-less inter-band CA, when conditions on RTD and reception power difference are 260ns/6dB respectively, SCell activation delay can be reduced to 3ms.

- Conclusion

The contribution presents our views on the solutions of Network energy savings for NR, with the following proposals:

Proposal 1: For band combinations with small frequency domain separation, there is no need to consider requirements based on TRS transmission during SCell activation.

Proposal 2: Scenario 2 seems to be more beneficial for network energy saving. Scenario 2/2a can be discussed simultaneously with Scenario 1.

Proposal 3: For SSB-less inter-band CA SCell operation for Scenario 1, between the SSB-less SCell and the FR1 inter-band active serving cell, RTD is 260ns and reception power difference is 6dB.

Proposal 4: It’s not necessary to define TCI state as a condition.

Proposal 5: The frequency domain separation or specific band combination can be defined in RF session.

Proposal 6: For SSB-less inter-band CA SCell operation for Scenario 2/2a, RTD and reception power difference are 260ns/6dB respectively, and other conditions can be further discussed.

Proposal 7: For band combination with small frequency domain separation, no need to carry out the performance evaluation.

Proposal 8: For FR1 co-located SSB-less inter-band CA, when conditions on RTD and reception power difference are 260ns/6dB respectively, SCell activation delay can be reduced to 3ms.

- References

- R4-2306369, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #106bis-e, April 2023.