3GPP TSG-RAN WG4 Meeting # 107	R4-2308028

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.5.3.1.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on 8Rx UE demodulation performance requirements

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed Rel-18 8Rx UE demodulation requirement and RAN4 agreed a WF [1]. This contribution presents our views on Rel-18 8Rx UE demodulation requirement.

2. Discussion

Based on the discussion so far, RAN4 made the following agreements on the 8Rx UE demodulation for FDD and 8Rx RF for FDD as given below [2-4]. Based on the agreements on the UE demodulation side, RAN4 agreed to suspend the 8Rx UE demodulation discussion for FDD until the completion of the 8Rx RF discussion for FDD. At the last RAN4 meeting, the RF side almost completed the discussion on FDD requirement. Therefore, we consider that the UE demodulation side can initiate 8Rx UE demodulation discussion for FDD from RAN4#107. Moreover, since the UE demodulation side has already made the tentative agreements of FDD test parameters including SCS/Bandwidth and the number of HARQ process, we propose to agree officially the test parameter based on the tentative agreements.

[UE demodulation: RAN4#105] [2] Duplex model - Focus on TDD as first priority - FFS on FDD pending on the progress in RF core requirements. SCS/Bandwidth - FDD: 15kHz/10MHz (If agreed) - TDD: 30kHz/40MHz Number of HARQ process - FDD:4 - TDD:8

[UE demodulation: RAN4#106] [3] Duplex mode - Focus on TDD and then define requirements for FDD when RF session has conclusion for FDD

[RF: RAN4#106 bis] [4] ΔRIB for FDD band n7 <Way forward/Agreement>: - [4.5] dB for n7

Proposal 1: Initiate the discussion on 8Rx UE demodulation requirements for FDD from RAN4#107.

Proposal 2: Define 8Rx UE demodulation requirements for FDD with the following parameters.

- SCS/Bandwidth: 15kHz/10MHz

- Number of HARQ process: 4

3. Conclusion

In this contribution, we present our views on 8Rx UE demodulation requirement. Our proposals are summarized as below.

Proposal 1: Initiate the discussion on 8Rx UE demodulation requirements for FDD from RAN4#107.

Proposal 2: Define 8Rx UE demodulation requirements for FDD with the following parameters.

- SCS/Bandwidth: 15kHz/10MHz

- Number of HARQ process: 4

References

- R4-2305888, WF on 8Rx demodulation and CSI requirements, Huawei, HiSilicon

- R4-2302942, WF for 8Rx UE performance requirements, Huawei, HiSilicon

- R4-2220613, WF on 8RX UE demodulation and CSI requirements, Huawei, HiSilicon

- R4-2306597, WF on 8Rx UE RF requirements, NTT DOCOMO, INC.