3GPP TSG-RAN WG4 Meeting #107 	R4-2308031

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Interoperability and Testability Aspects of AI/ML for NR Air Interface

Agenda item:	8.22.3

Document for:	Discussion

Introduction

RAN4#106-bis-e is the first RAN4 meeting to open the discussion of RAN4 requirements, testability, and interoperability aspects in relation to the feasibility study of AI/ML for NR air interface (FS_NR_AIML_Air). Following the WID [1], we would like to highlight the following related objectives of the SI that should be addressed in RAN4:

- The study should also identify areas where AI/ML could improve the performance of air-interface functions. - Evaluations to exercise the attainable gains of AI/ML based techniques for the use cases under consideration will be carried out with the corresponding identification of KPIs with the goal to have a better understanding of the attainable gains and associated complexity requirements. - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

Correspondingly, the main RAN4-related goals of the SI can be summarized as follows:

- Study requirements and testing framework for AI/ML enabled air-interface functions

- Identify requirements, KPIs and approaches to ensure that the performance of UEs and gNBs with the new AI/ML-enabled functions meet or exceed existing minimal requirements (including lifetime performance)

- Consider and minimize potential implications of the application of AI/ML-enabled functions, such as complexity, processing capabilities, Life Cycle Management (LCM) overheads, etc.

In the paper, we consider some of RAN4 related agreements from the previous RAN4 #106bis-e meeting and related discussed aspects from the other WGs, especially from RAN1 [1], and further share our view on the following topics:

- Testing framework

- Core requirements for AI/ML

- LCM Related requirements and functional testing

- Requirements on interoperability for ML-enabled Feature/ Functionality

Background

From RAN4#106bis outcome [5] we highlight the following core and interoperability and testing related agreements:

Agreements: 2.1.2 RAN4 Scope and baseline performance - General aspects - RAN4 will study how to define requirements and tests for inference - RAN4 does not need to study requirements/tests for training - If other WG defines the training procedure, RAN4 may need study to define the requirements for it. - RAN4 could evaluate feasibility of requirements/tests for LCM - Progress of the discussion will depend on RAN1/2 progress on these procedures - FFS if requirements for data collection (in particular for training) could/need be defined - Defining AI/ML requirements - For the cases with the existing legacy performance - Take the legacy performance as baseline for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - FFS how to define “legacy performance” (whether on meeting/exceeding existing RAN4 requirements, or a wider criterion taking into account generalization) - New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - For the cases without the existing legacy performance - New or enhanced performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - Generalization verification aspects - Study the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML - One sided and 2-sided models - RAN4 to consider both models, discussion can continue in parallel 2.2.2 Core requirements for AI/ML - Definition of RAN4 core requirements for the following procedures will be studied based on progress in RAN1/RAN2: - Performance monitoring procedure, including performance evaluation and decision making procedure for AI/ML functionalities/models - Functionality/Model management procedure, including functionality/model selection/activation/deactivation, and functionality/model switching/fallback/transfer/delivery/update - Latency/interruption requirement for above procedures - FFS is any other aspects should be studied 2.2.4 LCM Related requirements - Following LCM related requirements are to be studied: - Model/Functionality select/switch/activate/deactivate/fallback - Model/Functionality monitoring - FFS if requirements for data collection (in particular for training) could/need be defined - FFS if requirements for transfer/delivery/update - NOTE: RAN4 study should be aligned with the agreements in other working groups. - Further study under LCM related tests, if they are defined. - how the framework can address the possibility of updates/activation/deactivation/switching to the functionalities/models after the deployment of the devices in the filed

We reiterate also the most relevant agreements from the RAN1 #112bis meeting [4]:

Agreement: • For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: ○ Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. ○ Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. § FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities § FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level § FFS: Other aspects that may constitute Functionality □ FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. • For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: ○ model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capabilityof an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. ○ FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. ○ FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). ○ FFS: relationship between functionality-based LCM and model-ID-based LCM • Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

The above agreements can be depicted as in Figure 1.

Figure 1: ML-enabled Feature: Proposed relationship between ML-enabled Feature/FG, ML Functionalities enabled by configurations based on applicable conditions (conditions indicated by UE capability), and logical ML Models (set of physical ML Models) associated with specific applicable conditions and additional conditions.

Discussion

Testing framework

We consider the agreements and discussions at RAN1#112bis [4] and RAN4#106bis [5] as starting point for the RAN4 study. We believe that it is beneficial to exchange views between the companies on what could be a general approach to the formulation of requirements and testing of AI/ML-enabled Features and Functionalities in the early design phase of the various use cases.

Based on the WF (R4-2306299) agreement, logical block diagrams in R4-230505 can be used as reference.

Based on this setup, the AI/ML model within DUT is directly controlled by TE for testing. The model’s input, “Test Data” is provided by TE and the model’s output will be captured by the TE for testing, together with certain control signaling for other AI/ML Functionality based configurations for e.g., AI/ML model configuration. In order to make this test possible, UE (or gNB) of DUT will provide these testing interface:

- AI/ML model input: as vectors such as eigenvectors for CSI channel compression

- AI/ML model output: as CSI channel feedback for CSI channel compression

- AI/ML model control signaling

- Test setup interface

It seems that most of these interfaces are not related to physical layer channels other than the CSI feedback. These model interface shall be specified to enable the test, if this type of test framework is defined. It is an open question on how to specify these interfaces to cover various use cases.

RAN4 shall further study the possibility of testing setup for individual testing of AI/ML model. Alternatively, model interoperability shall be tested together with specific use case, such as CSI performance test for CSI compression.

In our view, the goal of RAN4 minimum requirements should not focus on the verification of ML models parameters, such as input features, artificial/deep/convolutional neural network (ANN/DNN/CNN) architecture, output inference accuracy, hyperparameters, etc. Majority of ML models implemented in the UE or gNB products will be vendor specific. It cannot be expected that proprietary implementations of ML models will be disclosed or exposed by UEs.

Proposed reference diagrams should be based on existing 3GPP interface and consider new RAN2 functional diagrams. Defining requirements and testing of the ML model or ML algorithm/architecture implementation (input features, inference output, hyperparameters, etc.) is hardly possible in RAN4.

RAN4 requirements, testing methodology and diagrams should be defined for AI/ML-enabled Functionalities and Features (supported by the logical AI/ML Models) and not for isolated AI/ML models.

Core requirements for AI/ML

Functionality/Model management procedure

The core of the Functionality-based LCM is the management procedure. In our view, this procedure include 4 main functions (discussed in more details in Section 3.3), which need to be addressed in RAN4 requirements:

- Registration: registration in the LCM management module of a given ML-enabled Feature/Functionality based on conditions indicated by the UE capability. This procedure could re-use most the existing UE capabilities reporting framework (ongoing discussions in RAN1 [4])

- Configuration: configuration of a registered ML-enabled Feature/Functionality using the conditions (configuration parameters) indicated by the UE capability.

- Monitoring: monitoring signaling procedure(s) for configured ML-enabled Feature/Functionality using the conditions (monitoring parameters) indicated by the UE capability.

- Selection, (de)activation, switching: signaling procedure(s) to allow the LCM management module operational control of configured ML-enabled Feature/Functionality.

RAN4 to study requirements and testing approaches for Functionality management procedures including the operations needed for (re)configuration, monitoring and selection/(de)activation/ switching.

The potential operations required for delivery/update of (logical) ML models supporting ML-enabled Features/Functionalities for certain use cases, can be addressed at later stage in RAN4 when more progress has been reached in RAN2 discussions.

Performance monitoring procedure

Currently, we are not expecting the UE modem firmware to be updated in the field. Therefore, after the UE has passed conformance testing and type approval certification, it is expected that its minimal performance in the network is guaranteed. This may not always be the case anymore with ML model updates at the UE side, that can have a considerable impact on the performance of the ML-enabled Feature.

To ensure consistent system performance, the network should have at least the possibility to monitor the UE side ML Functionality. For relevant (sub) use cases, the additional possibility to switch/activate/de-activate the ML Functionality or supporting logical ML Model used at the UE side can be considered. When this is available, the combination of functional/system and intermediary/proxy performance KPIs as observed by the network should allow network control as part of the Functionality based LCM [1].

Any potential ML model updates at the UE side can have a considerable impact on the performance of the ML-enabled Feature and Functionality supported by the ML Model. New monitoring and control mechanisms designed in RAN1 shall be used to protect the network from considerable negative impact on system performance.

The UE support for the new monitoring and control mechanisms after UE side ML Functionalities and/or ML Model updates/switches should be verifiable after the UE is deployed in the field (operational network).

RAN4 to study requirements and testing procedures for monitoring and control mechanisms that ensure consistent and predictable performance of ML-enabled features during the in-field use of the UEs.

Latency/interruption requirements

As part of the core requirements, it is also of profound importance to be able to verify by the network if/when the UE side ML Functionality is activated with the right trigger (signal, message) and within the allowed delay budget, for various conditions/scenarios/configurations (applicable conditions, in RAN1 terminology). The right trigger can be based on a optional UE internal performance check (when optional performance monitoring for the Functionality based LCM is also at the UE) and always on a request/command from the network (performance monitoring of the Functionality based LCM is always at the gNB). Some of the new RAN4 requirements designed as ML core requirements, should not limit implementation flexibility at the UE side, while always keeping the network capability to monitor and trigger changes on the UE side Functionality.

Knowledge of delay budgets for life cycle management operations for AI/ML enabled features/functionalities is essential for the reliable network operation, including monitoring and detection of UE side Functionalities behaviour and taking corrective actions.

From RAN4 perspective, one of the key objectives is to be able to check that UE ML Functionality (or logical Model for certain use cases) is enabled and it is ready to perform as expected, and to avoid unnecessary performance degradation or activation/deactivation cycle i.e., the allowed delay budgets for the activation and executing ML Functionalities actions must be considered. This is particularly important for the two-sided model solutions (CSI compression) [3]. Similarly, it is also important to ensure that Functionality can be disabled or switched by the UE upon a request/trigger received from the network as soon as it starts degrading the system performance. Therefore, we think that new requirements may be necessary for these performance checks and correct behavior of the UE Functionalities and supported ML Models.

RAN4 to consider the design of new core requirements for allowed delay budgets for activation/deactivation/switching of Functionalities, to ensure the network can rely on the outcome of Functionality based LCM actions and corresponding UE behavior.

UE Processing capability

Processing capability of devices plays a crucial role in determining their ability to enable and perform ML tasks. To ensure the device’s (UE’s) ability to perform (near) real-time inference and other on-device decision-making processes, its processing capabilities should be identified and indicated. Specifically, when the UE is expected to handle several ML-enabled features simultaneously, a signalling framework should be provisioned for corroborating the device’s ability to steadily maintain the expected ML performance.

Also, application of AI/ML methods can be viewed as an addition to the legacy approaches on the NR air interface. Therefore, RAN4 testing framework should be extended to include similar requirements, which focus on the UE’s computation and processing time capabilities. An example of such specification can be found in RAN1 3GPP TS 38.214 Release 17, in which details on, for instance, the CSI processing criteria and UE CSI computation time are introduced.

RAN4 to discuss the ways of identification and potential impacts of limited device processing and storage capabilities on performance of AI/ML-enabled functionalities.

LCM related requirements and functional tests

The use of Functionality-based LCM is part of the current RAN1 and RAN2 agreements [4].  However, in the ongoing RAN1 and RAN2 discussions there is no agreement so far on a ML Functional Framework for air-interface purposes.

In the ongoing RAN1 and RAN2 discussions there is no agreement so far on a ML Functional Framework for air-interface purposes, which RAN4 can use to design requirements or functional tests.

In our view, the main functions and interfaces needed for the Functionality-based LCM procedures can be captured in a diagram such as depicted in Figure 2. This diagram only shows the Functionality in ‘inference’ operating mode i.e., all functions related to training and/or delivering a specific (physical/logical) ML Model are not included. Furthermore. the focus of these discussion is on the use cases with UE-side models and UE-part of two-sided models.

For the use cases with UE-side models and UE-part of two-sided models, RAN4 to consider the design of new Functionality-based LCM related requirements and functional tests starting with the requirements needed for a Functionality operating in (ML) inference mode.

Figure 2: An overview of main Functionality- based LCM functions, applicable for UE-side models and UE-part of two-sided use cases, operating in inference mode.

With reference to Figure 2, for UE-side models and UE-part of two-sided models use cases, we make the following remarks:

- The Functionality UE-side includes two types of functions:

  - ML inference: functions required to execute the Functionality UE-side in inference operating mode.

  - Data collection: functions required to handle the data collection and pre-processing of inference data and collection and post-processing of monitoring data

- The Functionality management is located at the NW/gNB side and implements the management functions required by the NW to register, configure, select/activate/switch and monitor the Functionalities executed at the UE side [4].

- The Functionality gNB-side includes at least monitoring data collection i.e., function(s) required to handle the collection and post-processing of monitoring data received from the Functionality UE-side

- The ML inference and Data collection functions located in the same node, NW/gNB or UE node, communicate with each other using implementation specific solutions i.e., there is no need for RAN4 inter-operability testing of these.

- The communication interfaces between the Functionality management (gNB-side) and the Functionality UE-side requires 3GPP specification to ensure inter-operability and testing/validation.

The Functionality inference and Functionality data collection when located in the same node NW/gNB or UE node, can communicate with each other using implementation specific solutions i.e., do not require any RAN4 inter-operability testing.

Model/Functionality monitoring

Functionality monitoring procedure includes:

- Configuration of the monitoring at the UE-side and gNB-side, performed by the Functionality Management (gNB-side) using the conditions (and monitoring parameters) indicated by the UE capability.

- Collection of the monitoring data, from the data collection functions

For UE-side models and UE-part of two-sided use cases, RAN4 needs to address the requirements and tests for the communication interface used for monitoring data transfer between the data collection function at the UE-side and the data collection function at the gNB-side (Figure 2).

Model/Functionality select/switch/activate/fallback

Based on the Functionality monitoring function, the Functionality Management can take decisions regarding the Functionality switch/activate/fallback at the UE-side. These decisions and recommended actions are needed in order to allow the network to mitigate any undesired (system/link) performance deviations, while the UE is using the an activated ML-enabled Functionality.

For UE-side models and UE-part of two-sided use cases, RAN4 needs to address the requirements and tests for the communication interface used for Functionality Management actions (activation/switching/fallback) towards the UE-side (Figure 2).

LCM functional tests

Based on the discussions and observations in previous sections, we believe the LCM functional requirements must include requirements on the transmission/exchange of at least monitoring data and management actions. The exact signaling procedure to be used for this information exchange is still being discussed in RAN2. In the meantime, RAN4 can start study in parallel the higher-level requirements for these e.g., in terms of the delay budget and reliability.

RAN4 to consider the design of new LCM related requirements and functional tests starting with the transmission/exchange of Functionality monitoring data and Functionality management actions based on newly defined UE-gNB interfaces in RAN2.

Requirements on interoperability for ML-enabled Feature/ Functionality

Strongly related to the Functionality-based LCM, important aspect which needs to be considered in both RAN1 and RAN4 studies, is the interoperability testing. First, we believe, it is important to clarify whether interoperability tests are needed in both single-sided (UE or gNB) and dual sided (UE and gNB) ML model being used to support a certain Functionality. In our view, the interoperability aspect for two-sided models poses the largest challenge.

In the current scope of the AI/ML SI, the interoperability discussion is applicable to two-sided ML solutions, specifically to CSI compression use-case.

Several approaches are currently under discussion in RAN1, how such two-sided models can be trained e.g., UE-first, NW-first. Training of UE-NW models for every couple of UE-NW vendors will be a large overhead. However, due the identified potential impact from the use of a Functionality (supported by one or more ML Model(s), either in the UE or in gNB), on the performance monitoring and corrective actions signalling, we recommend RAN4 to study use case specific interoperability also for one sided ML model solutions.

An additional complication is that corresponding testing setups for the validation/verification of the ML-enabled Feature functionalities has to ensure that the Functionality under test will comply with minimum requirements also with different (vendor or even version-specific) Functionality/Model deployed in real gNB implementations.

Testing setups should ensure TE and DUT vendor-neutral way of testing for newly introduced ML-enabled Features and Functionalities.

RAN4 to consider interoperability testing aspects, for UE ML-enabled Features and Functionalities for both one-sided and two-sided ML Models in respect to performance monitoring and validation.

Due to the introduction of two-sided models, it is necessary to define some requirements for interoperability of the models at the DUT (Device Under Test) and the TE (Test Equipment). Especially for pre-deployment validation of the Functionalities before they are deployed in the field. E.g., in the case of CSI feedback use case, the test setup should make sure that the ML models selected at the DUT and the TE are interoperable and work seamlessly as an initial step in the test procedure. This can be covered as part of use case specific LCM requirements.

Pre-deployment validation of Functionalities and/or ML models across entities for interoperability aspects is a very important step in the test procedure.

RAN4 to study the possible ways of defining LCM procedures for DUT/TE to enable pre-deployment validation of Functionalities and/or ML model interoperability. This can be covered in RAN4 as a part of the pre-deployment validation of the UE-side Functionalities.

Finally, a potentially much more frequent ML-enabled features/functionality updates due to the updates in the underlying ML models may be expected in the live networks in the future. It could be practically impossible to perform testing of all those changes in specially designed testing environments before the deployment to the device. Moreover, it can be hard to ensure that functionalities and their updates can be used for all potentially possible combinations of device and network configurations. This could raise some additional inter-operability issues when the functionalities are updated independently by different network and UE vendors.

Therefore, one approach could be to introduce certain validation capabilities directly in the live network and in the already deployed devices so that the updated ML Functionalities, or ML Models, can be used only after they have passed a basic validation triggered and controlled by the NW. In this case, RAN4 requirements should ensure the presence of such mechanisms in the devices and/or network.

Due to potentially much more frequent updates of ML-enabled Features/functionality, it cannot be not always expected that the updates have been validate and test in specially designed testing environment and with all possible device and network configurations.

RAN4 to consider presence of ML-enabled Features/Functionality validation capabilities in the live network, triggered and controlled by the network, in addition to the traditional pre-deployment testing approaches.

Conclusion

In the paper, we consider some of RAN4 related agreements from the previous RAN4 #106bis-e meeting and related discussed aspects in other WGs, especially in RAN1 [1], and further share our view on the following topics: testing framework, core requirements for AI/ML, LCM Related requirements and functional testing, requirements on interoperability for ML-enabled Feature/ Functionality.

The following Observations and Proposals were made:

On RAN4 Testing framework:

Observation 1: Proposed reference diagrams should be based on existing 3GPP interface and consider new RAN2 functional diagrams. Defining requirements and testing of the ML model or ML algorithm/architecture implementation (input features, inference output, hyperparameters, etc.) is hardly possible in RAN4.

Proposal 1: RAN4 requirements, testing methodology and diagrams should be defined for AI/ML-enabled Functionalities and Features (supported by the logical AI/ML Models) and not for isolated AI/ML models.

On core requirements for AI/ML:

Proposal 2: RAN4 to study requirements and testing approaches for Functionality management procedures including the operations needed for (re)configuration, monitoring and selection/(de)activation/ switching.

Observation 2: Any potential ML model updates at the UE side can have a considerable impact on the performance of the ML-enabled Feature and Functionality supported by the ML Model. New monitoring and control mechanisms designed in RAN1 shall be used to protect the network from considerable negative impact on system performance.

Proposal 3: RAN4 to study requirements and testing procedures for monitoring and control mechanisms that ensure consistent and predictable performance of ML-enabled features during the in-field use of the UEs.

Observation 3: Knowledge of delay budgets for life cycle management operations for AI/ML enabled features/functionalities is essential for the reliable network operation, including monitoring and detection of UE side Functionalities behaviour and taking corrective actions.

Proposal 4: RAN4 to consider the design of new core requirements for allowed delay budgets for activation/deactivation/switching of Functionalities, to ensure the network can rely on the outcome of Functionality based LCM actions and corresponding UE behavior.

Proposal 5: RAN4 to discuss the ways of identification and potential impacts of limited device processing and storage capabilities on performance of AI/ML-enabled functionalities.

On LCM related requirements and functional tests:

Observation 4: In the ongoing RAN1 and RAN2 discussions there is no agreement so far on a ML Functional Framework for air-interface purposes, which RAN4 can use to design requirements or functional tests.

Proposal 6: For the use cases with UE-side models and UE-part of two-sided models, RAN4 to consider the design of new Functionality-based LCM related requirements and functional tests starting with the requirements needed for a Functionality operating in (ML) inference mode.

Observation 5: The Functionality inference and Functionality data collection when located in the same node NW/gNB or UE node, can communicate with each other using implementation specific solutions i.e., do not require any RAN4 inter-operability testing.

Observation 6: For UE-side models and UE-part of two-sided use cases, RAN4 needs to address the requirements and tests for the communication interface used for monitoring data transfer between the data collection function at the UE-side and the data collection function at the gNB-side (Figure 2).

Observation 7: For UE-side models and UE-part of two-sided use cases, RAN4 needs to address the requirements and tests for the communication interface used for Functionality Management actions (activation/switching/fallback) towards the UE-side (Figure 2).

Proposal 7: RAN4 to consider the design of new LCM related requirements and functional tests starting with the transmission/exchange of Functionality monitoring data and Functionality management actions based on newly defined UE-gNB interfaces in RAN2.

Requirements on interoperability for ML-enabled Feature/ Functionality:

Observation 8: In the current scope of the AI/ML SI, the interoperability discussion is applicable to two-sided ML solutions, specifically to CSI compression use-case.

Observation 9: Testing setups should ensure TE and DUT vendor-neutral way of testing for newly introduced ML-enabled Features and Functionalities.

Proposal 8: RAN4 to consider interoperability testing aspects, for UE ML-enabled Features and Functionalities for both one-sided and two-sided ML Models in respect to performance monitoring and validation.

Observation 10: Pre-deployment validation of Functionalities and/or ML models across entities for interoperability aspects is a very important step in the test procedure.

Proposal 9: RAN4 to study the possible ways of defining LCM procedures for DUT/TE to enable pre-deployment validation of Functionalities and/or ML model interoperability. This can be covered in RAN4 as a part of the pre-deployment validation of the UE-side Functionalities.

Observation 11: Due to potentially much more frequent updates of ML-enabled Features/functionality, it cannot be not always expected that the updates have been validate and test in specially designed testing environment and with all possible device and network configurations.

Proposal 10: RAN4 to consider presence of ML-enabled Features/Functionality validation capabilities in the live network, triggered and controlled by the network, in addition to the traditional pre-deployment testing approaches.

- References

- RP-221348, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Jun. 6 - 9, 2022.

- R2-2300680, “Consideration of procedure and signaling of CSI compression”, TSG RAN WG2 #121, Athens, Greece, February 27th – March 3rd, 2023.

- R4-2304126, On Use Cases for AI/ML in NR Air Interface, Nokia, Nokia Shanghai Bell, RAN4#106bis-e, April 17 – 26, 2023.

- R1-2304052, Summary#2 of General Aspects of AI/ML Framework, Moderator (Qualcomm), TSG-RAN WG1 Meeting #112-bis-e, Online, April 17th – April 26th, 2023.

- R4-2306299, “WF on AI/ML RAN4 studies”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #106bis-e, Online, April 17th – April 26th, 2023.

- R4-2304270, “On Interoperability and Testability Aspects of AI/ML for NR Air Interface”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting #106bis-e, Online, April 17th – April 26th, 2023.

- R2-2300680, “Consideration of procedure and signaling of CSI compression”, TSG RAN WG2 #121, Athens, Greece, February 27th – March 3rd, 2023.R4-2304126, “On Use Cases for AI/ML in NR Air Interface”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting #106bis-e, Online, April 17th – April 26th, 2023.