3GPP TSG-RAN WG4 Meeting #107 	R4-2308036

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Open-Space Deployment and Channel Modelling in HST FR2 Enhanced

Agenda item:	8.13.5.1

Document for:	Discussion

Introduction

At RAN4#106bis-e meeting a first discussion of demodulation requirements for HST FR2 High-power UEs that support simultaneous multi-panel reception took place. The outcomes are summarized in the WF [1].

It was agreed to focus on Bi-directional deployment scenario for PDSCH demodulation requirements with the timing different between TRPs is exceeding CP. Both Scenario-A and Scenario-B deployments needs to be analysed. At the same time, deployments and channel modelling for open space deployments still requires further discussion, as follows:

- Whether requirements should be defined based on Scenario-A, Scenario-B or both

- Serving beam coverage areas in different scenarios

- Which channel modelling approach should be selected

We share our view on the issues listed above in this paper. The discussion of tunnel deployment can be found in our accompanying paper [2].

Discussion

On open-space deployments

At RAN4#106bis-e the following WF and FFS were captured regarding Scenario-A and Scenario-B [1]:

Issue 1-1-1: Deployment Scenario for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario Agreement: RAN4 consider the Bi-directional deployment scenario for PDSCH demodulation requirements with simultaneous multi-Rx reception - Consider the scenario the timing different between TRPs is exceeding CP - RAN4 will evaluate the performance impact for this scenario with PC6 UE and specify corresponding requirements with receiver implementation agonistic approach. - RAN4 will discuss the test applicable rules if needed. - Consider both scenario A and scenario B as starting point for performance evaluation - FFS whether single requirement or separate requirements need to be introduced for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirements Issue 1-1-2: Deployment Scenario parameters for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario Way forward: - RRH locations parameters - Reuse the current RRH location parameters’ values of FR2 HST-DPS performance requirements evaluation for DL demodulation performance study of FR2 HST simultaneous multi-panel reception as a starting point for evaluation

On the one hand, current requirements and channel model in TS 38.101-4 are defined for both scenarios, i.e., for Scenario-A and Scenario-B. On the other hand, in Rel-17, Scenario-A was used for uni-directional deployment channel profile (Clause B.3.4.1), but Scenario-B was used for bidirectional deployment channel profile. As it is shown in deployment Figure 1, and following Rel-17 deployment assumptions, as agreed at RAN4#106bis-e, the deployments are rather different:

- In Scenario-B, even though multiple deployments are possible, the requirements are based on the deployment where the beams are pointed to the projection of the next closest RRH on the railways track.

- In Scenario-A, the RRHs are much closer to the track. Therefore, the beams are oriented almost parallel to the railway track.

Figure 1: HST FR2 bidirectional deployments in Scenario-B (top) and in Scenario-A (bottom).

Bidirectional Scenario-A and Scenario-B deployments are both possible when simultaneous two-panel reception is supported but have different beam orientation and coverage. It might be hard to conclude about the pefromance difference in those scenarios before channel model is clarified.

Therefore, both scenarios seem equally relevant for the evaluation of PDSCH requirements.

Continue the discussion of the open-space channel model both for Scenario-A and Scenario-B.

The next issue we would like to address is the beam serving coverage in Scenario-A and Scenario-B:

Issue 1-1-2: Deployment Scenario parameters for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario Way forward: - ... - Beam serving coverage - Reuse the beam serving range of uni-directional FR2 HST-DPS performance evaluation as the left-beam serving range of an RRH in bi-directional deployment, and right-beam serving range is symmetrical to the left-beam serving range about the RRH as a starting point for evaluation - Further study of coverage areas of the beams is needed.

As we already pointed out above, the Scenario-A and Scenario-B have different beam configuration and coverage. In Scenario-B, the beam coverage spans from the middle in between the RRH sites to the middle in between the following two RRH sites. In Scenario-A, the beam coverage is from one RRH site till the following RRH site.

Figure 2 and Figure 3 demonstrate RSRP traces in bidirectional Scenario-B and Scenario-A, correspondingly. Signal strength considers Tx power, RRH and UE antenna and beam gains, and pathloss.

For Scenario-B, it can be seen that beam switching on the same side/panel of the CPE (e.g., from RRH5 to RRH7 or from RRH6 to RRH8), on average, when there is not slow-fading, happens around the middle between the RRHs. Hence, there is no need to introduce Ds_offset.

Figure 2: RSRP traces per each RRH in bi-directional Scenario-B deployment, next closest RRH is used as serving, without slow-fading.

For Scenario-A, the beam coverage for directions is also symmetrical. However, in this scenario, switching from one RRH to the other can happen either before RRH site (for RRHs oriented in the opposite direction to the train travelling direction) or after RRH site (for RRHs oriented in the same direction with the train travelling direction). Therefore, the absolute value of Ds_offset can be the same as in Release 17 i.e., equal to 10m but the sign will depend on the CPE panel orientation and train moving direction.

Figure 3: RSRP traces per each RRH in bi-directional Scenario-A deployment without slow fading.

Consider symmetrical beam coverage areas both in Scenario-A and Scenario-B.

In Scenario-A, the absolute value of Ds_offset before and after RRH can be kept equal to 10m as in Rel-17.

As it can be seen from Figure 2 and Figure 3, the RSRP is visibly stronger in Scenario-A in comparison to Scenario-B. This can be explained by two reasons. Firstly, in Scenario-A, the RRHs are much closer to the railway track. The other reason is that this deployment is more efficient because the CPE is served by the closest RRH, whereas in Rel-17 bidirectional Scenario-B, the UE is served by the next closest RRH.

In Rel-18, when simultaneous multi-panel reception is considered, bidirectional deployment can be considered as a combination of two unidirectional deployments (per panel). Therefore, bidirectional Scenario-B where CPE is served by the closest RRH also becomes more feasible. Such a deployment is shown in Figure 4.

Figure 4: Bidirectional scenario-B where the closest RRH is used as serving.

Bidirectional Scenario-B deployment where the next closes RRH is used as serving does not provide the best possible RRH strength. This scenario is more feasible in Rel-18 when simultaneous multi-pen reception is considered.

Consider bidirectional Scenario-B deployment where CPE is served by the closes RRH (i.e., not by the next closest) as one of the references for propagation conditions for PDSCH requirements.

On propagation conditions for simultaneous two-panel reception

Two main approaches to channel modelling were discussed at RAN4#106bis-e:

Issue 1-1-5: Channel Model for FR2 HST simultaneous multi-panel reception performance evaluation Way forward: - Assuming CPE processing with 2x2 per TRP per panel separately - Option 1：Reuse the existing FR2 HST DPS channel model to perform FR2 HST simultaneous multi-panel reception performance evaluation as a starting point for evaluation - FFS on the modelling the propagation delay as a part of channel model - FFS on how to extend the doppler shift model to simultaneous multi-panel reception - Option 2: Reuse HST-SFN scheme A to study the UE receive signals from two RRHs simultaneously.

There are several main differences in between the DPS channel model described in B.3.4 and HST FR1 SFN channel models introduced in B.3.5 and B3.6:

- DPS Channel model (B.3.4):

  - One Doppler trajectory is modelled

  - Models for unidirectional and bidirectional deployments are introduced

    - Following our analysis of deployments in the previous section, bidirectional Doppler shift trajectory is not relevant to scenario with simultaneous multi-panel reception because per-panel reception can be considered as unidirectional, i.e., there is no need to switch between the panels under the RRH as in single-panel reception scenario.

    - Unidirectional Doppler profile can be used as a reference. However, the model is introduced only for Opposite direction, i.e., only for the panel moving towards the serving beams. Hence, a similar model should be introduced for the other panel, travelling in the Same direction with the serving beam orientation.

- HST-SFN Scheme A (B.3.5)

  - The model includes three trajectories, i.e., relative power and absolute delay trajectories in addition to the Doppler shift trajectory.

  - The model is designed in such a way that TRS k and TRS k+1 creates two different taps of the same channel.

    - This is not relevant to HST FR2 because it is expected that transmissions from two different RRH will carry the same signals, i.e., it is not SFN transmission.

  - Scheme A seems to assume omnidirectional coverage around the RRH, i.e., the Doppler sign is changing after the UE passes the RRH (Figure B.3.5-4). However, this assumption is not relevant to HST FR2 deployments.

  - Relative power relative trajectory does not take any beam gain into account what it not realistic for the HT FR2 deployments, either.

- HST-SFN Scheme B (B.3.6)

  - The main difference to the Scheme A is that beam is changing under the RRH (Figure B.3.6-4).

Based on the analysis above we can conclude that non of the schemes considered before cannot be directly re-used for the HST FR2 deployments with simultaneous two-panel reception.

Neither DPS channel profile introduced for HST FR2 (Clause B.3.4) not SFN channel profiles introduced for HST FR1 (Clause B4.5/B.3.6) cannot be re-used directly for HST FR2 deployment with simultaneous two-panel reception.

It does not mean, though, that some parts of the model cannot be reused, but, firstly, it is necessary to agree on the profile components to be modelled. In our view, since the channel characteristic on the right and the left panel of the CPE are different, i.e., not only Doppler shift but also Rx power and also delay, then all of these components can be considered in the channel profile.

Consider relative power trajectory and absolute delay trajectory as a part of HST FR2 channel profile for simultaneous two-panel reception in addition to the Doppler shift trajectory.

Next, we can consider each of the components of the channel profile model one by one.

Doppler shift profile is already defined for HST FR2 scenarios in DPS channel model, Clause B.3.4. Therefore, unidirectional Deployment Channel Profile from B.3.4.1 can be used as reference. However, this profile is defined only for the Opposite direction, i.e., for the panel moving in the direction opposite to the serving beam. Since simultaneous two-panel reception is considered in Rel-18, another Doppler shift profile must be defined for the panel moving in the Same direction. Then, CPE will receive two signals at a time with two different Doppler profiles like it is shown in Figure 5.

Figure 5: Doppler shift profile per panel for simultaneous two-panel reception in Scenario-A.

For Doppler shift trajectories, re-used existing DPS model from Clause B.3.4.1 for the panel moving in the direction opposite to the serving beam orientation. Additionally, introduce Doppler profile for the panel moving in same direction with the serving beam orientation.

The relative power trajectories were used in the past for some of HST models in FR1. Since simultaneous two-panel reception assumes different powers on different side of the UE, i.e., at different panels, it makes sense to feline power trajectories per panel. However, current SFN HT FR1 relative power profile is not re-usable directly in HST FR2 deployments because omni-directional reception and transmission is not realistic in HST FR2 context.

Relative power trajectories considered in HST FR1 SFN models cannot be re-used directly because they assume omni-directional coverage and do not take beamforming into account both on UE and RRH sides.

Design power trajectories for HST FR2 simultaneous reception propagation conditions taking beamforming pattern on the UE and RRH sides.

Conclusion

In this paper we discuss deployment assumptions and channel profile modelling in open-space HST FR2 deployments with simultaneous multi panel reception. The channel profile should be used for the definition of corresponding PDSCH UE demodulation performance requirements.

In the paper, the following Observations and Proposals were made:

Observation 1: Bidirectional Scenario-A and Scenario-B deployments are both possible when simultaneous two-panel reception is supported but have different beam orientation and coverage. It might be hard to conclude about the pefromance difference in those scenarios before channel model is clarified.

Proposal 1: Continue the discussion of the open-space channel model both for Scenario-A and Scenario-B.

Proposal 2: Consider symmetrical beam coverage areas both in Scenario-A and Scenario-B.

Proposal 3: In Scenario-A, the absolute value of Ds_offset before and after RRH can be kept equal to 10m as in Rel-17.

Observation 2: Bidirectional Scenario-B deployment where the next closes RRH is used as serving does not provide the best possible RRH strength. This scenario is more feasible in Rel-18 when simultaneous multi-pen reception is considered.

Proposal 4: Consider bidirectional Scenario-B deployment where CPE is served by the closes RRH (i.e., not by the next closest) as one of the references for propagation conditions for PDSCH requirements.

Observation 3: Neither DPS channel profile introduced for HST FR2 (Clause B.3.4) not SFN channel profiles introduced for HST FR1 (Clause B4.5/B.3.6) cannot be re-used directly for HST FR2 deployment with simultaneous two-panel reception.

Proposal 5: Consider relative power trajectory and absolute delay trajectory as a part of HST FR2 channel profile for simultaneous two-panel reception in addition to the Doppler shift trajectory.

Proposal 6: For Doppler shift trajectories, re-used existing DPS model from Clause B.3.4.1 for the panel moving in the direction opposite to the serving beam orientation. Additionally, introduce Doppler profile for the panel moving in same direction with the serving beam orientation.

Observation 4: Relative power trajectories considered in HST FR1 SFN models cannot be re-used directly because they assume omni-directional coverage and do not take beamforming into account both on UE and RRH sides.

Proposal 7: Design power trajectories for HST FR2 simultaneous reception propagation conditions taking beamforming pattern on the UE and RRH sides.

- References

- R4-2305892, WF on demodulation requirements for Rel-18 FR2 HST, Samsung, RAN4#106bis-e, Online Meeting, 17th April- 26th April, 2023

- R4-2308037, On Demodulation Aspects of Tunnel Deployment Scenarios in HST FR2 Enhanced Nokia, Nokia Shanghai Bell, RAN4 #107, Incheon, Korea, May 22 – 26, 2023.