3GPP TSG-RAN WG4 Meeting #107 	R4-2308111

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	LS reply on PSFCH and S-SSB transmissions over non-contiguous RB sets

Agenda item:	8.31.1

Document for:	Discussion

Introduction

In the last RAN WG1 meeting  #112-bis-e, RAN1 has sent an LS (R1-2304218) to RAN4, regarding feasibility of transmitting multiple PSFCH or S-SSB over non-contiguous RB sets. In this document we discuss the implications of RAN1 request and provide a tentative answer.

1. Overall Description: RAN1 has discussed the scenario where a RX UE needs to transmit PSFCHs (using existing R16/17 PSFCH format 0) across multiple unlicensed channels within a SL BWP in a same slot. Similarly, RAN1 is also discussing UE transmitting S-SSBs across multiple unlicensed channels within a SL BWP in a same slot. RAN1 would like to seek RAN4’s opinion on the following questions. Question 1: whether multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets? Question 2: If multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets, is there a limitation(s) on e.g., number of RB sets, max. frequency separation between the RB sets, etc? The above same two questions also apply to UE transmitting S-SSBs over non-contiguous RB sets. Note that, according to existing RAN1 agreements, it is not possible to transmit PSFCH and S-SSB within a SL BWP in a same slot. 2. Actions: To RAN4: RAN1 respectfully requests RAN4 to provide feedback to the questions above for both multiple PSFCHs and multiple S-SSBs cases.

Discussion

Current RAN4 specifications from NR-U have no restrictions which would prevent non-contiguous RB sets transmissions in single carrier UL. It should be noted that UL non-contiguous 2 or 3CC is not supported (R4-2306482), therefore it is recommended that RAN1 down prioritizes efforts for non-contiguous CCs until the support for this type of configuration is concluded in RAN4.

Non-contiguous RB sets transmissions are not prevented for NR-U by current RAN4 specification for 1CC UL, but not supported in non-contiguous CCs.

The number of RB sets and maximum frequency separation are just limited by the respective channel bandwidth of the configured 1CC. For example, for 80MHz, the limit is 4 RB sets and 40 MHz separation.

Therefore, we propose RAN4 response to R1-2304218 LS on PSFCH and S-SSB transmissions over non-contiguous RB sets should refer to below two proposals.

Multiple PSFCHs and S-SSBs can be transmitted over non-contiguous RB sets for a single carrier of any available channel bandwidths defined in 38.101-1 Table 5.3.5-1 for n46, n96 and n102.

Number of RB sets and frequency separation is mainly limited by the channel bandwidth for the single carrier configurations, such as, a limit of 4 RB sets and 40 MHz separation for 80MHz.

RAN1 should down prioritize efforts for non-contiguous CCs configurations until the support is concluded for NR-U UL.

Proposed LS response is attached.

Conclusion

This paper has provided a discussion for the LS sent by RAN1 regarding feasibility of transmitting multiple PSFCH or S-SSB over non-contiguous RB sets.

Based on the discussion, the following Observations and Proposals were made:

- Observation 1: Non-contiguous RB sets transmissions are not prevented for NR-U by current RAN4 specification for 1CC UL, but not supported in non-contiguous CCs.

- Proposal 1: Multiple PSFCHs and S-SSBs can be transmitted over non-contiguous RB sets for a single carrier of any available channel bandwidths defined in 38.101-1 Table 5.3.5-1 for n46, n96 and n102.

- Proposal 2: Number of RB sets and frequency separation is mainly limited by the channel bandwidth for the single carrier configurations, such as, a limit of 4 RB sets and 40 MHz separation for 80MHz

- Proposal 3: RAN1 should down prioritize efforts for non-contiguous CCs configurations until the support is concluded for NR-U UL.

- References

- R1-2304218, LS on PSFCH and S-SSB transmissions over non-contiguous RB sets, 3GPP TSG-RAN WG1 Meeting #112-bis-e, e-Meeting, 17 – 26 April 2023.

- R4-2306482, WF on issues with non-contiguous NRU UL CCs, Skyworks Solutions, Inc., BT-plc.

- Annex Reply LS on PSFCH and S-SSB transmissions over non-contiguous RB sets

3GPP TSG-RAN WG4 Meeting #107 R4-23xxxxx Electronic Meeting, April 17 – 26, 2023 Title: Reply LS on PSFCH and S-SSB transmissions over non-contiguous RB sets Response to: R1-2304218 Release: Release 18 Work Item: NR_SL_enh2-Core Source: RAN4 To: RAN1 Cc: Contact Person: Name: Johannes Hejselbaek Tel. Number: E-mail Address: Johannes.hejselbaek (at) nokia (dot) com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN4 would like to thank RAN1 for the information in LS R2-2304218. Q1: whether multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets? RAN4 has discussed the question in the LS from RAN1 and concluded that there is impediment from RAN4 for non-contiguous transmission in single carrier configuration. Q2: If multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets, is there a limitation(s) on e.g., number of RB sets, max. frequency separation between the RB sets, etc? RAN4 has discussed the question in the LS from RAN1 and concluded that the limit in number of RB sets and frequency separation between RB sets is limited mainly by the channel bandwidth of the 1CC. Based on the above considerations RAN4 would like to provide the following answer to RAN1: Answer: Multiple PSFCHs and S-SSBs can be transmitted over non-contiguous RB sets for a single carrier of any available channel bandwidths defined in 38.101-1 Table 5.3.5-1 for n46, n96 and n102. The number of RB sets and separation are mainly limited by the channel bandwidth of the single carrier. 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take the above information into account in their further work. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting#108 Aug 21 – Aug 25, 2023 Toulouse, France RAN WG4 Meeting #108bis Oct 9 – Oct 13, 2023 Xiamen, China