3GPP TSG-RAN WG4 Meeting # 107	R4-22308124

# Incheon, Korea, 22th - 26th, May, 2023

Source: 	Samsung

Title: 	Views on 8Rx for CPE FWA vehicle industrial devices

Agenda Item:	8.5.1.2

Document for:	Discussion

- Introduction

In this paper, we provide our views on some remaining issues captured in WF [1].

- 2. Discussion

2.1 ΔRIB,8R

In last meeting, the group reached below agreement in terms of PDCCH AL and ΔRIB for 8Rx.

Issue 1-1-1: Value of PDCCH AL in RAN4 assumption

<Agreement in GTW>

- No consensus on PDCCH aggregation level for 8Rx in RAN4, and RAN4 does not specify PDCCH aggregation level in RAN4 specification.

Issue 1-1-2: Information to RAN5

It is suggested to discuss with issue 1-2-1.

Issue 1-2-1: ΔRIB for TDD bands

RAN4 stop the discussion on the AL issue and directly discuss ΔRIB.

Further discuss ΔRIB for TDD bands with the following values as a baseline:

- - [4.0-4.5] dB for n41

- - [4.0] dB for n77/n78/n79

Issue 1-3-1: ΔRIB for FDD band n7

- [4.5] dB for n7

It was agreed that RAN4 stop the discussion on the AL issue and directly discuss ΔRIB, and as we elaborated in [2], PDCCH AL is regardless of what kind of UE and how many Rx Chain the UE equipped, and has no significant effect on PDSCH REFSENS, however for 8Rx, PDCCH AL=4 may lead to demodulation threshold for PDCCH higher than PDSCH which should be avoided.

For the exact ΔRIB values, we are fine with the majority views in last meeting, i.e., -4.0dB for all TDD bands, and -4.5dB for FDD band n7, -4dB is reused from LTE requirements for TDD bands and -0.5dB difference between TDD and FDD band is reused from 4Rx ΔRIB. With that, LS to RAN5 seems not needed, since PDCCH AL level would not become the bottleneck for conformance test.

Proposal 1: For ΔRIB,8R, it is suggested to adopt -4.0dB for TDD bands n41/n77/n78/n79, and -4.5dB for n7.

2.2 Release independent

The agreement in last meeting is reproduced below.

Issue 3-3-1: Which release 8Rx can be release independent from.

<Way forward/Agreement>:

FFS the following approach can be agreeable in next meeting.

- 8Rx without SRS-AS can be release independent from Rel-15.

- 8Rx with SRS-AS can be release independent from Rel-X.

  - Further discuss “Rel-X” by clarifying which SRS-AS patterns were introduced from which release.

- Further discuss how to describe it in TS 38.307.

4Rx operation is introduced as release independent from Rel-15, and SRS antenna switching 1T2R, 1T4R and 2T4R are introduced from Rel-15 as well, so 4Rx operation could support 1T2R, 1T4R and 2T4R SRS-AS without any issue in Rel-15. However, as of Rel-16, the highest capability of SRS-AS is 4T4R, and XT8R (X=1, 2, 4) was introduced in Rel-17, so 8Rx operation with XT8R (X=1, 2, 4) at least should be release independent no earlier than Rel-17.

Proposal 2: 8Rx operation with SRS-AS XT8R (X=1, 2, 4) should be release independent no earlier than Rel-17.

2.3 Other Rx requirements

Three options are listed in WF and we still support option3 as the proponent in last meeting’s 2nd round comment, we see no need to verify Rx performance for both 4Rx and 8Rx if UE is equipped with 8 antenna ports.

Proposal 3: The UE RF Rx requirements in clause 7.2(Diversity characteristics) of 38.101-1 is supposed to be modified to:

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four or eight Rx ports shall be verified in operating bands where the UE is equipped with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE)

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

In addition, in our view, sub-clause 7.3.1(General of reference sensitivity) of 38.101-1 the following part also requires modification, as well as the relevant 38.101-3 part. Please note that actually LTE band 41/42/43 had already been endowed 8Rx operation, but it has not been mentioned in relevant 38.101-3 clause.

Proposal 4: The UE RF Rx requirements in clause 7.3.1(General of reference sensitivity) of 38.101-1 is supposed to be modified to:

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

in all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1a,  Table 7.3.2-1b and Table 7.3.2-1c or Table 7.3.2-1d with 2 Rx antenna ports tested;

for bands where the UE is required to be equipped with 4 Rx antenna ports, the UE shall additionally be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 or Table [7.3.2-2a] with 4 or 8 Rx antenna ports tested (selected based on the maximum Rx antenna port supported by the UE).

2.4 CA requirements

We support to enable 8Rx operation for CA explicitly in spec in this WID, actually it consumes great efforts to enable 4Rx for CA in TEI WI though the description appears quite simple now in current spec, and generally the conclusion and methodology for 4Rx CA could be reused for 8Rx CA operation.

<Way forward/Agreement>:

For 8Rx UE RF CA requirement, encourage interested companies to bring contributions in RAN4#107 with the following aspects:

- Expected clarification/revision on the objective in WID if needed considering whether to consider band combinations in this WI.

- For a band supporting 8Rx in single band operation, whether the band is mandated to support 8Rx in a band combination.

- For whether example CA/DC band combinations (including EN-DC) is needed or not, check if proposal 2 is agreeable in next meeting:

  - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC).

- For whether additional MSD is needed, check if proposal 3 is agreeable in next meeting:

  - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1.

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

In our view, a band supporting 8Rx operation for single band does not mean it would definitely adopt 8Rx for CA operation, flexibility should be left for UE implementation for CA. In addition, we are fine with proposal 2 and proposal 3 above (minor modification needed), which is similarly as for 4Rx and implemented for both intra and inter-band NR-CA and EN-DC.

Proposal 5: One band declaring the support of 8Rx operation for single band should not be mandated to support 8Rx operation for the relevant CA.

Proposal 6: For NR-CA/EN-DC operation with 8Rx on the constituent band(s):

- 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC).

- Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence.

- For 38.101-1: For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

-   For 38.101-3: For operations with 8 Rx antenna ports in an E-UTRA band or an NR band, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.1-1aa of TS 36.101[4] for the E-UTRA band or in Table 7.3.2-x of TS 38.101-1 for the NR band when MSD > 0.

2.5 Whether 6Rx could be deemed as implementable feasible?

Given that 6Rx for FWA could be considered as a degraded capability compared with 8Rx, we think 6Rx could be deemed as implementable feasible after 8Rx is finalised though there is no specific Rx requirements for 6Rx. Implementable feasible here generally means there is no hardware challenge to implement 6Rx for FWA type UE for single band, intra-band CA and inter-band CA with overlapping bands such as DC_42_n77, and 6Rx could be a general feature applicable for other UE features such as Type-4a UE capability for non-collocated deployment which requires 2 dedicate Rx for 42 and 4 dedicate Rx for n77.

Proposal 7: 6Rx for FWA type UE could be deemed as implementable feasible after 8Rx is finalised.

- 3. Conclusion

Proposal 1: For ΔRIB,8R, it is suggested to adopt -4.0dB for TDD bands n41/n77/n78/n79, and -4.5dB for n7.

Proposal 2: 8Rx operation with SRS-AS XT8R (X=1, 2, 4) should be release independent no earlier than Rel-17.

Proposal 3: The UE RF Rx requirements in clause 7.2(Diversity characteristics) of 38.101-1 is supposed to be modified to:

  - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four or eight Rx ports shall be verified in operating bands where the UE is equipped with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE)

  - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.”

Proposal 4: The UE RF Rx requirements in clause 7.3.1(General of reference sensitivity) of 38.101-1 is supposed to be modified to:

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

in all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1a,  Table 7.3.2-1b and Table 7.3.2-1c or Table 7.3.2-1d with 2 Rx antenna ports tested;

for bands where the UE is required to be equipped with 4 Rx antenna ports, the UE shall additionally be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 or Table [7.3.2-x] with 4 or 8 Rx antenna ports tested (selected based on the maximum Rx antenna port supported by the UE).

Proposal 5: One band declaring the support of 8Rx operation for single band should not be mandated to support 8Rx operation for the relevant CA.

Proposal 6: For NR-CA/EN-DC operation with 8Rx on the constituent band(s):

- 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC).

- Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence.

- For 38.101-1: For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

-   For 38.101-3: For operations with 8 Rx antenna ports in an E-UTRA band or an NR band, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.1-1aa of TS 36.101[4] for the E-UTRA band or in Table 7.3.2-x of TS 38.101-1 for the NR band when MSD > 0.

Proposal 7: 6Rx for FWA type UE could be deemed as implementable feasible after 8Rx is finalised.

- 4. Reference

[1] R4-2306597 draft WF on 8Rx UE RF requirements_v16, NTT Docomo, RAN4#106-bis-e

[2] R4-2301106, Views on 8Rx for CPE/FWA/vehicle/Industrial devices, Samsung, RAN4#106