3GPP TSG-RAN4 Meeting#107	< R4-2308134>

# Incheon, Korea, 22th-26th, May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1549 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | ❌ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Cat A CR for 38.101-1 Correct the MSD test configuration for CA_n18-n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_2BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | <2023-05-10> |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Below test configuration for MSD due to harmonic mixing for CA_18-n41 is incorrect since n41 does not supports 5MHz for n18-n41 |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration for MSD due to harmonic mixingfor CA_18-n41 remains incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 7.3A.4-4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n26 | 5 | 15 | 25 (RBstart=0) | 5 | 3.7 | NOTE 1 | UL1/DL2 |
| n7 | n263 | 25 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE 10 | UL1/DL3 Near miss |
| n25 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.5 | NOTE 4 | UL1/DL3 |
| n25 | n713 | 20 | 15 | 100 (RBstart=0) | 20 | 15.3 | NOTE 4 | UL1/DL3 |
| n40 | n20 | 5 | 15 | 25 (RBstart=0) | 5 | 27.8 | NOTE 4 | UL1/DL3 |
| n40 | n20 | 20 | 15 | 100 (RBstart=0) | 20 | 20.3 | NOTE 4 | UL1/DL3 |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 30.3 | NOTE 4 | UL1/DL3 |
| n40 | n77 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 1 | UL3/DL2 |
| n40 | n77 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 1 | UL3/DL2 |
| n40 | n78 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 1 | UL3/DL2 |
| n40 | n78 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 1 | UL3/DL2 |
| n41 | n183 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 15 | 15 | 75 (RBstart=0) | 15 | 22.5 | NOTE 4 | UL1/DL3 |
| n41 | n48 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 9 | UL4/DL3 |
| n41 | n48 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 9 | UL4/DL3 |
| n41 | n78 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 1 | UL3/DL2 |
| n41 | n78 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 1 | UL3/DL2 |
| n46 | n7 | 5 | 15 | 12 (RBstart=0) | 5 | 8.3 | NOTE 7 | UL1/DL2 |
| n46 | n7 | 5 | 15 | 24 (RBstart=0) | 50 | 0.6 | NOTE 7 | UL1/DL2 |
| n46 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 22.6 | NOTE 2 | UL2/DL3 |
| n46 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 19.5 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 3,7 | NOTE 7 | UL1/DL2 |
| n77 | n5 | 5 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 | NOTE 8 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n12 | 15 | 15 | 75 (RBstart=0) | 15 | 26.2 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 1.1 | NOTE 7 | UL1/DL2 |
| n776 | n29 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n776 | n29 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 24 (RBstart=0) | 10 | 8.0 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 0.9 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |
| n776 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 7 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n85 | 15 | 15 | 75 (RBstart=0) | 15 | 26.2 | NOTE 5 | UL1/DL5 |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 80 | 4.5 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |
| n78 | n67 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n67 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 25 | NOTE 5 | UL1/DL5 |
| n96 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 5.8 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 0.5 | NOTE 2 | UL2/DL3 |
| n102 | n1 | 20 | 15 | 100 (RBstart=0) | 5 | 13.5 | NOTE 4 | UL1/DL3 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 9: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

<End of change>