# 3GPP TSG-RAN WG4 Meeting # 107                               R4-2308169

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on UE RF of intra-band non-collocated EN-DC and 		NR-CA

# Source: 	ZTE Corporation

# Agenda item:	8.12.2

# Document for:	Approval

# 1 Introduction

In RAN4#106 bis-e meeting, the discussion for Type-3a/3b and was postponed, while some issues about Type 2 UE need further discussion which are listed below[1].

< Issue 2-1-2: Whether Type 1 shall be default when no request comes from BS > Way Forward: - Type 1 is default as baseline to continue the following discussions. - How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2. - Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution. - FFS about the definition of Type1 in the next meeting. < Issue 2-1-2-1: How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2 > Way forward: - Encourage companies to provide their views on how UE behave in case of UE is configured change from default (Type 1 UE) to Type 2. - Continue further discussions in the next meeting. < Issue 2-1-2-2: Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution > Way forward: - Continue further discussions in the next meeting.

In this contribution, we give simple discussion on the type 1 and type 2 configuration for NW operating in co-located or non-colocated scenario.

# 2	Discussion

Type 1 and type 2 are introduced for different deployments. Type 1 is for co-located deployment and Type 2 is for non-co-located deployment which incurs large MRTD(RTD) and large power imbalance. As agreed last meeting that the default UE type is Type1 which requires co-located deployment.

The following table shows the architecture of different types of UE[2]

Table 1 UE architecture of different types

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

From the table above we can find that Type 1 UE support 4 layer MIMO per CC and Type 2 UE support 2 layer MIMO per CC.

As agreed in last meeting that Type 1 is the default type[3] , which means the default transmission mode from the network point of view is Type 1 transmission. If UE reports the capability of Type 2, the network can still configure the UE as Type 1 or Type 2. But maybe some interaction between UE and network is needed, such as a new BS signalling. In this case, when assistance information from BS signalling was sent to UE, in this case UE can know whether BS is operating in co-located or non-colocated scenario. In our understanding, to acquire higher order MIMO transmission and high throughput, UE should be configured as Type 1 transmission if UE knows NW is under the co-located deployment.

Proposal 1. To consider BS signalling for the interaction between UE and network to achieve more suitable type configuration between Type 1 and Type 2.

# 3 Conclusion

In this contribution, we give some discussions for inter-band ENDC and intra-band non-contiguous NR CA type 3a/3b UE. The conclusions are:

Proposal 1. To consider BS signalling for the interaction between UE and network to achieve more suitable type configuration between Type 1 and Type 2.

# 4	Reference

[1]  R4-2306605, WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI

[2]  R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI