# 3GPP TSG-RAN WG4 Meeting # 107                                  R4-2308173

# Incheon, KR, May 22 – May 26, 2023

# Title: 	On lower MSD for inter-band CA/ENDC

# Source: 	ZTE Corporation

# Agenda item:	8.5.1.3.2

# Document for:	Approval

# 1 Introduction

The lower MSD topic was extensive discussed in the previous RAN4 meetings. In this contribution, we continue to discussed some issues about the lower MSD issues based on the WF [1].

# 2	Discussion

## 2.1 Conditions to indicate the lower MSD capability threshold(s)

In RAN4 #106 meeting, it was already agreed that UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved. However, it was overturned in RAN4 #106bis meeting, instead new agreements were achieved, which were shown as below.

Issue 2-1-2: Conditions to indicate the lower MSD capability - Candidate options - Option 1: For the purpose of MSD improvement, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case (Samsung, Xiaomi, Nokia, AT&T, Skyworks, HW) - If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD - Option 2: For the purpose of MSD improvement, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case (QC, OPPO, vivo) - If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD - Option 3: Others (Meta, MediaTek, Apple)

It was agreed in previous meeting that it’s the UE reported MSD capability and let gNB use such information to determine final behavior considering the trade-off between UL performance gain and DL degradation rather than letting gNB determine candidate values and letting UE report which/whether is supported, which means reported lower MSD capability is purely UE autonomous. There are no conclusions for the specific NW behaviour and how to schedule UEs with different MSD capabilities is up to NW implementation.

Observation 1. Reporting lower MSD capability is UE autonomous, no restriction on NW behaviour on how to schedule UEs with different MSD capabilities.

In option 1 and option 2, the conditions are given when the MSD value defined in the spec falls between two threshold values. However, although the final sets of threshold values are not concluded, if when MSD value defined in the spec is beyond the max.threshold but actual MSD is in the range of the thresholds, we think UE can report the lower MSD capability. Therefore, we think the key point is whether or not the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), if the actual MSD is our of the range, the UE cannot report lower MSD capability for this REFSENS exception case, that is the purpose of introduction of threshold to our understanding.

As we discussed in our paper in [2], we also think it is meaningless to report lower MSD capability with much smaller amount of MSD improvement, for example, <1 dB MSD improved would be meaningless. This point was also somehow mentioned in option 1. In last meeting, company propose at least 3dB better than the corresponding MSD value in RAN4 specifications [3], we think it could be used it as starting point. We think there is no need to further define the conditions for the relationship between the MSD value defined in the spec and MSD threshold if RAN4 define appropriate sets of threshold values and TBD improved at least by TBD dB. Therefore, we have following proposal.

- Proposal 1. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case

- - If UE reports the lower MSD capability, the reported MSD value should be improved at least by [3] dB against a specified MSD

## 2.2 Lower MSD threshold(s)

The issue is the essential information included in the lower MSD capability, the agreements are:

- Issue 1-2-2: Whether CBW of aggressor UL and victim DL should be reported for lower MSD capability - Option 1: Yes (CHTTL, ZTE) - Option 2: No (Samsung, Xiaomi, HW, vivo, Meta, Skyworks, QC, Apple, OPPO, AT&T) - Option 3: Discuss together with conformance test (Samsung, Nokia) - Recommended WF - FFS in next meeting.

It was agreed that power class should be reported together with MSD values

For the CBW, this question was raised when the worst case MSD DL/UL config are not supported by a real UE or new maximum agg. UL or victim DL are introduced. In current specification, MSD requirements are usually defined based on the supported channel bandwidth for BCS0/1/2/3/4 and 5, and MSD test point may be updated if new maximum agg. UL or victim DL are introduced. For example, a UE support a BC with min. BW of 10MHz and max. BW of 40MHz for constituted band in BCS0 and the MSD value was defined for the test point 10MHz and 40MHz, but <10MHz and >40MHz CBW are introduced for the constituted bands and BCS 4 and 5 is introduced for this BC, then the original MSD test point for 10MHz and 40MHz would be erased and new MSD test point will be defined for the new channel bandwidths<10MHz and >40MHz with new MSD values since The MSD for the victim band are derived from the REFSEN of the victim DL bandwidth [2].

Therefore, we think CBW of aggressor UL and victim DL should be reported since we need to consider all the supported BCSs and future-proofing. Considering there is running LS to RAN5 for the test configuration, it is fine to to wait for RAN5’s feedback.

Proposal 2. CBW of aggressor UL and victim DL should be reported, but it is fine to wait for RAN5’s feedback.

## 2.3 MSD types and orders

- Sub-topic 1-3: MSD types and orders - WF - The MSD source to be reported is selected from the set of {UL harmonic, Harmonic mixing, cross-band ISO, IMDn}. - FFS handling of UL/DL harmonic order - FFS on max MSD order n - If new MSD type is identified and specified by RAN4 in the future, the new MSD type can also be considered for indicating lower MSD capability

For UL/DL harmonic order for harmonic mixing MSD, it seems only one UL/DL harmonic order defined each band combination, in the other words, there are no band combinations share more than one UL/DL harmonic order MSD in current spec. So it seem no need to include for harmonic mixing MSD. However, it should be included for harmonic MSD.

For IMD order n, in terms of the previous agreements in the WF R4-2105336, MSD analysis should be performed at least up to IMD13 for contiguous UL CA and IMD11 for non-contiguous UL CA if filter rejection is less than 20dB. In the other words, there is no need to analysis the IMD order larger than 13. In order to align with the agreements, n should be 13.

For cross band isolation MSD, for CA_n5-n28, on top of 1 aggressor NR UL band, cross band isolation caused by 2 aggressor NR UL bands are defined, so the types of MSD should also be considered for indicating lower MSD capability.

Proposal 3. For UL/DL harmonic order, there is no need to be included for harmonic mixing MSD, but it should be included for harmonic MSD.

Proposal 4. For IMDn, n should be 13.

Proposal 5. New cross band isolation MSD types, i.e. from 2 aggressor NR UL bands, should be considered for indicating lower MSD capability.

## 2.4 MSD thresholds

- Sub-topic 1-4: Candidate MSD thresholds - Candidate options: - Option 1: 0~15dB (Nokia, AT&T) - Option 2: 0~20dB (HW, Samsung, Skyworks, Meta, vivo, OPPO, Xiaomi) - Option 3: 0~[25/27]dB (LGE, QC) - Option 4: improved MSD in granularity of 1dB (MTK, ZTE)

This should be split into two issue:

1. Thresholds

2. Granularity

For the thresholds, at least we think 15dB is not enough as we discussed in [2]. Too large threshold makes little sense since we are discussing possible lower MSD. Although there are few BC has >35dB MSD, we believe the previous survey where 20dB improved MSD make sense. So 0~20dB MSD thresholds range with small granularity should be considered. For the granularity, integer granularity shall be considered, and less than [3]dB granularity can be considered.

Proposal 6. 0~20dB MSD thresholds range with small granularity such as [1, 2, 3]dB should be considered.

# 3 Conclusion

In this contribution, we give some discussion on the lower MSD for inter-band NR CA/ENDC based on the latest agreements. The conclusions are:

- Proposal 1. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case

- - If UE reports the lower MSD capability, the reported MSD value should be improved at least by [3] dB against a specified MSD

Proposal 2. CBW of aggressor UL and victim DL should be reported, but it is fine to wait for RAN5’s feedback.

Proposal 3. For UL/DL harmonic order, there is no need to be included for harmonic mixing MSD, but it should be included for harmonic MSD.

Proposal 4. For IMDn, n should be 13.

Proposal 5. New cross band isolation MSD types, i.e. from 2 aggressor NR UL bands, should be considered for indicating lower MSD capability.

Proposal 6. 0~20dB MSD thresholds range with small granularity such as [1, 2, 3]dB should be considered.

# 4	Reference

[1] R4-2306590, WF on lower MSD, Huawei, HiSilicon

[2] R4-2305130, On lower MSD for inter-band CA/ENDC, ZTE

[3]R4-2305656, Input on improved MSD signaling and requirement, Skyworks Solutions Inc.

[4] R4-2306591, draft LS on clarification of test configurations for CA/DC MSD requirements, Samsung