3GPP TSG-RAN4 Meeting #107												R4-2308195

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.20.2.2.1

Source: 	CMCC

Title: 	study on feasibility at BS side

Document for:	Discussion

# 1. Introduction

A WF on SBFD feasibility study and BS aspects RF impact - has been approved [1] and one LS send out to RAN1 with certain agreements [2]. In this contribution, we focus on the discussion of remaining issues.

# 2. Discussion

- NF modeling

In last meeting, there is only agreements for FR1 WA gNB NF modeling whereas no agreements for FR1 MR and LA gNB NF modeling.

For FR1 MR and LA BS classes, we could extend the modeling with following principle:

- Point A equals to in band blocking level

- Point C equals to NF which is assumed in legacy RAN4 analysis

- Point B is less than out of band blocking, i.e. -15dBm. in theory, MR and LA would accept larger blocking level compared with WA considering larger NF assumption. For WA, point B is 18dB larger than point A. so, for MR and LA, it is reasonable to assume the same or even larger shift between point A and point B

- The slope from point (A,C) to point (B,D) is 0.5, the same as WA

Proposal 1: following NF modelling assumption is reasonable. Besides, even larger point B is also OK.

- For FR1 MR: (A, B, C, D) = (-38, 10, -20, 19)

- For FR1 MR: (A, B, C, D) = (-35, 13, -17, 22)

- ACS and ICS value

Last meeting agreements are listed as below:

ACSBS value range achievable by typical FR1 gNB implementation: • The range from [46 or 50]dB as lower bound to [62]dB as upper bound is proposed in RAN4. • The above range is proposed by considering both noise figure model and ACSBS in the simulation, in which the noise figure model has captured the non-linearity of RF components and ACSBS refer to baseband suppression. The value of ICSBS by typical FR1 gNB implementation: • Based on the similar baseband suppression, the same range of ACS values can be achievable for ICS. o Lower bound is 46dB. o Upper bound is [62]dB.

As stated before, commercial gNB could at least achieve 50dB Rx selectivity for FR1, which is the contribution from both RF and digital suppression performance. So ACSBS/ICSBS which refer to baseband suppression only should at least better than 50dB.

Proposal 2: for the ACSBS/ICSBS which refer to baseband suppression should be at least larger than 50dB.

- spatial isolation to avoid blocking

For RSIC, one basic target is to avoid blocking at LNA input port. the total power at LNA input port is the sum of self-interference, inter-sector interference and inter-site interference.

- For self-interference, if we assume 80dB spatial isolation, input power at Rx side is -34 or -41dBm for 46dBm and 49dBm Tx power assumption.

- For inter-sector interference, input power at Rx side is from -16dBm to -44dBm with assumed 62 to 93dB inter sector spatial isolation

- For inter-site interference, this is mainly related to deployment layout of SBFD network and it’s better not restrict spatial isolation value to allow deployment flexibility. According to co-existence simulation, min gNB-gNB CL is almost 84dB, equally to -38dBm and -35dBm input power at Rx side for 46dBm and 49dBm Tx power assumption.

Now, it seems certain assumed spatial isolation value of self-interference and inter-sector interference may lead to larger input power than assumed -43dBm blocking level if without additional RF RSI or sub-band filter. Commercial gNB device could achieve better blocking capability and there is certain margin against -43dBm assumption. But even though, it seems additional RF RSI or sub-band filter solution is necessary to avoid severe interference considering some cases that spatial isolation is hard to be enlarged, e.g. the Tx aggressor and Rx victim part are in close proximity and hard to be distant to contribute larger spatial isolation.

Observation 1: it seems certain assumed spatial isolation value may lead to larger input power than assumed -43dBm blocking level if without additional RF RSI or RF sub-band filter solution.

Proposal 3: RF RSI or RF sub-band filter solution would help to reduce input power and avoid blocking in some cases when spatial isolation is limited. It’s encouraged to have detailed analysis of RF RSI feasibility or RF sub-band filter solution feasibility in final TR.

# 3. Conclusions

In this contribution, SBFD at gNB side is analyzed with following observations and proposals:

Proposal 1: following NF modelling assumption is reasonable. Besides, even larger point B is also OK.

	For FR1 MR: (A, B, C, D) = (-38, 10, -20, 19)

	For FR1 MR: (A, B, C, D) = (-35, 13, -17, 22).

Proposal 2: for the ACSBS/ICSBS which refer to baseband suppression should be at least larger than 50dB.

Observation 1: it seems certain assumed spatial isolation value may lead to larger input power than assumed -43dBm blocking level if without additional RF RSI or RF sub-band filter solution.

Proposal 3: RF RSI or RF sub-band filter solution would help to reduce input power and avoid blocking in some cases when spatial isolation is limited. It’s encouraged to have detailed analysis of RF RSI feasibility or RF sub-band filter solution feasibility in final TR.

# 4. Reference

[1] R4-2217464, WF on SBFD feasibility study and RF impact: BS aspects, Samsung