3GPP TSG-RAN4 Meeting #107												R4-2308198

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.21.2

Source: 	CMCC

Title: 	Discussion on low power WUS architecture

Document for:	Discussion

# 1. Introduction

In RAN4 #106bis e-meeting, one WF[1] and LS[2] are approved as starting point to help further discussion. In this contribution, we focus on the discussion of ACS related issues.

# 2. Discussion

- Categorize RF requirement set for different UE type

Based on RAN1 agreements, RAN4 approve to consider all the UE types mentioned in the SID, e.g. IoT devices, Wearable devices, and e-MBB devices. The cost aspect can also be considered for the design of LP-WUS/WUR scheme. Different types require different power consumption from a few uW to a few hundred uW. Different power consumption will lead to different NF. Following fig [3] show the relationship between power consumption and NF. RF requirement analysis may need to categorized by different UE types with different power consumption assumption.

Observation 1: RF requirement analysis may need to be categorized by different UE device with different power consumption assumption.

- Analysis of ACS

Usually, ACS is derived based on co-existence study to meet max 5% throughput loss. For LP-WUS, it’s incomplete to only focus on ACS performance analysis, i.e. typical filter characteristic, guard RB size and RF impairment analysis only. But now it’s immature to perform simulation since waveform is even not completed in RAN1. Therefore, it’s suggested to emphasize into final LS to RAN1 that there is no verification of ACS from co-existence simulation.

Proposal 1: it’s suggested to be emphasized into reply LS to RAN1 that final ACS is only based on LP-WUS performance analysis and only for RAN1 analysis purpose whereas there is no detailed verification of co-existence issue considering there is no conclusion of LP-WUS signal design.

According to approved framework in [1], RAN4 could conclude several sets of guard RB and corresponding adjacent channel selectivity performance based on typical filter characteristic. But if we want to down-select to one or several guard RB we have to know targe ACS value although current it’s hard to be concluded from simulation.

Proposal 2: the methodology for guard RB is that at first conclude the relationship between guard RB and adjacent channel selectivity based on typical LP-WUS performance. And then down-select guard RB based on target ACS value although current it’s hard to be concluded from simulation.

Following show the target ACS value analysis based on the same interference level and REFSENSE degradation assumption as legacy UE.

| parameter | value |  | comment |
| --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |
| P_interference: Interference power for ACS requirement | =REFSENS Normal_UE + 45.5dB=-52.48dBm/5MHz | -25dBm | We use n41 as example. The same as the value for ACS testing considering such interference mainly comes from adjacent channel gNB power even for LP-WUR case |
| REFSENSE degradation due to adjacent channel interference | +14dB | -56.5dBm | This is also related to target coverage. +14dB is the same as legacy UE. But when target coverage shrinks for other UE device, large REFSENSE degradation is allowed, which will lead to smaller ACS. |
| ACS | 31.7 for 9dB NF assumption 25.7 for 15dB NF assumption 16.7 for 24dB NF assumption | 31.5 for 9dB NF assumption 25.5 for 15dB NF assumption 16.5 for 24dB NF assumption |  |

Based on above analysis, we have following observations.

Proposal 3: if we assume the same coverage as normal UE, target ACS is about 32dB for 9dB NF, 26dB for 15dB NF and 17dB for 24dB NF.

For adjacent sub-carrier analysis, since there is no CBW-passband filter, above analysis value also applies for adjacent sub-carrier selectivity analysis.

Proposal 4: the same value of ACS is also applicable for sub-carrier selectivity.

- Dedicated LP-WUS operation band

LS [2] is sent to RAN1 to ask for the possibility of same CBW of LP-WUS as the carrier CBW.  If the case that LP-WUS occupying whole carrier CBW is contained in RAN1 study, then it’s RAN4’ responsibility to decide whether to consider dedicated LP-WUS operation band. Dedicated LP-WUS operation band or example band for analysis should be triggered by operator or spectrum management organization.  From deployment cost point of view, operator will not prefer to deploy LP-WUS dedicated network. Instead, updating commercial deployed network to support LP-WUS feature is more preferred. Therefore, global operation band with commercial network is more suggested.

Proposal 5: it’s RAN4’s responsibility to decide whether to consider dedicated LP-WUS operation band. besides, final example band should be global operation band with commercial network proposed by operator or spectrum management organization.

Band n28 and n41 have been commercially deployed by many operators among the world which could be used as example band if finally approved to define dedicated LP-WUS operation band.

# 3. Conclusions

In this contribution, LP-WUR architecture and RF requirements are discussed with following observations:

Observation 1: RF requirement analysis may need to be categorized by different UE device with different power consumption assumption.

Proposal 1: it’s suggested to be emphasized into reply LS to RAN1 that final ACS is only based on LP-WUS performance analysis and only for RAN1 analysis purpose whereas there is no detailed verification of co-existence issue considering there is no conclusion of LP-WUS signal design.

Proposal 2: the methodology for guard RB is that at first conclude the relationship between guard RB and adjacent channel selectivity based on typical LP-WUS performance. And then down-select guard RB based on target ACS value although current it’s hard to be concluded from simulation.

Proposal 3: if we assume the same coverage as normal UE, target ACS is about 32dB for 9dB NF, 26dB for 15dB NF and 17dB for 24dB NF.

Proposal 4: the same value of ACS is also applicable for sub-carrier selectivity.

Proposal 5: it’s RAN4’s responsibility to decide whether to consider dedicated LP-WUS operation band. besides, final example band should be global operation band with commercial network proposed by operator or spectrum management organization.

# 4. Reference

[1] R4-2306617, WF on LP-WUS, vivo

[2] R4-2306618, Reply LS to RAN1 on low-power wake-up receiver architectures, vivo

[3] R4-2302276, Low-power wake-up receiver RF aspects, Qualcomm Inc.

[4] R4-2304342, On low-power wake-up signal and receiver for NR, Apple