3GPP TSG-RAN4 Meeting #107												R4-2308199

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.7.3.1

Source: 	CMCC

Title: 	Beam correspondence requirements for initial access

Document for:	Discussion

# 1. Introduction

Last meeting, WF is approved [1]. In this contribution, we focus on the discussion of beam correspondence requirement for initial access and INACTIVE state.

# 2. Discussion

## 2.1 min peak EIRP and spherical coverage

<Way forward>: - Option 1: Do not specify the min peak EIRP requirements but Same spherical coverage as RRC_Connected mode - Option 2: Lower than the min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode - Option 3: Same as min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode - Option 4: For beam correspondence in initial access, reuse the same beam lock mode and the same beam type assumption as that of connected mode beam correspondence. And accordingly EIRP requirements and RS side condition can also be reused.

Above is the agreement of min peak EIRP in last meeting, option 3 is more preferred. The motivation of min peak EIRP is to enhance UL coverage. Usually when FR1 and FR2 are both used by one operator, FR2 will have higher cell selection priority at IA state. Initial access performance is the dominant factor for final commercial network coverage of FR2. For the UE with relatively bad min peak EIRP performance at IA, it can’t access network even with much better min peak EIRP performance at RRC_CONNECTED since UE even can’t come to RRC_CONNECTED state using FR2. If we finally define lower EIRP for IA compared with RRC_CONNECTED, this means we shrink FR2 UL coverage and the performance gain of better RRC_CONNECTED performance is limited.

Observation 1: If we finally define lower EIRP compared with RRC_CONNECTED, we shrink FR2 UL coverage and the performance gain of better RRC_CONNECTED performance is limited.

During previous meeting, the main argument for lower EIRP is that target SNR for PRACH is less than that of PUSCH at gNB side according to 38104. But referring to 38104, it seems there is still some cases where the required target SNR is similar for PRACH and PUSCH. For example, for QPSK G-FR2-A3-3/16/9/21, the target SNR for PUSCH is -2.5~1.4dB. for PRACH normal mode TDLA30-300 low condition, the target SNR is -1.7~1.2dB. UE usually is scheduled with QPSK scheme when it is at cell edge, where the min EIRP works. So from cell coverage point of view, peak EIRP for PRACH should be the same as RRC_CONNECTED state.

Observation 2: at cell edge, the required target SNR is similar for PRACH and PUSCH. For example, for QPSK G-FR2-A3-3/16/9/21, the target SNR for PUSCH is -2.5~1.4dB. for PRACH normal mode TDLA30-300 low condition, the target SNR is -1.7~1.2dB.

To not shrink UL coverage, it’s suggested to define the same min peak EIRP as RRC_CONNECTED as starting point. The same reason as min EIRP, there is no need to relax spherical coverage requirement in IA compared with CONNECTED state.

Proposal 1: it’s better to define the same min peak EIRP and spherical coverage as RRC_CONNECTED for IA and RRC_INACTIVE.

## 2.2 RAR

The agreement in last meeting is listed as below:

- Proposals - Option 1: RAR is included. - Option 2: RAR is not included. - Option 3: If no consensus on re-use the same requirement as in the connected for at least spherical coverage, the RAR reception-based BC test can be taken as an alternative method for accommodating different beam patterns and UE implementations. - Option 4: Keep the previous agreement on RAR to focus on Msg1 requirement first, then discuss RAR later. Way forward/Agreements: - focus on Msg1 requirement first, then discuss RAR later

For msg1, beam correspondence is to derive msg1 Tx beam based on SSB Rx beam. For RAR, the beam correspondence is to derive RAR Rx beam based on msg1 Tx beam. Different beam types lead to different requirements. following table show all Tx/Rx beam type combinations for msg 1 BC and RAR BC.

Table 1: all Tx/Rx beam type combinations for msg 1 BC and RAR BC

| Msg 1 Tx beam type | Msg1 BC |  | RAR BC |  |
| --- | --- | --- | --- | --- |
|  | Rough SSB Rx beam | Fine SSB Rx beam | Rough Rx beam | Fine Rx beam |
| rough | rough beam based on rough beam | rough beam based on fine beam | rough beam based on rough beam | fine beam based on rough beam |
| fine | fine beam based on rough beam | Fine beam based on fine beam, which could be verified by RRC_CONNECTED state | rough beam based on fine beam | Fine beam based on fine beam, which could be verified by RRC_CONNECTED state |

As analyzed above, if all beam types are allowed in IA. Msg 1 BC has already included all Tx/Rx beam type combinations.

Observation 3: msg1 has already covered all Tx/Rx beam type combinations and there is no need to define specific requirements for RAR.

## 2.3 tolerance requirements

Way forward/Agreements: - BC tolerance requirement defined in Rel-15 is not applicable to initial access and RRC_INACTIVE state, according to WID “Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping.” - Encourage companies to provide justification if relaxation is needed in IA for UE not supporting BC without UL beam sweeping.

Above are the agreements in last meeting.

At initial access state, UE can’t perform beam sweeping. If there is any relaxation for UE not supporting BC without UL beam sweeping, that means actual UL EIRP toward gNB is less than required min EIRP, this is not allowed. We have to make sure the min EIRP toward target gNB direction is all the same for all UEs.

Proposal 2: the min EIRP toward target gNB direction should be all the same for all UEs, rather than allow some relaxation.

Considering certain UE may can’t perform better beam correspondence, one options is that such UE are required to perform better peak EIRP, then even with some tolerance toward target direction, final EIRP toward gNB is the same for all UEs.

# 3. Conclusions

In this contribution, beam correspondence requirements for initial access are discussed with following observations and proposals:

Observation 1: If we finally define lower EIRP compared with RRC_CONNECTED, we shrink FR2 UL coverage and the performance gain of better RRC_CONNECTED performance is limited.

Observation 2: at cell edge, the required target SNR is similar for PRACH and PUSCH. For example, for QPSK G-FR2-A3-3/16/9/21, the target SNR for PUSCH is -2.5~1.4dB. for PRACH normal mode TDLA30-300 low condition, the target SNR is -1.7~1.2dB.

Proposal 1: it’s better to define the same min peak EIRP and spherical coverage as RRC_CONNECTED for IA and RRC_INACTIVE.

Observation 3: msg1 has already covered all Tx/Rx beam type combinations and there is no need to define specific requirements for RAR.

Proposal 2: the min EIRP toward target gNB direction should be all the same for all UEs, rather than allow some relaxation.

# 4. Reference

[1] R4-2306600, WF on beam correspondence requirements for initial access and RRC_inactive mode, Nokia