3GPP TSG-RAN WG4 Meeting #107 	R4-2308215

# Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on general aspects in R18 LTM

Agenda Item:	8.25.2.1

Document for:	Discussion

Introduction

In RAN4 106 and RAN4 106-bis-e, R18 L1L2-triggered mobility was discussed in RRM session and the WFs are agreed in [1], [2].

Moreover, in this meeting, RAN4 receives one LS [3] from RAN1 that needs RAN4’s action. Another LS [4] from RAN2 is also received.

Based on all above information, we provide our views on the general aspects for R18 L1L2-triggered mobility.

Discussion

<On	downlink synchronization before cell switch>

In last RAN4 meeting, the following issue is capture in [2].

Issue 1-2-1: Requirements for DL synchronization before cell switch command

< Way forward >:

  - RAN4 do not need to define any new requirements for obtaining symbol boundary and frame boundary of target cell before cell switch command, as legacy requirements for PSS/SSS detection and time index detection apply.

  - RAN4 do not need to define any new requirements for acquiring SFN of target cell before cell switch command.

  - Further discuss whether and how to define requirements for SSB based T/F fine tracking on neighbour cell based on further RAN1/2 progress.

RAN4 has already agreed to relax SFN offset alignment assumption if rough-Rx-beam-based L3 measurement is performed before L1 measurement. If L3 measurement is performed, UE can obtain rough sync information of the target cell in the L3 measurement. However, we are not sure whether the downlink sync mention by RAN1 only means the L3-measurement-based synchronization.

In previous RAN1 meetings, the following agreements are achieved. Based on RAN1 agreements, it is clear that TCI activation can be received before cell switch command, which is at least for scenario 2. That means, UE may or may not activate the corresponding TCI, i.e. at least perform DL sync based on the TCI, before the cell switch command is received. As also agreed in RAN1 below, such downlink synchronization would be at least based on SSB. In our understanding, if ICBM is considered, there is no additional effort for UE to support such DL sync, since anyway TCI activation for NSC will be done if UE supports and is configured with ICBM. But for non-ICBM scenario, in last meeting, some companies prefer to leave supporting of DL pre-sync to UE capabilities, due to the concern in unnecessary UE power consumption. In our understanding, for non-ICBM scenario, the DL sync before cell switch will of course cause more UE power, but may reduce the HO delay significantly. Therefore, we are open for UE capability in this case.

Observation 1  For LTM under ICBM scenario, there is no additional effort for UE to support downlink synchronization, i.e. activation of target cell TCI, before cell switch command is received.

Observation 2  For LTM under non-ICBM scenario, to support downlink synchronization, i.e. activation of target cell TCI, before cell switch command is received, additional UE effort/consumption of power as well as the reduction in HO delay can be expected.

Agreements in RAN1 #111

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

- Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

  - Further study the necessary mechanism, e.g. signaling and UE capability

Agreements in RAN1 112-bis-e

For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported:

- Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell,

- Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FFS: signalling details for TCI state activation

FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed

Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

Therefore, we have the following proposal

Proposal 1  RAN4 to achieve consensus like following and provide the information/recommendation back to RAN1:

- For ICBM scenario, T/F fine tracking i.e. activation of target cell TCI before cell switch command is received, is supported without any additional UE capability.

- For non-ICBM scenario, T/F fine tracking i.e. activation of target cell TCI before cell switch command is received, is supported with an additional UE capability.

As well known, R17 ICBM only consider the intra-frequency case. For the inter-frequency case, in our understanding, one of the reasons that RAN1 introduce early downlink synchronization is to support fine-Rx-beam-based L1 measurement of the candidate cell before cell switch. In the downlink synchronization, the BWP associated to the DL measurement is already activated, while the UE behaviour for activating the BWP would be similar to SCell activation if inter-frequency scenario is considered. The downlink sync here can also be maintained by the fine-Rx-beam-based L1 measurement on the target cell. Here the fine-Rx-beam-based SSB-based L1 measurement is configured in csi-SSB-ResourceSetList under CSI-ResourceConfig.

In legacy RRM requirements, UE performs downlink synchronization to the target cell, when the target cell, either target PCell for HO or target SCell in SCell activation, becomes active. That means the DL BWP is activated during downlink sync. In this case, UE would know the CSI-ResourceConfig that associated to this BWP. In legacy RRM requirements, L1-RSRP/L1-SINR requirements are only applicable if the measured RS is within the active BWP.

Therefore, we have the following proposal.

Proposal 2  For the inter-frequency cell switch, if UE can perform T/F fine tracking before cell switch, RAN4 discuss and clarify whether the DL BWP of target cell is activated during downlink sync before the cell switch. If so, RAN4 discuss the delay/interruption requirements if needed.

<On uplink synchronization before cell switch>

In last meeting, the following has been discussed and captured in [2].

Issue 1-3-2: Delay requirements for PDCCH ordered RACH before cell switch command

< Way forward >: FFS the following options

  - Option 1 (Apple, Ericsson, QC): Better to define similar delay requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4

  - Option 2 (MTK, Xiaomi, vivo, Huawei, ZTE, Nokia): RAN4 to study delay requirements for PDCCH ordered PRACH transmission to neighbour cell.

    - Option 2a (xiaomi):

      - If the reception of RAR is not configured/indicated, the delay of PDCCH-order based RACH transmission is defined as the time between the time of DCI command reception for PDCCH-order based RACH and the end of preamble transmission to the candidate cell plus RF retuning back time.

      - If the reception of RAR is configured/indicated, the delay of the PDCCH-order based RACH transmission is defined as the time between the time of DCI command reception for PDCCH-order based RACH and the end of RAR window plus RF retuning back time.

    - Option 2b (vivo):

      - RAN4 discuss and clarify whether the UL BWP of target cell is activated during uplink synchronization. Delay requirements need to be discussed after RAN1/2 concludes the corresponding procedure.

      - There is potential application delay and interruption for a cell on which DL sync is indicated to be performed by UE and/or UE needs to be prepared to transmit PRACH to the target cell. RAN4 will discuss the corresponding delay requirements.

    - Option 2c (QC):

      - RAN4 to discuss whether and how to define delay requirements for PDCCH ordered PRACH transmission to LTM cell for which UE needs additional processing to build and load RF scripts. It is also up to decisions from other working groups.

      - LTM requirements are applicable only when a QCL source reference signal of “PDCCH ordered PRACH to an LTM candidate cell before LTM handover” is the same or one of the reference signals configured and used for LTM L1-RSRP measurements from the cell.

Issue 1-3-3: Interruption due to PDCCH ordered RACH before cell switch command

< Agreement>:

  - Further discuss whether and how to define interruption and/or scheduling restriction requirements due to PDCCH ordered RACH before cell switch command once corresponding RAN1 design is stable.

In recent RAN1 meetings, the following is agreed. So far, RAN1 and RAN2 have agreed to introduce 2 types of PDCCH-ordered RACH for R18 LTM. Moreover, in [3], RAN1 has tasked RAN4 to discuss on the delay requirement related

|  | PDCCH ordered RACH without RAR | PDCCH ordered RACH with RAR |
| --- | --- | --- |
| Configuration | For each candidate cell, configure RA Preambles and indication of ROs with associated SSB indices (RAN1 #112) Separate IEs from candidate cell configuration (RAN2 #121bis) Power ramping-up value is optionally configured (RAN1 #112bis) | For each candidate cell, configure RA Preambles and indication of ROs with associated SSB indices(RAN1 #112) Separate IEs from candidate cell configuration (RAN2 #121bis) |
| Content of PDCCH order | Indication of cell identity of the candidate cell. (RAN1 #112) Other information as legacy (RAN2 #121bis) Indication of initial/ retransmission (RAN1 #112bis) | Indication of cell identity of the candidate cell. (RAN1 #111) Other information as legacy (RAN2 #121bis) |
| TA value indication | In cell switch command. (RAN1 #112) | Reception of RAR from serving cell for both intra-DU and inter-DU. UE should store at least one TA (RAN1 #112bis) |

Table 1  Summary of RAN1/2 agreements on PDCCH ordered RACH without RAR and PDCCH ordered RACH with RAR

Based on above summary, RAN4 is suggested to focus on the without RAR case. In this case, UE only needs to transmit the RAR when triggered by PDCCH order, and the TA is maintained by network when needed. Only when cell switch is performed, the corresponding TA is indicated to the UE. No need for UE to wait for RAR and to maintain the corresponding TA, which seems more flexible.

Proposal 3  In R18 LTM, RAN4 only considers to specify RRM requirements for the case of PDCCH ordered RACH without RAR.

As tasked by RAN1, RAN4 would need to study whether a longer gap is needed for this PDCCH ordered RACH for candidate cell. To clarify the issue, current TS 38.213 is cited below.

TS 38.213 v17.5.0

If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec, where

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission

-	∆BWPSwitching=0 if the active UL BWP does not change and ∆BWPSwitching is defined in [10, TS 38.133] otherwise

-	∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2

-	Tswitch is a switching gap duration as defined in [6, TS 38.214]

Based on current RAN1 specs, the gap is defined considering following aspects:

- The PUSCH preparation time, including the Tswitch for the SUL/NUL case, is counted. This includes the PDCCH decoding and PUSCH generation.

- The BWP switching time is counted. The PDCCH order can be sent by gNB on the CSS of active BWP. If this BWP is without RACH configured, according to TS 38.321, UE needs to perform BWP switching before RACH is sent.

- The MAC layer processing time is counted [5].

Based on above, firstly, RAN1 assumes that UE knows the downlink initial timing, which is normally obtained from SSB based DL synchronization. In legacy, the PDCCH ordered RACH is only sent by UE to the serving cell, and UE normally maintains the DL timing of the serving cell before the RACH is sent. For the R18 LTM, PDCCH order needs to be sent to candidate cell. If the candidate cell is not current active serving cell, UE needs to perform DL synchronization before the RACH is sent.

Proposal 4  If UE has not performed DL sync on the candidate cell before the PDCCH order triggering RACH on candidate cell, UE should be allowed with a duration to perform DL sync on the cell between the PDCCH order and the actually transmitted RACH.

- The conditions to determine that UE has performed DL sync on the candidate cell at least includes the case that UE is capable to perform DL pre-sync to the target cell, and the TCI activation MAC CE is received a duration before PDCCH order is received, while the duration is no shorter than the DL TCI activation delay defined in 8.15.3.

Secondly, before sending RACH, UE needs to activate the UL BWP of the target cell. For inter-frequency cell switch, at least the RF chain that corresponds to the target cell BWP needs to be activated firstly. In legacy, PDCCH ordered RACH are only sent in active serving cell and hence this issue was not considered in previous RAN1 spec. On the other hand, once the RF chain is activated, UE may also need to transmit SRS and/or PUCCH/PUSCH in the corresponding BWP hence the RF chain need not to be de-activated. However, for PDCCH ordered RACH on an inter-frequency candidate cell, UE may not know when the next PDCCH order will be received, hence it is not clear whether it should turn-off the RF train or not after PRACH is sent. In our understanding, the candidate cell that UE performs early RACH would be quite likely to be the target cell in cell switch. Therefore, we propose to only consider at most one inter-frequency layer on which UE is required to perform early RACH before cell switch. In this case UE only needs to perform RF retuning before the first PRACH transmission in this frequency layer.

Proposal 5  If UE has not turned on the RF chain for an inter-frequency candidate cell before the PDCCH order triggering RACH on this candidate cell, UE should be allowed with a duration to turn on the RF chain between the PDCCH order and the actually transmitted RACH.

- The conditions to determine that UE has turned on the RF chain for an inter-frequency candidate cell includes

  - the UE has sent one initial PRACH transmission in this uplink carrier

  - the maximum number of additional uplink carrier for LTM candidate cells, on which UE RF chains are turned-on before cell switch, is no more than one.

- In case the target cell is not the cell UE has performed early RACH, UE falls back to RACH based cell switch.

A draft reply LS to RAN1/RAN2 is provided in [6]

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  For LTM under ICBM scenario, there is no additional effort for UE to support downlink synchronization, i.e. activation of target cell TCI, before cell switch command is received.

Observation 2  For LTM under non-ICBM scenario, to support downlink synchronization, i.e. activation of target cell TCI, before cell switch command is received, additional UE effort/consumption of power as well as the reduction in HO delay can be expected.

Proposal 1  RAN4 to achieve consensus like following and provide the information/recommendation back to RAN1:

- For ICBM scenario, T/F fine tracking i.e. activation of target cell TCI before cell switch command is received, is supported without any additional UE capability.

- For non-ICBM scenario, T/F fine tracking i.e. activation of target cell TCI before cell switch command is received, is supported with an additional UE capability.

Proposal 2  For the inter-frequency cell switch, if UE can perform T/F fine tracking before cell switch, RAN4 discuss and clarify whether the DL BWP of target cell is activated during downlink sync before the cell switch. If so, RAN4 discuss the delay/interruption requirements if needed.

Proposal 3  In R18 LTM, RAN4 only considers to specify RRM requirements for the case of PDCCH ordered RACH without RAR.

Proposal 4  If UE has not performed DL sync on the candidate cell before the PDCCH order triggering RACH on candidate cell, UE should be allowed with a duration to perform DL sync on the cell between the PDCCH order and the actually transmitted RACH.

- The conditions to determine that UE has performed DL sync on the candidate cell at least includes the case that UE is capable to perform DL pre-sync to the target cell, and the TCI activation MAC CE is received a duration before PDCCH order is received, while the duration is no shorter than the DL TCI activation delay defined in 8.15.3.

Proposal 5  If UE has not turned on the RF chain for an inter-frequency candidate cell before the PDCCH order triggering RACH on this candidate cell, UE should be allowed with a duration to turn on the RF chain between the PDCCH order and the actually transmitted RACH.

- The conditions to determine that UE has turned on the RF chain for an inter-frequency candidate cell includes

  - the UE has sent one initial PRACH transmission in this uplink carrier

  - the maximum number of additional uplink carrier for LTM candidate cells, on which UE RF chains are turned-on before cell switch, is no more than one.

- In case the target cell is not the cell UE has performed early RACH, UE falls back to RACH based cell switch.

- References

[1]  R4-2303175	WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek.  RAN4 #106

[2]  R4-2306395  WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek.  RAN4 #106-bis-e

[3]  R1-2304276	LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, Fujitsu, MediaTek, CATT.  RAN1 #112-bis-e

[4]  R2-2304553  Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM, Fujitsu, CATT.  RAN2 #121-bis-e

[5]  R1-1805801  RAN1 #92bis Chairman notes.

[6]  R4-2308218  LS on the measurement and cell switch procedures in R18 LTM