3GPP TSG-RAN WG4 Meeting #107 	R4-2308218

# Incheon, KR, May 22 – May 26, 2023

Title:	[DRAFT] Reply LS on the measurement and cell switch procedures in R18 LTM

Response to:	R1-2304276, R1-2210727

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact Person:

Name:	Yanliang SUN

E-mail Address:	 Yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #107 meetings, RAN4 have further discussed the measurement and cell switch procedure for R18 LTM. Based on discussion RAN4 have reached the following agreements.

[Note: Highlighted parts will be updated based on agreements in RAN4 107]

Regarding RAN1’ Q1 in R1- R1-2210727, RAN4 provides the following additional information:

The L1-RSRP measurement requirements for the case that target SSB is not within active BWP can be hold on after the conclusion of BWP operation without restriction.

Moreover, RAN4 recommends RAN1 to consider classification of L1 measurements into L1 measurements for beam management and L1 measurement for mobility. The detailed signalling and mechanism design are up-to RAN1

- For L1 measurement for beam managements, RAN4 will NOT specify RRM requirements for the following cases

  - L1 measurement RS is outside active BWP of the UE (This may preclude most of inter-frequency scenarios)

  - RTD between cells is larger than CP

  - Rx SNR (i.e. the side condition) is below -3dB

  - TDM-based sharing between L3 measurement and L1 measurement is needed (This may preclude any sharing that impacts current L3 measurement performance)

- For L1 measurement for mobility, RAN4 will specify RRM requirements for the following cases

  - L1 measurement RS is outside active BWP of the UE (This may include most inter-frequency scenarios)

  - RTD between cells is larger than CP

  - Rx SNR (i.e. the side condition) is below -3dB but above -6dB

  - The current UE L3 measurement behaviour is assumed. (This means no need for any TDM-based sharing).

Regarding RAN1’s requested action in R1-2304276, RAN4 would like to provide the following feedback:

If UE has not performed DL sync on the candidate cell before the PDCCH order triggering RACH on candidate cell, UE should be allowed with a duration to perform DL sync on the cell between the PDCCH order and the actually transmitted RACH.

- The conditions to determine that UE has performed DL sync on the candidate cell at least includes the case that UE is capable to perform DL pre-sync to the target cell, and the TCI activation MAC CE is received a duration before PDCCH order is received, while the duration is no shorter than the DL TCI activation delay defined in 8.15.3 of TS 38.133.

If UE has not turned on the RF chain for an inter-frequency candidate cell before the PDCCH order triggering RACH on this candidate cell, UE should be allowed with a duration to turn on the RF chain between the PDCCH order and the actually transmitted RACH.

- The conditions to determine that UE has turned on the RF chain for an inter-frequency candidate cell includes

  - the UE has sent one initial PRACH transmission in this uplink carrier

  - the maximum number of additional uplink carrier for LTM candidate cells, on which UE RF chains are turned-on before cell switch, is no more than one.

- In case the target cell is not the cell UE has performed early RACH, UE falls back to RACH based cell switch.

2. To RAN WG1 group.

ACTION: 	RAN4 respectfully asks RAN1 to take above conclusions in consideration, specify the corresponding measurement and reporting mechanism, and specify gap for the PDCCH ordered RACH, provide feedback to RAN4 if any.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #108	Aug. 21 – Aug. 25	Toulouse, France

TSG-RAN4 Meeting #108-bis	Oct. 09 – Oct. 13	Xiamen, China