3GPP TSG-RAN WG4 Meeting #107 	R4-2308220

# Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on maximum uplink timing difference for multi-DCI multi-TRP with two TAs

Agenda Item:	8.30.3.2

Document for:	Discussion

Introduction

In RAN4 104e meeting, one LS [1] from RAN1 for this WID is received.

Conclusion: For multi-DCI multi-TRP operation with two TAs, the decision on the maximum uplink timing difference is left up to RAN4.

- send an LS to RAN4 asking them the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation.

RAN1 would kindly like to ask RAN4 to provide feedback on what maximum uplink timing difference that RAN1 can assume between the two TAs for multi-DCI multi-TRP operation.

RAN4 has discussed the related issue, and a LS has been sent back to RAN1 in [2] and [3].

ReplyLS to RAN1 in [2]

From RAN4 specification perspective, RAN4 so far specifies the maximum transmit timing difference between two uplink carriers as MTTD value in RAN4 specification TS 38.133. In the existing specification, MTTD requirements are specified only for CA and DC scenario.

Whether exiting MTTD requirements are applicable for multi-DCI multi-TA scenario or new requirements needs to be developed is currently under discussion in RAN4. We shall inform RAN1 once RAN4 has consensus on the MTTD value for multi-DCI multi-TA scenario.

ReplyLS to RAN1 in [3]

After RAN4 further discussion, following values are agreed as MTTD values.

For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs.

For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4.

In RAN4 #105, RAN4 has agreed to further discuss when NR_MIMO_evo_DL_UL WI starts.

In RAN4 #106 and RAN4 #106-bis-e, RAN4 has discussed the related issue. The status of the discussion is captured in [4]

Based on all above information, we provide our views on this issue.

Discussion

RAN 1 has already agreed that ‘the two TAs enhancement for uplink multi-DCI m-TRP operation’ is applicable to both TDM based multi-DCI uplink transmission and simultaneous multi-DCI uplink transmission. The related RAN1 agreements are cited below.

Agreement in RAN1 #109-e

Two TA enhancement for uplink multi-DCI based multi-TRP operation are applicable to at least:

- TDM based multi-DCI uplink transmission

- simultaneous multi-DCI uplink transmission (if simultaneous uplink multi-DCI uplink transmission is supported in Agenda 9.1.4.1)

- Note: Whether two TA enhancement is applicable to other schemes is a separate discussion, which is not in the scope of AI 9.1.1.2.

Agreement in RAN1 #110-bis-e

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

Conclusions in RAN1 #110-bis-e

For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs.

- Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

Agreements in RAN1 #112

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

As in [3], for the case of simultaneous multi-DCI uplink transmission, the MTTD value has already been agreed in [3].

For the TDM based multi-DCI uplink transmission, in our understanding it means UE may or may not be able to simultaneously Rx from different TRPs, but is certainly not able to Tx to different TRP simultaneously.

- For FR1, this is typical UE implementation assumption. Normally UE can be equipped with 2 PAs for a given band. However, the 2 PAs are normally used for UL MIMO in the single carrier scenario. For the case when UE only capable of TDM based multi-DCI uplink transmission, the 2PA are not assumed to be used for the 2 associated TAs.

- For FR2, single panel UE is of course within this scope. Even for multi-panel capable UE, the UE may not be capable of simultaneous uplink transmission due to power consumption issue.

Observation 1  The 2 TA enhancements for TDM based multi-DCI uplink transmission can be applicable to FR1 UEs and FR2 UEs, who do not support the capability of simultaneous uplink transmission.

According to current TS 38.213, if UE receives TAC command indicating the change of TA, there could overlapping in the uplink between the old TA and the new TA, and the latter slot will be reduced.

TS 38.213 v17.4.0 clause 4.2

If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot. The UE does not change N_TA during an actual transmission time window for a PUSCH or a PUCCH transmission [6, TS 38.214].

Observation 2  TS 38.213 has define the dropping/reduction rule when two UL slots overlap due to the TA command.

However, the 2-TAs enhancement discussed in R18 is slightly different. As quoted in above conclusion in RAN1 110-bis-e, the 2 TRPs may or may not have information regarding the overlapping between the 2-TAs. Therefore, although the RAN1 rule in observation 2 is like the potential ‘punishment’ to the network if a too-large TA-adjustment is indicated to the UE, it will unnecessarily degrade the performance of the UE who do not support simultaneous uplink transmission in the 2-TA scenario. In our understanding, that is why RAN1 asks the related question in the LS.

Observation 3  The RAN1 rules dealing with UL slots overlapping due to the TA command may not be feasible for the 2-TA scenario.

Looking at the agreements achieved in RAN4 [3], the UE capable of RTD > CP would mean that UE is at least capable of simultaneous DL transmission in a single carrier, i.e. UE can deal with the case when the DL overlapping region between 2 DL slots exceeds the boundary of CP, while the 2 DL reference signals for the 2 TAGs are from different TRPs. In case UE only supports RTD < CP, i.e. potentially not capable of simultaneous DL transmission, we do not see the need to force UE to support simultaneous uplink transmission.

In last meeting, there is quite good discussion on this issue, and current status of discussion is as follows.

Issue 2-1-1: What is the assumption on M1/M2 for MTTD for UE not capable of supporting RTD>CP?

- Proposals:

  - MTTD for UE not capable of supporting RTD > CP

  - Option 1: (MediaTek)

    - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=0 and M2=0

  - Option 2: (Nokia, Samsung, Xiaomi, ZTE, Huawei, QC, Ericsson, vivo, Apple)

    - If UE supports sTxMP

      - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=1.6us and M2=0.5 us

    - If UE doesn’t support STxMP

      - Wait for RAN1 further progress for gap/scheduling restriction

      - No MTTD requirements for this case.

In previous RAN4 requirements, MTTD defines the uplink timing difference between 2 simultaneous uplink transmission. In R18, supporting the 2-TA enhancement for single carrier does not always mean simultaneous uplink transmission is supported for the 2-TAs. There is ‘TDM-based’ 2-TA in R18.

For the ‘TDM-based’ 2-TAs, RAN1 would need to know how big the gap between two uplink transmission are needed. It is up to RAN1 how many factors needs to be considered. RAN1 can define gap based on the worst case.

Proposal 1  For UE not supporting simultaneous UL transmission, only TDM-based 2-TA is supported in RAN1. RAN4 do not define MTTD requirements for this case, but recommend RAN1 to define gap/scheduling restriction based on the worst case MTTD values that derived based on simultaneous UL transmission.

Moreover, in R18, RAN1 has agreed to associate TAG to the UL/joint TCI. For this issue, the situation of RAN4 is captured in [4], quoted below.

Issue 2-1-3: Reference timing

- Proposals:

  - Option 1: (Nokia, Ericsson)

  - The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission.

  - Specify for each UL/joint TCI state the DL RS the UE must use for DL time tracking.

  - Option 2: (Apple, Nokia, MediaTek, ZTE, vivo, Huawei, Samsung)

  - In UL timing requirements, some clarification needs to be added to accommodate

  - Two DL reference timings are supported where each DL reference timing is associated with one TAG.

  - Two TAGs associated with different UL/joint TCI state.

  - Option 3: (MediaTek)

  - Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing

  - FFS whether RAN4 needs distinguish same or different DL timing reference in the discussion for 2 TAGs.

  - Option 4: (Huawei)

  - For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission.

Note: FFS on whether single reference shall be allowed or not. More RAN1 input is expected for further RAN4 discussion.

As cited above, RAN1 agreements are not yet clear on which RS will be used as the downlink reference timing. Normally, UE will derive UL timing based on the downlink timing it used. In TS 38.214, normally downlink timing is indicated in DL TCI. The association between DL reference timing and UL TCI does not necessarily means the source DL RS of UL TCI will be used as DL timing reference. In our understanding, more RAN1 input on this issue is needed.

Proposal 2  For the DL reference timing of 2-TAG, wait for more RAN1 input regarding whether the source DL RS of associated UL TCI will be used as DL timing reference of the corresponding TAG.

Based on above analysis, observations and proposals we provide our preference for the reply LS in the annex.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  The 2 TA enhancements for TDM based multi-DCI uplink transmission can be applicable to FR1 UEs and FR2 UEs, who do not support the capability of simultaneous uplink transmission.

Observation 2  TS 38.213 has define the dropping/reduction rule when two UL slots overlap due to the TA command.

Observation 3  The RAN1 rules dealing with UL slots overlapping due to the TA command may not be feasible for the 2-TA scenario.

Proposal 1  For UE not supporting simultaneous UL transmission, only TDM-based 2-TA is supported in RAN1. RAN4 do not define MTTD requirements for this case, but recommend RAN1 to define gap/scheduling restriction based on the worst case MTTD values that derived based on simultaneous UL transmission.

Proposal 2  For the DL reference timing of 2-TAG, wait for more RAN1 input regarding whether the source DL RS of associated UL TCI will be used as DL timing reference of the corresponding TAG.

- References

[1]  R4-2211512	LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs, Ericsson.  RAN4 #104e

[2]  R4-2214354	LS on LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs, Ericsson.  RAN4 #104e

[3]  R4-2217279	Reply LS on MTTD for multi-DCI multi-TRP with two TAs, Ericsson.  RAN4 #104-bis-e

[4]  R4-2303258	WF on R18 NR MIMO RRM requirements, Samsung.  RAN4 #106

[5]  R4-2306362	WF on R18 NR MIMO RRM requirements, Samsung.  RAN4 #106-bis-e

Annex

Title:	[DRAFT] Reply LS to RAN1 on MTTD for Multi-DCI Multi-TRP with two TAs

Response to:	R4-2211512(R1-2205593)

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:

E-mail Address:

Attachments:

In RAN4 #107 meetings, RAN4 have discussed the remaining issues for MTTD values for Multi-DCI Multi-TRP with two TAs. Based on discussion RAN4 would like to provide the following feedback.

[Note: Highlighted parts will be updated based on agreements in RAN4 107]

For UE not capable of supporting RTD>CP:

- If UE supports sTxMP

  - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=1.6us and M2=0.5 us

- If UE doesn’t support STxMP

  - Wait for RAN1 further progress for gap/scheduling restriction

  - No MTTD requirements for this case.

RAN4 respectively ask RAN1 to take above conclusions into consideration.

# 2. Actions:

# To RAN WG1

# RAN4 kindly request RAN1 to take above conclusions in the further discussion, and provide feedback if any.

# 3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting #108	Aug. 21 – Aug. 25	Toulouse, France

TSG-RAN4 Meeting #108-bis	Oct. 9 – Oct. 13 	Xiamen, China